STANDARD MICROSYSTEMS CORPORATION

IBM LAN Server v 2.0 and v3.0 OS/2 Driver

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the IBM LAN server driver from the

SuperDisk to your PC.

o  How to install the IBM LAN server driver.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

The EZStart default Destination Directory is a:\NDIS.  Please make

a backup copy of your SuperDisk before EZStart copies files to the

SuperDisk to create an "option" diskette.

INSTALLING THE DRIVER

1. Insert the SMC or IBM LAN Adapter Protocol Support (LAPS)

Version 1.00 diskette into your computer's disk drive.

2. Type: LAPS

3. Press the Enter key.

The IBM logo appears.

4. Select the INSTALL button.

5. Specify the target drive (usually C:\) and press Enter or

select OK.

6. Select the Configure button.

7. Select "Configure LAN transports" followed by Continue.

The Configure Workstation screen appears.

8. From the Network Adapters window, select the "SMC Ethernet

Adapter for OS/2" and select the Add button.

This adapter name appears in the Current Configuration window.

9. From the Protocols window, select "IBM IEEE 802.2" followed by

the Add button. Then select "IBM OS/2 NETBIOS" followed by Add.

(You may also need to select the "IBM NetWare Requester" option

and Add, depending on your network.)

Each time you select the Add button, the selected option appears

in the Current Configuration window, below the chosen driver.

10. Select OK.

11. Select Exit.

The CONFIG.SYS Update screen appears.

12. Specify the drive where the CONFIG.SYS file resides.

13. Select the Continue... button.

14. Select OK.

15. Select Exit.

16. Reboot your computer. Use the OS/2 Shutdown process to exit from

the program in an orderly fashion.

This completes the LAPS installation procedure.

Refer to IBM's server installation procedure guides to install a

standard OS/2 server.

SMC and Standard Microsystems are registered trademarks; and

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, Elite Series, EliteView, EZStart, SuperDisk, and UltraChip

are trademarks of Standard Microsystems Corporation.  Other products

and company names are registered trademarks or trademarks of their

respective holders.