BACKGROUND INFORMATION ON THE SOFTWARE

BOOL 2 was originally developed for my own use. I used it to

design digital circuits as well as a research tool in

boolean theory and HDL synthesis. The software was released

as a commercial product initially on the IBM/PC and was

followed later by a SUN version.

The software was used by small as well as large companies in

the United States, Canada, Europe and Asia for system level

and ASIC design. Among the users were Intel, IBM, AT&T, AMD

and Siemens.

This public domain package is a fully functional version of

the software for IBM/PC. I hope you will enjoy using it as

much as I did.

Alon Kfir

Cornell Design Tools

WHAT IS BOOL 2

BOOL is a general tool for synthesizing two level logic from

a behavioral description, mainly aimed at ASIC and PLD

synthesis.  It contains an HDL compiler, a two level

minimizer, a high level event driven simulator, and output

formatting routines.

For a more detailed description of BOOL 2, read the file

HIGHLITE.DOC (you may either type it or print it)

The complete manual is provided on the file MANUAL.DOC which

can be directly printed to your printer.

COPYRIGHT NOTICE

Copyright by Alon Kfir 1989-1994.

The use, copying or redistribution of this software is

permitted only when done in compliance with the following

terms:

You may copy and give away copies of the software, but you

may not charge any fee, except for nominal shipping and

handling fees, for any such copies.

You may not alter the executable files (*.EXE files). You

may however, modify any of the text files for your own use.

Page 1

When this software is redistributed or given to another

user, each copy must include at least all the files in the

original package.

DISCLAIMER

The information furnished with this software is believed to

be accurate and reliable. However, no responsibility is

assumed by Cornell Design Tools for its use, nor for any

infringements of patents or rights of third parties

resulting from its use.

In no event shall Cornell Design Tools or Alon Kfir be

liable for any loss of profit or any damage resulting from

the furnishing, performance, or use of this software.

INSTALLING AND USING BOOL UNDER DOS

The following pages contain important information specific

to running BOOL under DOS.

GENERAL INFORMATION

BOOL can run on any class of IBM/PC and compatibles under

DOS 2.0 or above. This version does not use any graphics or

extended memory and should run without any problem on any

machine with any hardware configuration. A hard disk is

highly recommended but not strictly necessary.

In the following sections we explain how to install BOOL on

your system.  The procedure we show is only an example.  You

may choose other configurations. Basically, the installation

is just copying the files to your hard disk, and setting a

search path to the executable and library files.

The original distribution disks should be kept as Backup.

Page 2

SUSPENDING A RUNNING PROGRAM

The DOS version will check the keyboard while the program is

running.  Hitting the ESC or ALT/X keys will suspend

execution with a message. At this point, the program can

resume operation or abort, depending on the response from

the user.   The keyboard is checked only at specific break

points during execution.   If a time consuming operation

such as minimization is in progress, the response might take

a while.

BASIC INSTALLATION STEPS

Here we show how to install BOOL in a directory named BOOL

under the root directory of drive C.  First we create a

directory named BOOL, and then copy all the files from the

distribution disks to that directory.  Other configurations

are also possible (e.g. you can install BOOL under a

different drive or directory).

1. Create a directory with the name BOOL under the root.

2. copy all the files from the distribution disk(s) to the

newly created directory. If your distribution disk contains

the files in a compressed format, you have to uncompress the

files.

3. Add to the PATH command in your AUTOEXEC.BAT file (Which

is located under the root directory) the name of the

directory you have just created, making sure that it is

separated from the rest of the line by a semicolon.

For example the whole line may look like:

PATH C:\DOS;C:\MYPROGS;C:\BOOL

For more details on PATH consult your DOS manual.

You may also consider moving all the executable files to the

directory where you keep other executable files and which

already has a search path established to it.

By default, BOOL library files are searched for in the

directory \BOOL.

If you chose a different name for the BOOL directory, you

have to set the environment variable BOOL to point to that

directory.  This is explained in more detail in the

following section.

Page 3

We recommend that you create another working directory in

which you maintain your BOOL source files and results. Into

this working directory you may copy all the files with the

extensions .B and .PLA from the BOOL directory.  These are

design examples which you can try to run. Files which have

an extension of .B are BOOL source file which are run simply

by the command:

BOOL filename.B

files with extension of .PLA are PLA table files which are

used as input data files by some of the examples. For

example, the CODECONV.B example needs a data file to run.

You can run it for example with the command line:

BOOL CODECONV.B -DGRAY2BIN.PLA

Assuming as an example that your working directory will be

named JOHN, and you want to locate it under the root, the

command sequence to do this is:

MKDIR  \JOHN

CD  \JOHN

COPY  \BOOL\*.B

COPY  \BOOL\*.PLA

A QUICK TEST OF YOUR SOFTWARE

You can try to run any of the design examples that come with

this package. All the files with the extension B are design

examples. For example, try to run the majority vote circuit

by typing:

BOOL MAJORV7.B

SEARCH PATH FOR INCLUDE FILES

Your designs may use the standard include files from the

BOOL standard library.  You may also create your own include

files to define symbolic names for constants or to define

frequently used functions. (For more information on include

files refer to the description of the preprocessor #include

directive in the manual.)

When BOOL encounters an include directive, in the form of:

#include "filename"

It searches the current directory for a file with that name.

Page 4

If the search fails, but you passed the parameter -Ipath in

the command line, then path is searched.

If this fails, BOOL tries to locate the file in a directory

named \BOOL (under the root of the current drive).

However, this can be overridden by specifying a different

search path in the DOS environment.   This means that if the

DOS environment contains a string in the form: BOOL=path

then path is searched instead of BOOL.

You can set this parameter with the DOS command SET as in

the following example:

SET BOOL=C:\JOHN\MYLIB

If you don't want to type this line every time you boot your

computer, just add it to your AUTOEXEC.BAT file.   You will

find more information on the SET command in your DOS manual.

USING THE LIST UTILITY

BOOL outputs all warnings and error messages with line

column numbers pointing to the character or word where the

warning or error was detected.

If your editor cannot be directed to position the cursor on

a line given by its number, you might have difficulties

locating the errors in your source file.  To solve this

problem, you can use the listing utility which will append

line numbers to the left of each source line.   The utility

will also convert illegal non printable characters to the

character "?", and will convert each horizontal tab to the

correct amount of spaces.  The output file from the utility

reflects therefore the format as interpreted by BOOL.

To use the utility type:

BOOLIST  filename

filename is the full name of the source file including

drive, path and extension if any.  The utility will generate

an output file with the same name but with an extension of

LST.  For example, if your source file is named DESIGN1.B

type the command:

BOOLIST  DESIGN1.B

The output will be written to the file DESIGN1.LST.

Page 5

You can print this file on your printer by typing:

PRINT  DESIGN1.LST

Page 6