MACHINE.EXE

Written by Jim Tuttle, Productivity Partners, 1009 W. 13th, Houston,

TX, 77008.

This is an MS-DOS program, meaning you can run it from the command

prompt (i.e. C:>, etc.). The minimum system is a 128K IBM PC with an

MDA video card. This means if your system was manufactured after

about 1983, it will run.

If the path is such that MACHINE can be accessed directly, simply

type the word MACHINE.  Else change directories or disks until you

can access it directly. You can also run it from within Windows 3.XX

using Program Manager's RUN command found in the FILE menu. I

haven't tried it under Windows 95, and although it was created with

a Microsoft DOS compiler, it does use some assembly language

routines to detect your video hardware so it can set the display

colors correctly.

Use of the program is self-explanatory.

You can exit the program at any prompt by pressing the F1 key and

then the Enter key.

In machining operations it is often very useful to be able to

predict the forces created and the horsepower required for a

specific machining operation. This will tell whether a machine tool

is being overloaded, and give numbers for use in calculating

workpiece and fixture deflection. Many people, even those

experienced in machining, are unaware how large the forces are

which are generated during machining operations. They may also be

unaware of just how much horsepower higher speed machining

operations require.

This program was written to perform this function for drilling,

milling and turning.

You will be led through a series of questions - workpiece material,

cutter sizes, feeds and speeds, etc. You'll need to answer the

questions correctly to get an intelligible output. All units used

are American standard: inches, feet, pounds, inch-pounds, etc. After

making the first calculation, you can change some or all of the

variables and recalculate, start over, or quit.

The calculations are based on formulas from Machinery's Handbook,

and from experimentally developed equations. The results should not

be taken as hard and fast, but as estimates.

This program is public domain, and may be freely copied and

distributed PROVIDED no fee exceeding $5.00 is cherged for such

copying and distribution, it is not modified in any way, and this

.doc file accompanies it.

The author makes no claims whatsoever about fitness for any purpose,

and disclaims any and all liability resulting from its use. If you

find a "bug" (not likely as it's a pretty simple program and has

been used in various forms for about 5 years) please write and let

me know.