TechDoc  : Tcpip

Composer : Ecl@inter.NL.net (LESi)

Targets  : NetWare *.*

Revision : Tue  19-12-1995

These are 2 files, 1 is a compilation of what went on in some newsgroups

and their answers, I usually don't keep names around compilations so you

will miss them in here, note that all of my documents carry the word

'Composer' which by all means mean that I may have contributed to the

document or may not, I don't hold copyrights to any of them so don't start

bothering me about it either. Most parts have been edited for spelling

mistakes or correcting syntax. If you have anymore questions, join us in

news://comp.os.netware.connectivity :-)

-----------------------------------------------------------------------------

>> you loaded TCPIP with the "forward=yes" option, but did you "BIND

>> TO CARD1 ADDR=xx.xx.xx.xx" and "BIND IP TO CARD2

>

> That's the problem: I tried to BIND the cards as you described it but I

> don't know exactly if the CARD parameter for both cards can be a dummy? I

> don't wanna 'waste' a real IP for it because I got only five IP's from my

> provider. The IP for the router which is connecting us on the 'right

> branch' to the provider is 194.59.120.14, the address of the computer on

> the 'left branch' is 194.59.120.12. The subnet MASK for our whole net is

> 255.255.255.0 but I wasn't sure, if this is important for the

> Novell-Server.

> And BTW: what do you mean with 'subnet my IP-Adresses'?

>

> Sorry for this stupid questions, but I'm not familiar with TCP/IP and the

> company which built and supports our Novell-net never heard of TCP/IP on a

> Novell-server with two network-cards...

FWIW, you need to come up to speed on TCP/IP (in a NetWare

environment) in a big hurry.  Start studying!

OK, let's give it a quick try.  When you put multiple network cards

in a NetWare server, each of those cards must be on a separate IP

network.  The server will act as an router.  It's the only way that

nodes on one network can communicate with nodes on the other network.

Period.  (This assumes the default of RIP for the routing technique.)

You load the TCPIP.NLM with FORWARD=YES.  That's good.  But you

cannot BIND IP TO [cardname] ADDR=194.59.120.xx for both cards

because the network portion of the address is the same -- they *must*

be on different networks.  One solution is to use one of your other

5 IP addresses on the "right branch".  Another solution is to subnet

the 194.59.120.00 network, effectively creating two subnetworks.

Try this sequence on for size:

LOAD TCPIP FORWARD=YES

BIND IP TO RIGHT_BRANCH ADDR=194.59.120.129 MASK=255.255.255.192

BIND IP TO LEFT_BRANCH ADDR=194.59.120.65 MASK=255.255.255.192

In order for this to work, you may use only the following address

ranges:

Left Branch  -- 194.59.120.65 through 129.59.120.126

Right branch -- 194.59.120.129 through 129.59.120.190

That's a total of 124 node addresses you can use, out of the

original 254 in your Class C network address.  Note that you *cannot*

continue to use the node addresses you've already assigned, as they

aren't compatible with the addressing restrictions of subnetting.

Further, *ALL* nodes must have the subnet mask of 255.255.255.192

applied, including the router to the Internet.

Frankly, it would be easier to use one of your other Class C

addresses on the "left branch".  Suppose you also have network

addresses 129.59.121.0 through 129.59.124.0 available.  Then you

might do this on your server:

LOAD TCPIP FORWARD=YES

BIND IP TO RIGHT_BRANCH ADDR=194.59.120.1 MASK=255.255.255.0

BIND IP TO LEFT_BRANCH ADDR=194.59.121.1 MASK=255.255.255.0

Now all the nodes on the "left branch" must use addresses in the

range of 194.25.121.2 through 194.25.121.254, the NetWare server

(acting as a router) will be perfectly happy, and you'll live happily

ever after.  <g>

-----------------------------------------------------------------------------

Subject: Re: IPX Bridge?? KA9Q?  PCroute?

Date: 19 Sep 1995 15:33:16 GMT

>We have two LANs that we wish to connect via a 56k leased-line.  Put

>simply, we want LAN B to be able to talk to the Novell server on LAN A.

>And we want to do it as cheaply as possible.  The original plan was to

>use something like KA9Q or PCroute on inexpensive 386s -- with a CSU/DSU

>hanging off a serial port on each one.  However, I've been informed that

>KA9Q doesn't support IPX packets -- and I don't believe PCroute does

>either (I simply can't find much information on PCroute any more).

>

>Does anyone have any suggestions?  Anyone know if PCroute will route IPX

>packets -- or if someone has a modified KA9Q that simply sends all

>packets across?

"As cheaply as possible" is open to lots of interpretation.

So... pull up a chair and let's talk.

A leased line is an on-going bleeding ulcer.  If the sites are less than

5 miles apart, and you have a clear line of sight, I would suggest that

you use a wireless solution.  In particular, NCR's WaveLAN with

directional antennas.  It yields about 2.5mbps throughput, and once

installed will have no real ongoing expenses.  I've used it in the past,

and it is quite good.  Installation cost, depending on how much you do,

and how much you have NCR do, will range from $2,000 to $10,000.  In

particular, DO NOT BUY THE NCR EXTERNAL ANTENNA KIT!!!!  It's a total rip

off.  And either you, or a competent TV antenna specialist, can do as

well as NCR at a fraction of the cost.  Details upon request.

If it is more than 5 miles, and some might suggest that 3 would be a

better limit, or if you don't have a line of sight, there are other

options to consider.  This is a great time to be setting up WAN's.  Not

as good as tomorrow, but so much better than yesterday!

Let's start with an understanding that for any interactive use 56kbps is

woefully inadeaquate.  Something must be done to boost the effective

bandwidth if you are passing more than mail.  And that for just mail,

often a 9,600 bps link is adequate.

If you want a dedicated line, a very good approach is to use NewPort's

Lan2Lan cards. I think that Cisco bought them.  NewPort's point of view

is that on a slow link, the number of packet's you can forward per second

is not your bottleneck, it's the link itself.  Especially under IPX.  So,

they compress the line.  And they do it very well.  Rips and saps become

very small tokens.  A 9,600bps line with LAN2LAN cards outperforms a

56kbps line without compression, at least in my experience.  The cards

are about $1,500 each, and fit into a file server or a PC you want to act

as a router.  Each card can support two lines, so you can build a net

with them at reasonable costs.

Dedicated lines aren't as attractive as they used to be for a lot of

people.  In many areas ISDN has become very very cheap.  Here in Texas,

Southwestern Bell will install it at no charge if you agree to use the

service for 2 years.  And the monthly charge is a flat $50.00.  No

metering on local service.  But, what do you need to connect?  Lots of

options again.  Digiboard has the DataFire board.  Last time I checked, I

think that they were around $500 per board.  Ascend has their Pipe-25

system that is an external box.  Both can dial up and maintain a

connection over ISDN.  Both can do channel bonding and data compression

to get good throughput over the ISDN line.  In situations where usage is

metered, the most Ascend products can disconnect when the line is

inactive and relink when needed - I haven't looked at the Pipe-25 in

regards to that.  Reconnection typically occurs in 200 milliseconds.

The Ascend will need to be put on an EtherNet segment, and to make

sure that unwanted traffic is not sent across the WAN, it would be a

good idea to put it on it's own segment.  This adds to the cost of the

product as you have install another Ethernet card into a server.

For more information about ISDN, http://cws.wilmington.net/ is a good

place to start.

The major problem regarding IPX over a WAN is the RIP and SAP information

being broadcast.  As your net grows larger, a greater and greater part of

your traffic is tied up with this chatter.  One of the major benefits of

compression is that it reduces the amount of chatter.  I suspect that for

IPX users that NLSP can offer a large part of the benefits offered by

compression.

-----------------------------------------------------------------------------

[The following discussion assumes NetWare's TCP/IP support for

NetWare v3.12 and earlier, which does not allow for subnet or host

addresses of either all 0 or all 1.]

The use of three subnets (and up to six) requires that you "rob"

three bits from the host address portion of your IP addressing and

use them as part of the network address.  You'll have to apply a

subnet mask of 255.255.255.224 to *all* interfaces, including the IP

router to the Internet.  This reserves the first three bits of the

fourth octet as subnet addresses.  You have six available subnet

addresses at this point, but you only need 3.  Here's an example

approach:

load tcpip forward=yes

load ne2000 int=3 port=300 frame=ethernet_802.3 name=ipx-net1

load ne2000 int=5 port=340 frame=ethernet_802.3 name=ipx-net2

load ne2000 int=10 port=240 frame=ethernet_802.3 name=ipx-net3

bind ipx to ipx-net1

bind ipx to ipx-net2

bind ipx to ipx-net3

LOAD NE2000 INT=3 PORT=300 FRAME=ETHERNET_II NAME=IP1

LOAD NE2000 INT=5 PORT=340 FRAME=ETHERNET_II NAME=IP2

load ne2000 int=10 port 240 frame=ethernet_ii name=ip3

BIND IP TO IP1 ADDR=204.41.185.33 GATE=204.41.185.130 MASK=255.255.255.224

BIND IP TO IP2 ADDR=204.41.185.65 GATE=204.41.185.130 MASK=255.255.255.224

bind ip to ip3 addr=204.41.185.129 gate=204.41.185.130 mask=255.255.255.224

The router's address will have to be changed from 204.41.185.2 to

204.41.185.130, as you see above (it has to be a valid subnet

address).  Workstations connected to the IP1 interface can then use

addresses from 204.41.185.34 through 204.41.185.62; those connected

to the IP2 interface can use addresses from 204.41.185.66 through

204.41.185.94; those connected to the IP3 interface can use

204.41.185.131 through 204.41.185.158.

This leaves you with three more subnets for future use, each of which

will support about 30 node addresses.

Another option is to get more Class C addresses from your Internet

Service Provider.  <g>

-----------------------------------------------------------------------------

Net.cfg

Link support

buffers 8 1568

mempool 4096

Max Boards 3

protocol tcpip 8137 Ethernet_II

ip_address 130.15.144.3

ip_router 130.15.144.1

ip_netmask 255.255.255.0

tcp_sockets 8

udp_sockets 8

raw_sockets 1

bind ne2000

Link driver NE2000

INT 10

PORT 320

frame Ethernet_II

protocol IPX 8137 Ethernet_II

drivers.bat

@echo off

rem Drivers.bat does not log a person on but loads all network drivers

c:

cd \nw

lh lsl

lh ne2000

lh ipxodi

lh netx

lh odipkt 0 96       this is your IP packet driver

lh winpkt 0x60       this is the windows virtual packet driver

cd\

Both are available at the major FTP sites.

We are using Netware v3.11, and workstations run Windows v3.10, 3.11 and

3.11 for workgroups.

Add is a winsock stack (eg Peter Tatum's trumpet winsock v2.1f) and various

winsock TCPIP clients and off you go!

-----------------------------------------------------------------------------

The 1.20a DOS/Windows Client Kit, available from Novell's ftp sites includes

and ODI-based TCP/IP stack, TCPIP.EXE and a WINSOCK.DLL that can make use of

it for all your WINSOCK applications on Windows 3.X machines.

The 1.20a disks are a great idea for anyone running Netware 3.12 or above,

as they incorporate a wealth of previous fixes. They can also automatically

configure your net.cfg's for TCP/IP.

-----------------------------------------------------------------------------