CacheSizeTester [v009] by iLUVATAR

What the hell does CST ?

This tiny program determines your CPU type/clock, L1-data/L2 size/speed,

RAM speed and the cacheable memory size (CMA).

These tests, depending on CPU clock and RAM size, can take up to several

minutes. That's why the keyboard led flashes the whole time. So you

always know if CST still works or hangs due to an error.

How to find out cache size (very brief) ?

Hmmm...CST times the speed of memory block (offset always 200000h=2mb)

moves starting with a block size of 1kb. This is looped with doubled

block size till the block size amount to 2mb. Afterwards CST takes a

look at the measured times (starting with the time for 1kb block) and

searches these ones which lessen by at least 1/8 compared to the last

one. CST supposes at such a transfer speed fall the end of a cache

or the end of the CMA.

Don't forget being not too particular about the discovered cache sizes

owing to possible bugs and inacurate measurements.

Restrictions

To perform Cache/RAM tests it's absolutely neccessary to set a new GDT

(global descriptor table) in order to enable 32bit offsets. For that

purpose the processor must work in real mode. That's why CST will abort

if you try to run it under a v8086 manager like EMM386/QEMM/WIN9x/WINNT.

But there is another problem: CST requires Pentium-compatible MSR's to

time memory accesses as exactly as possible. These machine specific

registers are implemented in all newer Intel (some 486DX, Pentium and

above) and AMD (K5,K6,K6-2/3) processors. Older processors won't work

with CST since they simply doesn't know the RDMSR instruction.

Tested configurations

CST was tested successfully on the following systems.

PentiumMMX-200      L1-data:16KB L2:256KB  RAM:80MB  CMA: ends at 64MB

PentiumMMX-200      L1-data:16KB L2:512KB  RAM:64MB  CMA: not found

PentiumII-300       L1-data:16KB L2:512KB  RAM:96MB  CMA: not found

PentiumII-333       L1-data:16KB L2:512KB  RAM:64MB  CMA: not found

PentiumII-450       L1-data:16KB L2:512KB  RAM:256MB CMA: not found

PentiumIII-600EB    LI-data:16KB L2:256KB  RAM:196MB CMA: not found

AMD K6-200          L1-data:32KB L2:512KB  RAM:16MB  CMA: not found

AMD K6-2-300        L1-data:32KB L2:1024KB RAM:64MB  CMA: not found

AMD K6-2-350        L1-data:32KB L2:1024KB RAM:256MB CMA: ends at 255MB

AMD K6-2-450        L1-data:32KB L2:512KB  RAM:256MB CMA: ends at 128MB

You must no wonder why the AMD K6-2's CMA ends at 255MB (and not at

256MB). This is because some newer motherboards cache the memory up

to ((2^x)MB-L2 cache size).

History

[v000]-[v006]:

changed, added and removed too much

[v007] 11/26/98:

first public version

[v008] 06/25/99:

added detection for AMD K6-3

updated adress/email

[v009] 04/18/00:

fixed cpu mode detection (could find v86 mode instead of real mode)

fixed PII/PIII/Xeon/Celeron detection (thanks to www.sandpile.org)

added AMD K7 detection

Source code

If you wanna take a look at the sources, feel free to contact me by

inet at eschwarz.sda@t-online.de or snail mail.

Author

Christian Schwarz

Schottenstra�e 1

99084 Erfurt

Germany

eschwarz.sda@t-online.de

End of DOC :)