PARmonitor V1.02

NOTE: Don't use a proportional font for this document!

1.   DESCRIPTION

1.1  short intro

1.2  the PC parallel port

2.   INSTALLATION NOTES

2.1  requirements

2.2  package description

2.3  using PARmon under DOS

2.4  PARmon and Windoof95

2.5  general usage

3.   OUTRO

3.1  known bugs

3.2  history

3.3  contacting the author

3.4  warnings

---------------------------------------------------------------------

1    DESCRIPTION

---------------------------------------------------------------------

1.1  short intro

^^^^^^^^^^^^^^^^

What is PARmon?

PARmon is a short program that allows you to monitor (and change) the

status of your PC parallel port(s). The program is very easy to use

and can be particularly helpful for parallel port related hardware

projects.

---------------------------------------------------------------------

1.2  the PC parallel port

^^^^^^^^^^^^^^^^^^^^^^^^^

NOTE: This is only a short description of the PC parallel port.

It is meant to be a quick reference rather than a detailed

description. If you're new to the PC parallel port you

should definitely read a "PC-parallel-port-FAQ" before

you start breaking your PC!

DO NOT PLAY WITH YOUR PC'S PARALLEL PORT AS IT IS QUITE EASY

TO BREAK!

The PC parallel port comes as a female DB-25S connector on the

computer:

input/output     pin    signal name    bit   IOAddr

--------------    ---    -----------    ---   ------

output          1       _STROBE       C0   base+2

output /*input     2        DATA 0       D0   base+0

output /*input     3        DATA 1       D1   base+0

output /*input     4        DATA 2       D2   base+0

output /*input     5        DATA 3       D3   base+0

output /*input     6        DATA 4       D4   base+0

output /*input     7        DATA 5       D5   base+0

output /*input     8        DATA 6       D6   base+0

output /*input     9        DATA 7       D7   base+0

input         10        ACK          S6   base+1

input         11       _BUSY         S7   base+1

input         12        PAPEREND     S5   base+1

! input         13       !SELECTin     S4   base+1

output         14       _AUTOFD       C1   base+2

input         15        ERROR        S3   base+1

output         16        INIT         C2   base+2

output         17       _SELECT       C3   base+2

GROUND         18         GND

GROUND(should be) 19-25     (GND)

###########################################

* = only on bi-directional ports (C5[base+2] to toggle between in-/output)

_ = inverted (bit is set[1] when pin is low[0 to +0.8V] and cleared[0]

when pin is high[+2,4 to +5V])

! = SELECTin is an exceptional input:

All inputs except SELECTin are TTL high. This means that they have

to be pulled to ground to go low (low = 0 to +0.8V).

SELECTin (as an exception) is low unless it receives a high (+2.4

to +5V).

Get base at 0x408 (max.4 (PS/2 max.3) 16-bit words)...

---------------------------------------------------------------------

2.   INSTALLATION NOTES

If you happen to have a bi-directional parallel port and your BIOS

allows you to select between SIMPLE, EPP and EPC - go for EPP if you

have "high speed" hardware as EPP seems to create the cleanest signal

(nearly a rectangle).

---------------------------------------------------------------------

2.1  requirements

^^^^^^^^^^^^^^^^^

In order to use PARmon you'll need a PC (386 or above) and at least

one parallel port.

PARmon runs under DOS or in a DOS-box (Windoof) as well.

---------------------------------------------------------------------

2.2  package description

^^^^^^^^^^^^^^^^^^^^^^^^

PARmon comes in a ZIP file. You may have unzipped it already as

you are reading this...

The ZIP file contains the following files:

size in bytes    filename                    description

-------------  ------------  ------------------------------------------------

863  ReadMe.1st    short info about PARmon

9.320  PARmon.doc    PARmon "documentation" (you're reading it;o)

26.678  PARmon.ASM    Assembler Source for building PARmon (MASM/TASM)

1.876  PARmon.COM    PARmon V1.02 executable - color version

1.876  PARmon_M.COM  PARmon V1.02 executable - monochrome version

326  PARmon.ICO    crappie icon you might (not) want to use (Win95)

967  PARmon.PIF    Win95-PIF which you have to fix to your needs

---------------------------------------------------------------------

2.3  using PARmon under DOS

^^^^^^^^^^^^^^^^^^^^^^^^^^^

First choose the version you like most (color or monochrome). Then

copy that version to some directory within your PATH and rename it

to PARmon.COM (if you chose the color version it should already

have that name).

Then type "PARmon" to start PARmon V1.02. PARmon will automatically

choose the first parallel port it finds and show its status.

If you happen to have more than one parallel port you may want to

specify the port number manually. You can do this by passing a port

number between 1 and 3 as a command line argument:

usage:  PARmon.COM [port number (1-3)]

PARmon will complain if the desired port isn't available.

---------------------------------------------------------------------

2.4  PARmon and Windoof95

^^^^^^^^^^^^^^^^^^^^^^^^^

Edit PARmon.PIF (right click on it and choose "properties".

Set the working directory to the one containing PARmon

(full path, just use "browse").

Set the icon path to the PARmon.ICO icon (if you want;).

Edit the command line. If you have more than one parallel

port choose the one you want to monitor and type it in as

a command line argument (i.e.: "PARmon.COM 2" if you want

to monitor port number two).

If you want you can also use several instances of PARmon

in parallel. Just copy the PARmon.PIF file to PARmon2.PIF

and enter "PARmon 2" as a command line argument for port

number two for example. This way you can create several

copies - one for each parallel port...

(You may also run several instances of PARmon on the same

parallel port, thus slowing down your system.;o)

Run PARmon in full screen mode (ALT+RETURN) if it reacts

too slowly.

---------------------------------------------------------------------

2.5  general usage

^^^^^^^^^^^^^^^^^^

Once you have started PARmon you will see the status of the selected

parallel port. Pins that have a TTL high voltage (+2.4 to +5V) appear

in uppercase letters and a different color than those appearing in

lowercase letters (thus having a TTL low voltage (0 to +0.8V)).

Pins having arrows pointing towards the pin number are inputs. Those

"coming from the pin" are outputs.

Pins that can be toggled (outputs) have an highlighted char indicating

the key you may want to press to switch their status.

For example by pressing "T" you toggle the status of the STROBE pin.

If you have a mouse-handler installed (and a mouse;) you can also click

on "STROBE" to experience the same effect...

WHEN THERE IS SOMETHING ATTACHED TO THE SELECTED PARALLEL PORT

DO NOT CHANGE THE STATUS OF ANY PIN IF YOU DON'T KNOW WHAT YOU ARE DOING!

(for example, a parallel port streamer might react "funny" to such a

status change)

By pressing "D" (or clicking at "switch data to input <-> output") you

can toggle the DATA pins to either in- or output (only if you have a

bi-directional port).

To end PARmon press the "ESC" key or click at the "press ESC to abort"

text.

---------------------------------------------------------------------

3.   OUTRO

---------------------------------------------------------------------

3.1  known bugs

^^^^^^^^^^^^^^^

Yet none, although the Source is everything but "decent" (I simply

"programmed" it right a head - without any plan (never do so your-

self;) + I was ill (fever) when I did so)...

---------------------------------------------------------------------

3.2  history

^^^^^^^^^^^^

Version 1.00 didn't have any mouse support and had several bugs. The

most awful one was the colors it came with (I do only have a mono-

chrome LCD;o).

Version 1.01 was a fixed one with (hopefully, coz I didn't see'em

that long;) much better colors.

Version 1.02 (this one) got mouse support and experienced some minor

"Source cleaning".

---------------------------------------------------------------------

3.3  contacting the author

^^^^^^^^^^^^^^^^^^^^^^^^^^

If you want (one at a time please!;) you can reach me at:

aborto@usa.net

(known to lose Mail)

Please feel free to send bug reports (even though I might not care;),

pajamas or books from E. Kant...

---------------------------------------------------------------------

3.4  warnings

^^^^^^^^^^^^^

DO NOT PLAY WITH YOUR PARALLEL PORT(S) IF YOU DON'T KNOW WHAT YOU ARE

DOING!

DO NOT PLAY WITH PARmon WHEN HAVING HARDWARE ATTACHED TO YOUR PARALLEL

PORT (AND YOU DON'T KNOW WHAT YER DOIN)!

DO NOT FEED AFTER MIDNIGHT!

...

The author is not responsible for any damage caused due to bogus

software or false descriptions.

USE IT AT YOUR OWN RISK!

Blah, blah, blah and so on...