THRUST 1.11

TABLE OF CONTENTS

WHAT IS THRUST? . . . . . . . . . . . . . . . . . . . . . . . . . .    2

HOW TO USE THRUST . . . . . . . . . . . . . . . . . . . . . . . . .    3

DIGITAL JOYSTICKS . . . . . . . . . . . . . . . . . . . . . . . . .    6

OTHER PROGRAMS  . . . . . . . . . . . . . . . . . . . . . . . . . .   10

WARRANTY  . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   11

REVISION HISTORY  . . . . . . . . . . . . . . . . . . . . . . . . .   12

THRUST 1.11            Page 1 of 12

WHAT IS THRUST?

���������������

THRUST is a very simple, low-level, DOS program that lets you easily

test your game port and/or joysticks to verify that they are working

correctly.

It takes two parts to get a joystick to work on a computer - the

joystick itself, and the game port (the port you plug the joystick

into).  If you are having problems with your joystick(s), THRUST can

definitely help you determine whether the problem is with your hardware

or with the program that is trying to use the joysticks.  However,

THRUST cannot tell you whether the hardware culprit is the game port or

the joysticks.

From personal experience, however, I can tell you this.  I have had

several different joysticks and game ports on my computers over the

years.  I have never had a bad joystick (at least where the defect

wasn't VERY obvious).  Well over half of my game ports have been bad

(they either don't work at all or they only work partially).  A game

port is so simple that it boggles me to imagine how manufacturers can

screw it up so bad - but they seem to do it on a regular basis.

There are two different ways that a program can look at the game port on

your computer.  The first way is through a BIOS function that is built

into all modern computers.  The only computers that should not have the

standard BIOS function are IBM PC's and the very first IBM PC/XT's.

Later versions of the PC/XT and every computer built since November 1982

should have the BIOS function.  This means that every computer with at

least a 80286 CPU, and almost every computer with an 8088 CPU, has the

BIOS function.

A program can also look at the game port directly, without using the

BIOS at all.  This is faster and more accurate than using the BIOS, but

is also more difficult for programmers to implement.  So they usually

don't do it.

THRUST can look at the game port using either method.  It can either use

the BIOS, or it can look at the game port directly.  See the section

below, "HOW TO USE THRUST", on page 3, for directions on how to tell

THRUST which method you want it to use.

When THRUST is looking at the game port directly, it is looking at the

lowest hardware level possible.  What that means, in simple terms, is

this:  if THRUST is looking directly at the game port, and THRUST can't

see it, it isn't there.  If THRUST can't see it, no other program will

be able to see it either.  If THRUST can see it, but another program

can't, there's something wrong with the other program (and that "other

program" could be your BIOS).  More details are discussed in the

sections below.

THRUST 1.11            Page 2 of 12

HOW TO USE THRUST

�����������������

As noted in the "WHAT IS THRUST" section above, there are two different

ways that THRUST can look at the game port.  It can either use the BIOS,

or it can look at the game port directly.  The default is for THRUST to

look at the game port directly (this is faster and much more accurate

than using the BIOS).  If you simply type "THRUST" at a DOS prompt, you

will see something that looks like this:

THRUST 1.11, (C) 1998-1999, Bret Johnson.

A game port & joystick tester.

To get status from the BIOS, type "THRUST B".

Getting status DIRECTLY from the game port.

Press a key on the keyboard to stop the test.

AX   AY   BX   BY  A1 A2 B1 B2

---- ---- ---- ---- -- -- -- --

952  933    0    0 On -- -- --

This is showing you the current, "real-time" status of your joystick(s).

The last line on the screen (the one with the numbers and "On" and "--")

will constantly be changing as you move your joysticks and press your

joystick buttons.  Exactly what the numbers mean, how they change, etc.,

is discussed on the next page.  When you are done testing your

joystick(s), hit a key on the keyboard.  THRUST will return you to the

DOS prompt.

The fourth line of the output, "Getting status DIRECTLY from the game

port.", indicates just what it says - the information is coming directly

from the game port, not from the BIOS.  If you want to see what the

output of the BIOS looks like, just type something (anything) on the

command line after the "THRUST".  Any of the following would make THRUST

look at the BIOS instead of looking directly at the game port:

THRUST p

THRUST 9

THRUST BIOS

THRUST Use the BIOS, Please!!!!

If you do this, the fourth line of the output will change to "Getting

status from the BIOS."  The rest of the screen will look pretty much the

same.  You will notice, however, that the magnitude of the numbers will

be different (instead of being more than 900, as they are in the example

above, they will probably be somewhere around 50).  This is because of

the different ways the two methods measure time.  Exactly what the

numbers are (and what they should be) is discussed on the next page.

THRUST 1.11            Page 3 of 12

Let's look at a sample of the THRUST output again:

AX   AY   BX   BY  A1 A2 B1 B2

---- ---- ---- ---- -- -- -- --

952  933    0    0 On -- -- --

This is what the columns are:

AX = The X (horizontal or left-right) coordinate for joystick A

AY = The Y (vertical or up-down) coordinate for joystick A

BX = The X (horizontal or left-right) coordinate for joystick B

BY = The Y (vertical or up-down) coordinate for joystick B

A1 = The status of button 1 on joystick A

A2 = The status of button 2 on joystick A

B1 = The status of button 1 on joystick B

B2 = The status of button 2 on joystick B

The first 4 columns (AX, AY, BX, & BY) show the horizontal (X) and

vertical (Y) coordinates of the joystick(s).  THRUST calls the first

joystick "A" and the second joystick "B".   When a joystick is centered,

the X and Y coordinates are usually between 500 and 1000, if THRUST is

looking directly at the game port, or between 25 and 50 if THRUST is

using the BIOS.

As you move the joystick handle to the left, the X (horizontal)

coordinate gets smaller.  When it is all the way to the left, the

coordinate is usually between 25 and 50, if direct, or between 3 and 6,

if using the BIOS.  When you move it to the right, the X coordinate gets

bigger.  When it is all the way to the right, it should be about twice

as big as when it was centered (between 1000 and 2000 if looking at the

game port directly, or between 50 and 100 if using the BIOS).

As you move the handle up, the Y coordinate gets smaller.  As you move

the handle down, the Y coordinate gets bigger.  The numbers should range

from about 25~50 (3~6), when the handle is all the way up, to 1000~2000

(50~100), when the handle is all the way down.

It usually doesn't matter exactly what the coordinate numbers are.  What

really matters is that they change appropriately as you move the

joystick handle(s).

If THRUST says a coordinate is 0, it means that it doesn't exist.

Either the joystick isn't plugged in, the joystick is broken, the

joystick is "special" (for instance, it is a digital joystick), or the

game port is bad.

The last four columns (A1, A2, B1, & B2) show the status of the buttons

on the joystick(s).  If a button is pressed, THRUST says "On".  If a

button is not pressed (the normal case), THRUST says "--".  If you press

a button and THRUST still says "--" for that button, something is

broken, not plugged in, or "special".

THRUST 1.11            Page 4 of 12

It is important to note that THRUST is no more than a troubleshooting

and monitoring tool for your game port and your joysticks.  THRUST

simply shows you raw data as it comes from the game port (or the BIOS).

You'll need another, different program (like my JOYKEYS program) to

actually be able to do something useful as you move your joystick handle

and press the buttons.

THRUST 1.11            Page 5 of 12

DIGITAL JOYSTICKS

�����������������

Modern joysticks fall into two categories - analog and digital.  Analog

joysticks are "real" joysticks, and have been around at least since IBM

has been building PC's (1981).  When a program talks about a joystick,

it is usually talking about an analog joystick (this includes THRUST).

Over the last couple of years, though, manufacturers have started making

"digital" joysticks.  Digital joysticks don't "talk" to the computer the

same way an analog joystick does.  A program that thinks it talking to

an analog joystick, but is actually talking to a digital joystick, will

think that the joystick is broken.  If you look at a digital joystick

with THRUST, you may see some weird stuff coming out of the game port.

If you see anything at all (other than zeroes), it definitely will not

resemble what you see when you look at an analog joystick.

The manufacturers claim that digital joysticks are "better" than analog

joysticks because they can have more buttons and handles, and can be

faster than analog joysticks.  The problem is, every digital joystick

manufacturer builds their joysticks a different way than everybody else

does.  Even different digital joysticks made by the same manufacturer

are made differently than each other, and they all "talk" to the

computer differently.  It is impossible to create a generic program that

will work with all digital joysticks, because there is no such thing as

a "generic digital joystick" (unlike analog joysticks, which are always

"generic", at least to some degree).  Therefore, in order to use a

digital joystick, you have to install a special software driver for your

computer to even know that it's there.

As with almost everything else in the computer world these days, though,

manufacturers will only give you a driver for the latest version of

Windows (nowadays, Win95/98 or WinNT).  The don't make a DOS driver, and

have no intention of EVER making a DOS driver.  They also don't make a

driver for older versions of Windows (anything before Win95), and also

have no intention of EVER making any.

You will usually have to create a DOS driver yourself, or try to find

someone who has already made one for your particular joystick.  Don't

bother asking the manufacturer who made the joystick, because they will

ignore you (even though that is where you SHOULD be able to get the

drivers).  I've made one DOS driver myself, for a Microsoft SideWinder

digital GamePad (you can download it for free from my Home Page).  I'll

probably be making some others in the future, too, especially if people

will help me buy, or at least borrow, some digital joysticks to test

with.

THRUST 1.11            Page 6 of 12

THRUST reacts differently when you have a DOS digital joystick driver

installed.  In my digital drivers, I will always have a special function

that allows other programs (like THRUST and JOYKEYS) to automatically

recognize that there is a special (digital) joystick driver installed.

The DOS driver replaces the BIOS joystick routine so that a program can

talk to the digital joystick through the BIOS, even though a program

that looks directly at the joystick will think it is broken.

Therefore, if THRUST detects that a special joystick driver has been

installed, it will indicate this to you when it starts (you will see

"Found a digital joystick driver." on the screen).  It will then,

automatically, use the BIOS to look at the game port rather than looking

at it directly (you will also see a "Getting status from the BIOS." on

the screen).

If, for some reason, you want to have THRUST look at the game port

directly instead of using the BIOS, just type something (anything) on

the command line after the "THRUST" (just like you did with an analog

joystick to have it use the BIOS instead of looking at the game port

directly):

THRUST p

THRUST 9

THRUST Direct

THRUST I know there's a special BIOS, but don't use it!!!!

All of the other stuff in the section above, "HOW TO USE THRUST" (page

3), still applies, whether you are looking at the digital joystick

directly or using the BIOS.  Of course, when you look at a digital

joystick directly, what you see will probably make no sense at all (at

least if you compare it to an analog joystick).  But THRUST will still

let you look at it if you're interested.

THRUST 1.11            Page 7 of 12

A WORD FROM THE SPONSOR

�����������������������

THRUST is a free program.  If you like it and use it, do something nice

for someone else in return.  I will accept niceties to myself in the

form of $.

You can freely copy and distribute THRUST.COM, as long as it is

distributed along with this THRUST.DOC, and neither file has been

modified in any way.  You cannot charge anyone in any way for THRUST,

except to recover your actual costs for disks and shipping (you cannot

charge for "handling").  You do need my permission to distribute THRUST

as a "companion" to some other program.

You can also freely download the source code for THRUST.  It is

available from my home page:

http://members.aol.com/bretjohn

I haven't included the source code here, to keep the size of the

download small, and because most people don't care about source code,

anyway.  But it's available if you want it.

I wrote THRUST for 3 reasons.  First of all, I use it in combination

with one of my other programs, JOYKEYS.  I needed a way to test

"special" joysticks, that don't follow standard joystick conventions

(like digital joysticks and ThrustMaster analog joysticks).  The fact

that joystick manufacturers even decide to come up with non-standard

ways of doing things really torques me off.

The second reason for writing THRUST was to give people an easy way to

test their game port and joystick(s).  THRUST gives you a small, simple,

no-frills look at your game port.  As stated above, if THRUST can't see

it, it isn't there.  There are other game port testing programs

available, but most of them only use the BIOS to look at the joystick.

This is not nearly as effective or as accurate as THRUST's ability to

look at the game port directly.

The third reason for writing THRUST is for other programmers.  I have

had many people tell me that turning on the built-in joystick support

for certain programs slows them way down (sometimes to the point of

being virtually unusable, especially on a slower computer).  This is

ridiculous.  Most programmers these days think that everything can be

done in a high-level language (like C or Basic).  There are certain

hardware things on a computer, like Joysticks and Serial ports, that

need to be written in Assembly Language.  If they aren't, they simply

aren't fast enough to be reliable or useful - no matter how fast the CPU

is.

THRUST 1.11            Page 8 of 12

The routine in THRUST that looks directly at the game port is called

GetCoords.  It is the same basic routine that is used in my JOYKEYS

program.  JOYKEYS is a TSR that is so fast it can even be used on the

original IBM PC (4.77MHz 8088 CPU) with almost no speed degradation.  If

you want to see how I do it, download the source code and peruse the

GetCoords routine.  I guarantee you it works a lot better than the BIOS!

THRUST 1.11            Page 9 of 12

OTHER PROGRAMS

��������������

If you find THRUST useful, I have written some other programs you may

find even more useful.

CLOCK

Shows a clock on your screen all the time.  Also has two alarms

that beep at you when it's time for an appointment.

JOYKEYS

Lets you use your joystick with ANY program.

MOUSKEYS

Lets you use your mouse with ANY program.

PRTSCR

Sends all of your PrintScreens to a file instead of a printer.  A

good way to transfer data between two programs, even if they won't

normally "talk" to each other.

PRTSCRFF

Modifies your PrintScreen so that each new PrintScreen gets put on

a new sheet of paper (especially useful if you have a Laser

printer).

SCANCODE

Type keystrokes automatically in the middle of other programs.

Helpful in getting past the "starting screens" of most programs, or

to create macros.  Can also be used to remap the keyboard.

SDWRGMPD

DOS driver for a Microsoft SideWinder digital GamePad.

SERIAL

Shows the status of your serial (COM) ports on screen.  Shows the

bit rate, parity, status of the leads (DTR, DSR, etc.).  An

excellent way to troubleshoot modem problems.

SLOWDOWN

Slows down a fast computer so you can run older programs that

require a PC/XT or PC/AT.

THRUST 1.11            Page 10 of 12

WARRANTY

��������

There is no warranty of any kind, either expressed or implied, supplied

with THRUST.  THRUST is a very simple DOS program, and should not cause

you any problems.

If you find any "bugs" in this program, or see something that you wish

were different, please let me know.

HAVE FUN!

Bret Johnson

6775 South Logan Street

Littleton, CO  80122-1249

(303) 795-5084

bretjohn@juno.com  or

bretjohn@aol.com

I intend to always keep the latest versions of all of my programs on my

own web page.  The URL is:

http://members.aol.com/bretjohn

Keep looking there for updates to my programs.

THRUST 1.11            Page 11 of 12

REVISION HISTORY

����������������

1998/01/26  v1.00

1999/05/13  v1.10

* Added the ability to look at the game port with the BIOS.

* Added the ability to automatically recognize that a "special"

(digital) joystick driver has been installed.

* THRUST would poll the game port and display the coordinates as fast

as the computer would go.  This made it nearly impossible to

recognize the last digit or two of the coordinate numbers.  THRUST

now only polls the game port and updates the screen about 6 times

per second.  You can now recognize the coordinate number much easier

than before.

* Incresed the maximum allowable coordinate, when accessing the

joystick directly, from 8000 to 10000.

1999/09/16  v1.11

* Added the ability to recognize bad Joystick BIOS routines.

THRUST 1.11            Page 12 of 12