SWATVXD  -- Version 3.02 -- A Debugger for WIN386

(C) Copyright 1990-8 Qualitas, Inc.	All rights reserved.

The SWATVXD.EXE file is the accompanying VxD to 386SWAT.  As long as

it is in the same directory as the debugger file (386SWAT.LOD), it is

loaded automatically by 386SWAT.  If for some reason you don't wish

that to happen, using the command "WIN NOSWAT" prevents the VxD from

loading.

Switches

--------

The VxD provides Windows services for 386SWAT, and is useful for

debugging VxDs.

The VxD's operation is controlled by various switches all of which

appear in the [386SWAT] section of your SYSTEM.INI file.

SWATDualDisplay

Squirt cool info to the mono display -- you must use this option to

get the most useful output to the mono screen.

SSF=d:\path\name.SSF

Specify the WIN.SSF file -- this file is generated when SWAT VxD is

built and contains useful symbol information about Windows VxDs.

SWATVxDROM

In Sys_Critical_Init, mark the Page Table Entries of all VxD code

segments as read-only (normally, they are read-writable!).	This

feature was implemented in an attempt to figure out why Windows was

crashing (I thought that there might be a bad pointer out there).

While I didn't find anything in particular, it is handy to rule out

this problem.  If a VxD code segment is written into, an Invalid Page

Fault is generated (see below).  Likely, you'll find that a single IPF

due to this feature is generated at the start of Windows.  The default

action (taken by pressing Esc at the command line) is to test for a RO

page and set the RW bit.  This clears the IPF for this one page only.

From that point on, all IPFs should be examined carefully.	If this

switch is set, the SWATVxDIPF and SWATPL0WP switches are also set

automatically.

SWATVxDIPF

In Sys_Critical_Init, hook the Invalid Page Fault handler.  This

feature, in conjunction with the IPF command in SWAT can be useful for

debugging these beasts.  The mono screen contains a formatted message

of the incoming data to the IPF hook procedure.  This feature is

preliminary and provides a way to get control at the point of the IPF

-- after that, you're on your own.  If you can suggest more things

SWAT can do to help debug IPFs, I'm very open to suggestions.

SWATPL0WP

In Sys_Critical_Init, set the WP bit in CR0 so we can trap writes

into Read-Only pages at PL0.

HookIRQ

Hook IRQ0-7 locally if SWAT hooks them.  This switch is necessary

for some VxDs (RAM Doubler in particular) as they don't install

correctly if they find some IRQs not hooked by selector 28h.  Go

figure.

Debugging VxDs

--------------

To debug VxDs at their entry points, use the following switches:

HookVxD=DDB_Name[,#s]

Trap at a particular VxD by name.  Note that <DDB_Name> is case

sensitive, and <#> specifies message numbers (optional).  See the VxD

Filename and Message # tables below.

HookPM=DDB_Name,reg,#

Trap at a particular VxD's PM API entry.  <reg> specifies a 16-bit

register used by the VxD for the function code, and <#> specifies the

functions to trap.	For example, HookPM=SHELL,DX,3 traps at the

WINOLDAP hook

VxD Filenames

-------------

Some VxD filenames, DDB names, Device_ID, and their switches:

WINA20.386	 'LA20HMA ' xxxxh

NOLOW64KPAGING

NOLOW64KPAGINGPORT

Debug		 '        ' 0002h

*VPICD		 'VPICD   ' 0003h

*VDMAD		 'VDMAD   ' 0004h

*VTD		 'VTD     ' 0005h

*V86MMGR	 'V86MMGR ' 0006h

*PAGESWAP	 'PageSwap' 0007h

*VKD		 'VKD     ' 000Dh

INITPS2MOUSEATEXIT=ON/OFF

*DOSMGR 	 'DOSMGR  ' 0015h

*WSHELL 	 'SHELL   ' 0017h

*PAGEFILE	 'PageFile' 0021h

386MAX.VXD	 'LoadHi  ' 001Ch

SWATVXD.EXE  'SWATVXD ' 2400h

VDDVGA.386	 'VDD     ' 000Ah

VNETWARE.386 'DOSNET  ' 001Ah

PM_BIOS.386  'PMVDD   ' 2250h

VxD Message #s

--------------

0000h  Sys_Critical_Init		 001Bh Sys_Dynamic_Device_Init

0001h  Device_Init				 001Ch Sys_Dynamic_Device_Exit

0002h  Init_Complete			 001Dh Create_Thread

0003h  Sys_VM_Init				 001Eh Thread_Init

0004h  Sys_VM_Terminate 		 001Fh Terminate_Thread

0005h  System_Exit				 0020h Thread_Not_Executeable

0006h  Sys_Critical_Exit		 0021h Destroy_Thread

0007h  Create_VM				 0022h PNP_New_Devnode

0008h  VM_Critical_Init 		 0023h W32_DeviceIOControl

0009h  VM_Init					 0024h Sys_VM_Terminate2

000Ah  VM_Terminate			 0025h System_Exit2

000Bh  VM_Not_Executeable		 0026h Sys_Critical_Exit2

000Ch  Destroy_VM				 0027h Vm_Terminate2

000Dh  VM_Suspend				 0028h Vm_Not_Executeable2

000Eh  VM_Resume				 0029h Destroy_VM2

000Fh  Set_Device_Focus 		 002Ah VM_Suspend2

0010h  Begin_Message_Mode		 002Bh End_Message_Mode2

0011h  End_Message_Mode 		 002Ch End_PM_App2

0012h  Reboot_Processor 		 002Dh Device_Reboot_Notify2

0013h  Query_Destroy			 002Eh Crit_Reboot_Notify2

0014h  Debug_Query				 002Fh Close_VM_Notify2

0015h  Begin_PM_App			 0030h Get_Contention_Handler

0016h  End_PM_App				 0031h Kernel32_Initialized

0017h  Device_Reboot_Notify	 0032h Kernel32_Shutdown

0018h  Crit_Reboot_Notify

0019h  Close_VM_Notify

001Ah  Power_Event

Message Number Traps

--------------------

To stop inside SWAT at various message numbers, use the following

switches:

SWATSysCriticalInit

SWATDeviceInit

SWATInitComplete

SWATSysVMInit

SWATSysVMTerminate

SWATSystemExit

SWATSysCriticalExit

SWATCreateVM

SWATVMCriticalInit

SWATVMInit

SWATVMTerminate

SWATVMNotExecuteable

SWATDestroyVM

SWATVMSuspend

SWATVMResume

SWATSetDeviceFocus

SWATBeginMessageMode

SWATEndMessageMode

SWATRebootProcessor

SWATQueryDestroy

SWATDebugQuery

SWATBeginPMApp

SWATEndPMApp

SWATDeviceRebootNotify

SWATCritRebootNotify

SWATCloseVMNotify

SWATPowerEvent

SWATSysDynamicDeviceInit

SWATSysDynamicDeviceExit

SWATCreateThread

SWATThreadInit

SWATTerminateThread

SWATThreadNotExecuteable

SWATDestroyThread

SWATPNPNewDevnode

SWATW32DeviceIOControl

SWATSysVMTerminate2

SWATSystemExit2

SWATSysCriticalExit2

SWATVMTerminate2

SWATVMNotExecuteable2

SWATDestroyVM2

SWATVMSuspend2

SWATEndMessageMode2

SWATEndPMApp2

SWATDeviceRebootNotify2

SWATCritRebootNotify2

SWATCloseVMNotify2

SWATGetContentionHandler

SWATKernel32Initialized

SWATKernel32Shutdown

Internal Use Only

-----------------

The following switches are for internal use only.

SSFPrefix=name

Specify device name and DDB prefix.

SWATDebugThruExit

Call SWAT's INIT_PROT during Critical_Exit.  This allows debugging

on the way out of SWAT.  Now that SWAT is a Windows kernel debugger,

this switch is unnecessary.

RealModeBreak=ON/OFF

Hit an INT 1 early in RealModeInit.

Rehook123=ON/OFF

Blast INTs 1, 2, and 3 into the IDT.	To avoid trouble with WDEB386.

Now that SWAT is a Windows kernel debugger, there should be less need

to run WDEB386.