FreeWare  	     +++ ENH_FONT.EXE v5.7 +++		  	  FreeWare

A computer isn't a machine

but way of life.

(Programmers' joke)

Dear friends !

I propose the ENHANCED FONT DRIVER. I had to write this program because

many font  drivers which  I tested  did not load correctly Russian letters

(especially, when  I dealt with CGA-compatible modes). Plus I added pretty

useful  and   compact  keyboard   driver.  So,  judge  for  yourself.  Any

prepositions, hopes and notes are accepted with pleasure.

Well, what my driver can do.

Firstly, the driver can load national letters in  all extended modes of

EGA and  VGA adapters.  It allows  You to  work in spread-sheets using 132

columns. The  driver supports all known VGA adapters and nearly all  EGAs,

and works  with some  exotic video  modes for  Tseng  ET-4000, Video7 VGA,

VESA SuperVGA and ULTRAVISION.

Secondly, the driver can write font tables into the "shadow" video-BIOS

on those computers which have such an ability. In this case  the main  DOS

memory is  left for  system needs.  The maximal  driver's  size  for  VGAs

together with  the keyboard  driver is 11440 bytes, and minimal size (with

font tables  written into the "shadow" video-BIOS) - 1152 bytes. The  list

of chipsets supported by the driver is presented below.

Thirdly, You can  load font  tables and/or  the  keyboard  driver  into

UMB (Upper Memory Blocks -  memory  between 640Kb and 1Mb). Starting  from

MS-DOS version 5.0 users are  able  to load drivers into UMB using command

DEVICEHIGH (on 80386-based and higher computers). The switch "/U" gives an

auxiliary way to load into UMB via XMA 2.0  or  EMS LIM 4.0  protocols. In

this case  of  EMS, the driver tries to allocate itself in one of pages of

the expanded memory.

+------------------------------------------------------------------------+

|          The driver is fully FREEWARE and distributed "AS IS".         |

|          It can be freely used and distributed but NOT BE SOLD !       |

+------------------------------------------------------------------------+

ENH_FONT.EXE  is  installed  ONLY  AS  A DRIVER.  For this purpose  You

should insert the following string into the file CONFIG.SYS:

DEVICE= <Path>ENH_FONT.EXE <Switches>.

or

DEVICE= <Path>ENH_FONT.EXE @<command_file> (from ver. 3.1a).

For changing some parameters of  already  installed  driver You  should

run the  ENH_FONT.EXE  from  the  MS-DOS  command string. In this case the

driver is used as a monitor. The command file may be  256 characters long.

Switches

--------

/F[=<File_Name>] - "Font table"

For driver and monitor

----------------------

It loads  user-defined  font table.  For CGA adapter  may be loaded

font of  size of  8x8  pixels (with codes from 80h to FFh), for EGA

You  can load font  8x8 and 8x14  pixels, for VGA 8x16 pixels else.

The switch  "/F"  can  be  repeated  many times for loading various

font tables. For example, the string:

"ENH_FONT.EXE /F=8x8.fnt /F=8x14.fnt /F=8x16.fnt"

loads three font tables at once.

The font file consists of  256 x N  bytes,  where  N is a number of

bytes per one character.  Each  of  this  byte  is a  bitmap  which

forms shape of a character, and "N" corresponds to the font height,

e.g. 8, 14 or 16.

N O T E S

1). By  default there used font tables without Russian letter  "yo"

and other letters of Ukrainian and Belorussian national  alphabets.

For their  loading  You  should  use  font files with  extension of

"866", which  conform  to  the  standard  code page number  866 for

MS-DOS or DR-DOS.

2). The switch /F, typed without a file  name  reloads  either font

tables embedded into the EXE-file, or tables loaded  with  the very

last switch /F, followed by a file name. Thus, for example, the co-

mmand line "ENH_FONT.EXE /F=Font_8x8.Fnt /F" will load a 8 x 8 font

table which  was read from the file "Font_8x8.Fnt" but not that one

which was initially embedded into the driver. For font tables 8 x14

and  8 x 16 (in this example) there will be used embedded ones.

3). If  You  wish  to  patch font tables rigth into the body of the

driver ENH_FONT, You can do it with help of a program ENH_PATC.COM.

/K[=<Toggle-key>] - "Keyboard driver"

For driver and monitor

----------------------

It installs (in addition to the screen driver) the  keyboard driver

of  alternative   coding  with  standard  QWERTY/������  layout and

defines a key for toggling LATIN/NATIONAL KEYBOARD.

Toggle-keys may have the following names:

L_SHIFT - left SHIFT,    R_SHIFT - right SHIFT,

L_CTRL  - left CTRL,     R_CTRL  - right CTRL,

L_ALT   - left ALT,      R_ALT   - right ALT,

F11,                     F12,

MACRO   - used on some keyboards (from version 3.1a),

XMACRO  - non-standard "MACRO" used on some  keyboards. It produces

scan-code 56h, and ASCII code "\" (from version 5.0).

NONE    - special name, see comments below.

The  following  combinations mean that a key toggles mode  ONLY  if

the key left SHIFT has already been pressed (from version 2.3):

L_SHIFT+R_SHIFT - left SHIFT + right SHIFT,

L_SHIFT+R_CTRL  - left SHIFT + right CTRL,

L_SHIFT+L_CTRL  - left SHIFT + left CTRL,

L_SHIFT+R_ALT   - left SHIFT + right ALT,

L_SHIFT+L_ALT   - left SHIFT + left ALT,

L_SHIFT+F11     - left SHIFT + F11,

L_SHIFT+F12     - left SHIFT + F12,

L_SHIFT+MACRO   - left SHIFT + MACRO  (starting from version 3.1a).

L_SHIFT+XMACRO  - left SHIFT + XMACRO (starting from version 5.0).

If the  switch /K is entered with no name of toggle-key then R_CTRL

(right CTRL) is set by default.

The value "NONE" is used in two ways:

- For the DRIVER it means loading of the keyboard driver  WITHOUT

setting a toggle-key.

- For the MONITOR this value  blocks the keyboard driver  without

its unloading from the memory. In this case the  driver toggles

into LATIN mode  if BOTH LATIN and  NATIONAL KEYBOARD "hotkeys"

have values NONE (see below).

It sets "hotkey" for toggling into modes LATIN or NATIONAL KEYBOARD

apart  from  the main toggle-key  which is set with help of  switch

/H[=<Hotkey LATIN>,<Hotkey NATIONAL KEYBOARD>,<Temporary toggle>] -

"Hotkeys" (from version 3.2)

For driver and monitor

----------------------

It sets "hotkey" for toggling into modes LATIN or NATIONAL KEYBOARD

apart  from  the main toggle-key  which is set with help of  switch

"/K". You can also set a key for "temporary" toggling of keyboard's

mode. It means that keyboard's mode changes to the  opposite  while

the given key is pressed (starting from version 5.1).

Values  for the  first two key are the same as for the switch  "/K"

but for the "temporary toggle"  You  may  use  any of left or right

ALTs, SHIFTs and CTRLs.  The value "NONE"  is  used here to disable

any of these keys.

In order to change any of these "hotkeys" You can use the following

commands:

/H=,F12		 - Changing "hotkey" of NATIONAL  KEYBOARD

only;

/H=F12, or /H=F12 - Changing "hotkey" LATIN only;

/H=,,L_SHIFT	 - Changing a key for  temporary  toggling

of keyboard mode;

/H or /H= or /H=, - Leave all the "hotkeys" unchanged.

/M=<Delay><Rate> - "Program keyboard typeMatic rate" (from version 5.1)

For driver and monitor

----------------------

It allows to set keyboard's typematic rate and delay.

"Delay" determines a typematic delay in seconds and it is expressed

by a hexadecimal number:

0  - 0.250,   1  - 0.500,   2  - 0.750,    3  - 1.000.

"Rate"  determines keyboard typematic rate in characters per second

and it is expressed by a hexadecimal number in the range from 00 to

1F:

00 - 30.0,    01 - 26.6,    02 - 24.0,     03 - 21.8,

04 - 20.0,    05 - 18.4,    06 - 17.1,     07 - 16.0,

08 - 15.0,    09 - 13.3,    0A - 12.0,     0B - 10.9,

0C - 10.0,    0D -  9.2,    0E -  8.5,     0F -  8.0,

10 -  7.5,    11 -  6.6,    12 -  6.0,     13 -  5.4,

14 -  5.0,    15 -  4.6,    16 -  4.2,     17 -  4.0,

18 -  3.7,    19 -  3.3,    1A -  3.0,     1B -  2.7,

1C -  2.5,    1D -  2.3,    1E -  2.1,     1F -  2.0.

Recommended variants:

/M=000 - Maximal typematic rate;

/M=31F - Minimal typematic rate;

/M=10F - Medium typematic rate;

/M=10C - Typematic rate set by default just after computer restart.

/Z[=Y/N] - "Zero-filling for scan-codes" (starting from version 3.2).

For driver and monitor

----------------------

It sets a sort of filling for scan-codes of national alphabets let-

lers. By default, the driver sets up national letters scan-codes to

null (0)  (many keyboard drivers do the same). To get a real  scan-

code of a button pressed You should type the following key: /Z=N.

The key /Z, entered withiot "Y" or "N" will set up zero-filling.

/T=<Toggles_Names> - "Toggles on loading" (starting from version 3.2).

For driver

----------

It sets the toggles Num Lock, Scroll Lock, Caps Lock or Ins to "ON"

when the driver is loaded.  You can type  these names in  arbitrary

order and one after other without a space, for instance, /T=CSN.

The toggles names may be the following:

C - Caps Lock, N - Num Lock, S - Scroll Lock, I - Insert.

/B[=<Border_Color>] - "Border color"

For driver and monitor

----------------------

It sets  a color  or  screen  border  when  toggling  into mode  of

NATIONAL KEYBOARD.

The color may accept the following values:

0 - Black,              8 - Gray,

1 - Blue,               9 - Light-blue,

2 - Green,             10 - Light-green,

3 - Cyan,              11 - Light-cyan,

4 - Red,               12 - Light-red,

5 - Magenta,           13 - Light-magenta,

6 - Brown,             14 - Light-yellow,

7 - White,             15 - Bright-white.

S - Usage of "Scroll Lock" indicator LED (in driver's version 2.0).

L - Usage of literal indication of mode in the upper-left corner of

the screen (in version 2.2) where letters  show  the following:

L - English (Latin); � - Russian;  � - Ukrainian;

� - Belorussian;     A - Armenian.

The switch  /B without  border color  value sets color 2 (Green) by

default.

/L[=<Language_layout>] - "Layout/language" (starting from version 1.2)

For driver and monitor

----------------------

It sets key layout for the keyboard. In this  version the following

layouts are implemented:

0 - Russian standard layout ������,

100 - Russian standard with letter "�" (yo),

200 - Russian standard with "(" and ")" instead of "?" and "%",

300 - Combination of two above-mentioned layouts.

1 - Ukrainian layout,

101 - Ukrainian (national standard PCT 2019-91 from 01.01.92).

2 - Belorussian layout

3 - Armenian layout (for typewriters).

The Russian layout (0) is set by default.

In order to  load  any  user-defined  layout  You  should  use  the

following format:

/L=?,<name_of_a_file_containing_the_layout>

The file containing the user-defined layout is created with help of

ENH_EDIT.EXE (presents in the distribution).  The  program  is  ran

with the command string:

ENH_EDIT.EXE <new_or_existing_layout_file>

The program allows to  setup  the  keyboard in  four  modes:

F1 - Original national layout (e.g. lowercase letters);

F2 - Original nat. layout + Shift (e.g. uppercase letters);

F3 - Original nat. layout + CapsLock;

F4 - Original nat. layout + CapsLock + Shift.

Besides, pressing the key  INS  You can call full ASCII table  in

order to insert any character from the set.

If You wish to patch a keybord layout rigth into the  body  of  the

driver ENH_FONT, You should use the same program ENH_PATC.COM.

/G[=Y/N] - "Gong"

For driver

----------

It  turns   on   bell   sound  when  toggling the   keyboard  modes

LATIN/NATIONAL KEYBOARD. By default the bell sound is turned OFF.

For monitor

-----------

It turns ON/OFF  the  bell  sound.  You should type  "/X=G" to turn

the bell sound OFF.

In both cases the switch /G  typed without  "Y"  or  "N"  turn  the

bell sound ON.

/X[=Y/N] - "FOXpro characters translation"

For driver

----------

It turns on  translation  of  Russian  letters  "H"  and  "p"  into

analogous by script  Latin letters  "H" and  "p" for  working  with

FOXPRO and FOXbase.

By default the letters are not translated.

For monitor

-----------

It turns ON/OFF such a translation.  You should type "/X=N" to turn

the translation OFF.

In both cases the switch /X  typed without  "Y"  or  "N"  turn  the

translation ON.

/W[=Y/N] - "MS WORD characters translation"

For driver

----------

It  turns  on  translation  of Russian letter "o" into analogous by

script Latin letter "o" for working with Microsoft WORD.

By default the letters are not translated.

For monitor

-----------

It turns ON/OFF such a translation.  You should type "/W=N" to turn

the translation OFF.

In both cases the switch /W  typed without  "Y"  or  "N"  turn  the

translation ON.

/S[+|!|~|1|2[=<Chipset_number>]] - "Shadow writing"

For driver and monitor

----------------------

It attempts to write font tables into "shadow" video-BIOS.

The driver automatically defines adapters which  have  font  tables

with  non-standard sizes 8 x 11 and 8 x 19, and writes proper ones.

Starting from driver's version 5.0  You can apply a number of chip-

set  with  "shadow"  mode  ("Chipset_number").  Switch  "/S"  typed

without chipset number will be looking through all the types.

Now the following memory control chipsets are supported:

Version 1.0, 15-Mar-92:

-----------------------

1. Chips and Technologies CS8230 386/AT, CS8231 Turbo Cache 386/AT,

CS8233 PEAKset, CS82310 PEAKset DM/386 (386DX/486DX with cache).

Chip ID(s):	82C302, 82C307, 82C311, 82C351.

2. Chips  and Technologies  CS8221 NEAT,  CS8281 NEATsx (other name

NEAT-205), CS8223 LeAPset, CS8283 LeAP-sx;

Texas Instruments (TI) TACT82S411 Single Chip AT (SCAT);

United Microelectronics (UMC) UM82C210.

Chip ID(s):	( C & T ) 82C212, 82C241 (286);

82C812, 82C841 (386SX).

(   TI  ) TACT82S411     (286 & 386SX).

(  UMC  ) UM82C210       (286 & 386SX).

3. Chips and Technologies 82C235 SCAT, 82C836 SCATsx, CB8291 ELEAT,

CB8295 ELEATsx,  CS8285  PEAKset-sx,  CS8227  CHIPSlite,  CS8288

CHIPSlite-sx.

Chip ID(s):	82C235 (for 80286), 82C836 (for 80386SX).

4. Headline Technology SCAT HT12 for 80286.

Chip ID(s):	82C206, 82C212.

5. VLSI Technology SCAMP, VL82C486.

Chip ID(s):	VL82C310, VL82C311 (286 and 386SX);

VL82C311L	   (286);

VL82C486-FC2	   (386DX and 486SX).

6. OPTi  Sx/AT, Sx/AT  Cache for 386/AT (80386SX with cache).

Chip ID(s):	82C281, 82C282, 82C283.

7. Unknown chipset for 80486 (AMI BIOS).

Version 1.1, 04-May-92:

-----------------------

8. United Microelectronics (UMC) UM82C481 for 80386 or 80486.

Chip ID(s):	UM82C481.

Version 1.3, 07-Dec-92:

-----------------------

9. ETEQ ET82C4901/ET82C4903 for 80386 (w/ cache) or 80486.

Chip ID(s):	ET82C4901, ET82C4903.

Version 2.0, 06-Jan-93:

-----------------------

10. ACC Microelectronics 2036 Chipset for 80386 or 80486.

Chip ID(s):	ACC 2036.

11. Chipset of HP Company for machines HP VECTRA (386 or 486).

Chip ID(s):	Unknown.

Version 2.1, 15-Jan-93:

-----------------------

12. FOREX FRX36C300/FRX46C400

Chip ID(s):	FRX36C300	      (386DX);

FRX46C402 & FRX46C411 (386DX w/ cache).

Version 2.2, 04-Feb-93:

-----------------------

13. Western Digital WD7600 for 80386.

Chip ID(s):	WD76C10, WD76C20, WD76C30.

Version 2.3, 24-Feb-93:

-----------------------

14. VLSI Technology VL82C200 for 80286 or 80386SX.

Chip ID(s):	VL82C201, VL82C202, VL82C203, VL82C204

(all 4 chips required).

15. Elite Microelectronics Eagle, Falcon

Chip ID(s):	e88C311 (386DX);

e88C411 (486).

16. OPTi DX/BB PC/AT for 80386DX or 80486.

Chip ID(s):	82C496.

Version 2.4, 18-Mar-93:

-----------------------

17. OPTi HiD/386 AT, HiB/486 AT.

Chip ID(s):	82C382 (386);

82C482 (486).

18. OPTi 386WB PC/AT, 486SXWB PC/AT;

Chips and Technologies PC Chips (80386DX w/ cache).

Chip ID(s):	(  OPTi ) 82C391 (386DX);

82C493 (486);

82C495 (486SX). (*)

( C & T ) Chip 5 and Chip 6.

19. ACER M1217 for 80386SX.

Chip ID(s):	ACER M1217.

Version 3.0, 01-Jul-93:

-----------------------

20. ALD Technology 93C305 Single Chip for 80386SX.

Chip ID(s):	ALD 93C305.

21. ACER Laboratories M1419 A1 Single Chip for 80386DX with cache.

Chip ID(s):	ALI M1419 A1.

Version 3.1, 31-Aug-93:

-----------------------

22. VLSI Technology VL82C286, VL82C386 TOPCAT;

Intel 82340SX, 82340DX.

Chip ID(s):	( VLSI )  VL82C320     (286, 386SX);

VL82C330     (386DX);

VL82C320A    (286, 386SX, 486).

( Intel ) 82343, 82346 (286, 386SX);

82343A       (286, 386SX, 486).

Version 3.2, 29-Oct-93:

-----------------------

23. Chipset MXP9 of unknown vendor for 80386SX with cache.

Chip ID(s):	MXP9.

24. United Microelectronics (UMC) UM82C336 for 80386SX.

Chip ID(s):	UM82C336.

Version 3.3, 01-Nov-93:

-----------------------

25. USA Single Chip 9204 for 80386DX and 80486DX with cache.

Chip ID(s):	USA SC-9204.

26. ACC Microelectronics 2168 Chipset for 80386 or 80486 w/cache.

Chip ID(s):	ACC 2168.

Version 3.4, 16-Nov-93:

-----------------------

27. Headline Technology Advanced Memory Manager Chip HT18;

Chipset CHIP2/3 of PC CHIPS Inc.

Chip ID(s):	( Headland ) HT18            (286, 386SX);

( PC CHIPS ) CHIP2 and CHIP3 (286, 386SX).

Version 4.0, 10-Dec-93:

-----------------------

28. Silicon Integrated Systems High Performance ISA486 chipset.

Chip ID(s):	SIS 85C401 for 80486.

Version 4.1, 11-Jan-94:

-----------------------

28. WINBOND Single Chip W83C320 for 80386SX.

Chip ID(s):	W83C320.

Version 4.2, 16-Mar-94:

-----------------------

30. United Microelectronics (UMC) UM82C491 for 80386/486 w/ cache.

Chip ID(s):	UM82C491. (*)

Version 4.3, 18-Aug-94:

-----------------------

31. Chipset for DECpc LPx 433dx for 80486 (66MHz) with cache.

Chip ID(s):	Unknown.

Version 5.0, 27-Nov-94:

-----------------------

32. Silicon Integrated Systems High Performance chipset.

Chip ID(s):	SIS 85C460 ATQ	(386);

SIS 85C471	(486DX).

33. OPTi 486WB 82C498 for 80486 with cache.

Chip ID(s):	82C498.

34. CONTAQ 82C596 for 80486 with cache and AMI BIOS of 1992.

Chip ID(s):	82C596A. (*)

Version 5.1, 21-Feb-95:

-----------------------

35. ACER M1429 for 80386DX or 80486DX with cache.

Chip ID(s):	ACER M1429. (+)

36. Texas Instruments for 80286 or 80386SX.

Chip ID(s):	TACT83000/83442.

Version 5.2, 17-Mar-95:

-----------------------

37. Chipset VIA GMC VT82C482A for 80386DX or 80486DX with cache.

Chip ID(s):	VT82C482A. (*)

Version 5.3, 07-Aug-95:

-----------------------

38. Chipset HiNT CS8005 for 80486SX/DX/DX2 with cache.

Chip ID(s):	HiNT CS8005.

Version 5.4, 21-Feb-96:

-----------------------

39. United Microelectronics (UMC) UM82C498 for 80386/486 w/ cache.

Chip ID(s):	UM82C498 (UM8498F). (+)

40. OPTi 596/597/822 for Pentium with PCI.

Chip ID(s):	82C596. (+)

Version 5.6, 11-Jun-96:

-----------------------

41. Chipset Triton of Intel Corporation for Pentium.

Chip ID(s):	Intel Triton.

Version 5.7, 23-Jul-96:

-----------------------

42. Silicon Integrated Systems High Performance chipset.

Chip ID(s):	SIS 85C4966	(486DX).

N O T E S

1.)  For  correct  writing  of fonts into "shadow" memory You  MUST

set up "shadow" onto BOTH  16-Kb partitions of video-BIOS,  e.g. on

address ranges C0000...C3FFF and  C4000...C7FFF (for  laptops it is

as a rule E0000...E3FFF and  E4000...E7FFF). Besides,  for  laptops

with  chipset  of  Chips and Technologies CS8281 NEATsx (82C812  or

82C841, see list, number 2) You should set "shadowing" onto all the

range E0000...EFFFF.

2.)  On  some  contemporary  fastest  computers  there can appear a

strange effect when  the driver  cannot write into  "shadow" memory

even if correct settings of the "shadowing" are made in  computer's

SETUP.  This takes place, for instance,  with  chipsets  OPTi 386WB

PC/AT (see list, number 18),  and  with  all  chipsets marked  with

asterisk  (*) in the list.  It turns  to  be because  of additional

video-BIOS  caching. Nowaday, I still haven't been  able  to "beat"

this problem down completely by program technique. Thus,I recommend

You either to disable this video-BIOS caching in  machine SETUP, or

attempt to disable the caching in the range  of  C0000...C3FFF  and

C4000...C7FFF (or E0000...E3FFF and E4000...E7FFF), or set up cach-

ing mode as non-WRITE-BACK, if Your machine's SETUP allows it.

For correct operation with the CONTAQ 82C596 chipset (see list

number 34)  You  SHOULD either disable video-BIOS  caching, or turn

this  caching  into the mode WRITE-BACK-DIRTY (WB-Dirty in terms of

SETUP), or into the mode WRITE-THROUGH (WT).

3.)  Sometimes any problems can occur  with other keyboard drivers.

This is because of the driver defines the addresses of ROM-fonts in

video-BIOS using INT 10h, function AX=1130h but other keyboard dri-

ver returns incorrect value. The result is memory corruption, crash

of the system and some other troubles. My recommendation is:  don't

use any keyboard drivers that hook INT 10h together with my driver.

4.)  If You use  expanded memory manager QEMM386.SYS of  Quaterdesk

Corp. of version 6.0  and higher  with  the  switch  "/ST" (stealth

memory), then the mode "F" (/ST:F) doesn't create any problems  for

fonts written into the shadow memory. From the other side, the mode

"M" (ST:M) sets up pretty cruel mode for searching UMB blocks which

"smashes down" any "shadow" memory. This problem may be solved with

two methods proposed by Serge Topilsky from Karaganda (thank  him a

lot for that).

Method 1:

---------

device=c:\qemm\dosdata.sys

device=c:\qemm\hookrom.sys

device=c:\drv\enh_font.exe /s /k /h=,,r_alt /b=l /g /m=0 /q

device=c:\qemm\qemm386.sys RAM ST:M BF:N SH:N I:0800-0FFF

device=c:\qemm\dos-up.sys

The driver  HOOKROM.SYS  is  intended  namely for such  situations.

It is described in Quaterdesq's documentation. The loading order is

CRITICAL. DOSDATA/DOS-UP may be used if You wish.

Method 2:

---------

device=c:\qemm\dosdata.sys

device=c:\drv\enh_font.exe /s /q

device=c:\qemm\qemm386.sys RAM ST:M BF:N SH:N I=0800-0FFF

device=c:\qemm\dos-up.sys

devicehigh=c:\drv\enh_font.exe /r /k /h=,,r_alt /b=l /g /m=0 /q

More  elegant variation, without  usage  of any additional drivers.

Fonts are loaded into shadow BEFORE stealth-mode, then the keyboard

driver is loaded alone.

For successful writing  of fonts into shadow memory You should

use QEMM386.SYS without /ROM as well.

5.)  If You use some  extender  (accelerator)  for Your video-BIOS,

for example,  RAMBIOS.SYS  by  Cirrus Logic,   or the same for  the

appropriate EGA/VGA card, You should place the driver  ENH_FONT.EXE

BEFORE this extender (accelerator) in the file CONFIG.SYS.

S W I T C H   M O D I F I E R S

1.)  The switch "/S+" is used  in the case  when it is possible  to

open the "shadow" memory for reading/writing.  Some UMB drivers are

able to do it. For instance,  UMB_DRVR.SYS by  Christopher Blum  of

version 5.22 and higher, or LASTBYTE.SYS by KEY SOFTWARE PRODUCTS.

2.)  The switch "/S!"  is  used  for some VGA adapters, on which in

spite  of  successful writing of fonts into "shadow" memory, letter

of  a  national  (non-english) alphabet is displayed incorrectly in

the standard text  modes.  Such  takes place on some notebooks with

VGA of  Cirrus  Logic Inc. where a font 8 x 19 is  used. Also, this

probably will  take  place on some other VGA's when the driver will

not  "see" non-standard  fonts there.  Thus  You should placee  the

given modifier "!". In this  case  a little portion  of  correcting

code is loaded.

Another usage of this switch is  correction  of  a  strange  effect

taking place on some fastest machines when regardless of successful

writing into "shadow" memory, not all national letters are display-

ed O'K. This is because of loading these letters from other part of

video BIOS while text mode reset.  In this case the switch "/S!" is

equivalent to the switch "/R+" (see below).

3.)  The switch "/S1" or "/S2"  (starting from version 5.4)  allows

to use various techniques of writting of font into "shadow" memory.

Chipsets marked in the list with cross mark (+)  (for instance, #35

- ACER M1429),  have  two techiques  of shadow usage.  Thus, in the

case  of  failure writting with help of the switch "/S"  You should

try these modifiers.

4.)  The  switch  "/S~"  (starting from version 5.4)  is  SPECIALLY

intended for working  together  with  Expanded Memory Manager (EMM)

QEMM386.SYS, and  uses  its  uniq capability to move the video-BIOS

from ROM into RAM in mode "/ST:M". In this case, QEMM386 makes some

kind  of  pseudo-"shadow", opened for reading/writting.  Thus,  the

switch "/S~" is equivalent to the switches "/S+" and "/S!" but with

alternative method of loading fonts when text mode resets. The swi-

tches "/S~" and "/S!" are not combined. Also,this modifier helps in

that  moments  when a  computer have not "shadow" memory at all, or

when memory  chipset is unknown for the driver.  Sometimes for suc-

cessfull  writting into  pseudo-"shadow"  You  will  have  to "map"

video-BIOS with help of QEMM386's option "/ROM=C000-C7FF".

ted with the fact that the author's computer just has "shadow"  but

because of  QEMM386's highly  "intellectuality"  this  manager  can

distinguish "shadow" somehow and optimizes memory.  In any case the

author couldn't "push" video-BIOS into RAM. On the contrary, EMM386

"franckly" rewrites BIOS into other place.  The author didn't expe-

riment with other memory managers and will be grateful  to  anybody

5.)  The switch "/S#" (starting from version 5.5)  uses some tricky

technology of writting into "shadow" and it is going in two stages:

a). If video-BIOS  is already "moved" (mapped) from ROM into  RAM

by some  EMM (Expanded Memory Manager) then  the given switch

works like the switch "/S~" does. It is known that the memory

managers set virtual protected mode V86 and page memory orga-

nization. This allows to "map" any linear  address  onto  any

physical  point in memory. In  this  mode the switch works on

100% with the memory manager EMM386.EXE by Microsoft Corp. In

this case You should use its option "/ROM=C000-C7FF" (though,

as a rule the value "/ROM=C400-C7FF" is suitable).  With  the

manager  QEMM386  by  Quaterdesk  author  still can't get any

positive result (it  is  needed the same option "/ROM=..." or

"ARAM=...").

�). If the first  stage  has  finished  with  failure, the driver

attempts  itself  to  "move"  (map)  some parts of video-BIOS

which contain font table from ROM into RAM. For this  purpose

the mechanism  of  XMA 2.0 is  used. In this case the pseudo-

"shadow" works with any memory manager. Maximal size of memo-

ry taken from extended memory is no more than 16 Kb.

The switch "/S#" is not combined with the switches "/S+" and "/S~".

These modifiers can be combined together, e.g. "/S+!", "/S1!".

A T T E N T I O N !

Since the  driver attempts to distinguish  all known memory control

chips reading proper computer's ports then the usage of this switch

may (though,  not always)  hung up  some machines  which DON'T HAVE

memory "shadowing". So, if You are not sure, You'd better skip this

switch. In  any case  I recommend You to check the ability to write

into "shadow" memory just typing the following command (taking into

account the above-mentioned demands): " ENH_FONT.EXE /S /F ".  When

positive result the driver will print a number of supposed  chipset

as described in the above-mentioned list.

/U - "UMB allocation"

For driver

----------

It tries to allocate the font table and/or the keyboard into UMB.

/R[+] - "ROMed font tables"

For driver

----------

It is applied for EGA/VGA  adapters  with  ROMed  Russian  letters.

Together with  switch /K  it loads the keyboard only.

Switch "/R+" should be used for some not very  correctly  "patched"

VGA adapters. These adapters display Russian letters  "�", "�", "�"

and some others in a  wrong way.  It is  connected  with  the  fact

that these letters are loaded from another font  table  inside BIOS

during text mode resetting.

/C or /E - "Force detection of CGA or EGA adapter" (from version 5.0)

For driver

----------

They are  used for  some  non-standard  or  merely "bugged"  video-

adapters which are detected neither as a CGA nor a EGA.  Thus,  the

appearence of one of  these switches forces the  driver to consider

the video-adapter, respectively, as a CGA or as a EGA.

/2 - "Load font table into the 2nd block" (starting from version 5.0)

For driver

----------

It is applied  for  laptops  and  notebooks  which  use  additional

"bold" fonts.  During  every resetting  into TEXT video mode,  the�

screen driver will  load an appropriate  font into the 2nd block of

the 2nd bit plane as well.

/Q - "Quiet" (starting from version 2.4)

For driver and monitor

-----------------------

It disables all messages.

/P - "Pause" (starting from version 3.2)

For driver

----------

It stops printing of messages on the screen untill a key is pressed.

/I[+] - "Get Information" (starting from version 3.2)

For monitor

-----------

It shows information on current settings of the keyboard driver.

The  switch  "/I+"  is used for obtaining an additional information

about  chipsets supported  in  the  current  version  of the driver

(from version 5.4).

*** CALL-interface to the driver from users' programs (in version 2.5) ***

Checking of presence of the driver in the system:

mov	ax,14FFh	; Loading of a special code.

int	10h

It must return AX = 4F66h ("Of");

BX = Starting segment of loaded driver ENH_FONT.

The control over the driver is carried out via INT 2Fh (by Microsoft standard):

mov	ax,0AD83h	; Function 83h - Get driver's state.

int	2Fh

Returns AX = 1 - Latin mode is set;

2 - National keyboards mode is set.

mov	ax,0AD84h	; Function 84h - Change driver's mode.

mov	bl,<CODE>

int	2Fh

where <CODE> can have the following values:

0 - Enable Latin mode;

1 - Enable national keyboards mode.

Author  thanks all the users who sent their prepositions and notes, and

especially mister Vladimir Shneider from "KAMBI" firm, and mister  Michael

Gorodetsky from Moscow State University for valuable information and  code

for operating with chipset HT-18 (CHIP2), and BIOS from SIS 85C401.

Copyright (c) 1992-96 by Goryachev Dmitry A.

All Rights Reserved.

Sosnovy Bor, Saint-Petersburg region, Russia, 188537.

E-mail: dimon@sus.spb.su