Mouse++ Version 4.0

Copyright (c)1992 by Carl Moreland

10/06/92

-----------------------------------------------------------------------

Using the Mouse Class

To  incorporate  the  mouse routines in your  program,  simply  add

mouse.lib to your project or make file and #include mouse.h in any  mo-

dule that calls a mouse function.  mouse.lib contains all the functions

in mouse.cpp and cgc.cpp,  but the functions are separated into differ-

ent object modules for greater efficiency.  Note that mouse.lib is com-

piled for the large memory model.

An instance of the Mouse class (there can only be ONE instance)  is

declared as extern in mouse.h,  so any file that #includes mouse.h will

automatically have access to the mouse.  You should NOT declare an  in-

stance of the Mouse class in any part of your program. If you intend to

change the mouse cursor, then you also need to #include cursor.h.

When you link the mouse code into your program,  an instance of the

mouse class is declared:

Mouse mouse;

The  constructor Mouse::Mouse() calls the mouse driver  reset  function

(all mouse driver functions are called via interrupt 33h)  and  initia-

lizes  the flags exists, enabled, and visible.  exists is set to 1 if a

mouse  was  found and should be checked first.  All other  mouse  class

functions check exists before issuing interrupt 33h calls and abort  if

it  is zero.  enabled is initially zero meaning the mouse is  disabled,

and  visible is used by Show() and Hide() to keep track of  the  cursor

visibility. An example of initializing the mouse is:

#include "mouse.h"

main()

{

if(mouse.Exists())		// check for mouse

{

mouse.Enable();		// enable the mouse

mouse.Show();		// display the cursor

...

}

}

Again,  including  mouse.h automatically creates  an  instance  of  the

class, so all you have to do is start using it.

Once  the  mouse is initialized,  it may be necessary to  set  some

parameters based on what video mode is being used. Strangely, the mouse

driver  uses  pixel coordinates for both text and graphics  mode,  with

(0,0)  being  the upper left hand corner of the screen,  and  (639,199)

being  the default lower right hand corner.  This would be correct  for

CGA graphics mode and for normal 80x25 text mode - for text mode,  this

means that each character cell is treated as an 8x8 pixel array. If you

are programming for the EGA or VGA as is the norm these days,  you will

want  to change the y-limit of 199 for high resolution graphics and  43

or 50 line text modes.  Failure to do this means that the mouse  cursor

will not move below the 200th line in graphics mode, or the 25th row in

text mode.  (Older mouse drivers might not support more than 200 lines.

If  this is the case,  you will need to update your mouse driver.)  For

640x480 VGA graphics, you need to set the y-limit to (0,479) by calling

yLimit(0,479).  For  43 or 50 line text mode,  the mouse  driver  still

treats  each character cell as an 8x8 pixel array,  so you need to  set

the  y-limit  to (0,349) or (0,399), respectively.  Since  the  default

width  of 640 pixels is correct for most video modes,  xLimit() is only

necessary  when using SuperVGA graphics.  (See the section on  SuperVGA

support) Both xLimit() and yLimit() can also be used to limit the mouse

cursor to a small portion of the screen, such as a menu.

You  may want to use a mouse cursor other than the default  cursor.

To do this,  call the function SetCursor() with the appropriate  cursor

name.  Many programs (especially graphics) will use  several  different

cursors  depending on the location of the mouse (such as an  arrow  for

menus  and  an i-beam for text) or the particular function  being  pro-

cessed (such as an hourglass for wait).  Several cursors are predefined

in the file cursor.h.  To use them,  simply include this header file in

whatever  module changes the cursor.  For more information on  cursors,

see the Mouse Cursors section.

Finally,  you may want to set the motion parameters,  which include

the  mickey-to-pixel ratio and the double-speed threshold.  The mickey-

to-pixel  ratio  is set by MickToPix() and defines how many mickeys  it

takes  to move the mouse 8 pixels.  (A mickey is single count of  mouse

motion. Most mice are 200 "dots-per-inch", which means that one inch of

movement  results in 200 mickeys.)  If both the x and y parameters  are

set  to 8,  then there will be a one-to-one correlation  between  mouse

motion  and cursor motion.  The default values are 8 in the  horizontal

direction and 16 for the vertical.  The vertical value of 16 means  the

mouse must be moved twice as far vertically as horizontally to  get the

same cursor movement.  This is fine for text mode where the  equivalent

screen  resolution  of 640x200 results in an  equivalent  pixel  aspect

ratio of 2.4-to-1.  In EGA/VGA graphics mode the y-direction will  seem

noticeably  slower,  particularly for 640x480 which has  an  equivalent

pixel  aspect ratio of 1-to-1.  A vertical ratio of 8 will  cure  this.

The lower the ratio, the farther the cursor will move for a given mouse

movement.  Setting either ratio lower than 8 means that the cursor can-

not be located on every pixel. For example, setting the ratio to 4 will

cause  the cursor to move 2 pixels for every mickey.  The  double-speed

threshold  is set by a call to SetSpeedThreshold().  When the speed  of

the mouse (in mickeys per second) exceeds the speed parameter passed to

SetSpeedThreshold(), the motion speed of the cursor will double. A com-

plete initialization might look like this:

#include "mouse.h"

#include "cursor.h"

main()

{

{ initialize screen };

if(mouse.Exists())

{

mouse.SetCursor(cross);

mouse.yLimit(0,479);	// VGA 640x480

mouse.MickToPix(8,8);

mouse.SetSpeedThreshold(32);

mouse.Enable();

mouse.Show();

...

}

}

The  mouse should be reset before your program terminates  so  that

the  calling program does not inherit any strange parameters.  This  is

particularly true of the event handler. Because it is an interrupt rou-

tine,  failure to reset the mouse could lead to a system crash  if  the

handler is still pointing to the address of what used to be your  hand-

ler routine. The destructor Mouse::~Mouse() first resets the mouse sta-

tus by calling function 00h, and then restores the original event hand-

ler with function 14h.

-----------------------------------------------------------------------

Reading the Mouse

Most  of the standard mouse functions stuff any return  values  di-

rectly  into the class variables and have a return type void.  This  is

because many mouse functions return more than one item of  information.

You  can then use the appropriate inline function to read the  required

private  variable.  To check for the mouse position, for instance,  you

would call Position() followed by either x() or y(), or both, such as:

for(;;)

{

mouse.Position();

if(mouse.x() > 320 || mouse.y() > 100)

do_something();

}

This  method has the advantage of capturing both the x and y  positions

with  a single function call,  and those variables can then be read  at

your  leisure.  However,  we generally don't care where  the  mouse  is

located  unless  a button event that we are looking for  has  occurred.

Position() also returns the status of the mouse buttons, so we can also

check them:

for(;;)

{

mouse.Position();

if(mouse.LB_Dn())		// check for left button down

{

if(mouse.x() > 320 || mouse.y() > 100)

do_something();

}

else

do_something_else();

}

This  loop will continuously check to see if the left button  has  been

pressed.  If the do_something_else() code is slow then there is a  pos-

sibility  that a button press could be missed since Position()  returns

the  real-time status of the mouse.  That is, during execution  of  the

do_something_else()  code,  the mouse button could be pressed  and  re-

leased and Position() would not capture it. The functions Pressed() and

Released() are best suited for checking for a button event because they

will return the button status since the last time they were called. For

example, this code

for(;;)

{

if(mouse.Pressed(LEFTBUTTON))

{

if(mouse.x() > 320 || mouse.y() > 100)

do_something();

}

else

do_something_else();

}

is guaranteed to capture a button press even if the do_something_else()

code is slow to execute. Since Pressed() and Released() also return the

cursor position, a separate call to Position() is not necessary.

In using the mouse,  it is often necessary to know if the cursor is

located  within a certain area of the screen.  For example,  in a  menu

interface,  you want to know if the cursor is located on a menu  selec-

tion  when a mouse button is clicked.  The function InBox()  adds  this

capability.  The parameters passed describe the upper  left  and  lower

right corners of the box. The return value is 1 if the cursor is in the

box, 0 if not. Therefore, the code

if(mouse.InBox(40,40,100,100))

will  be true if the mouse cursor is in a box with corners (40,40)  and

(100,100). Like other mouse functions, the coordinates are pixels, even

if the screen is in text mode.

In some cases you may want to hide the cursor if it falls inside  a

certain area.  The mouse driver provides such a function which,  in  my

opinion, has two drawbacks. Function 10h defines an exclusion window in

which  the mouse cursor will turn itself off.  However,  the programmer

must  then manually turn the cursor back on once it is outside the  ex-

clusion area,  and this requires continuously checking the mouse  posi-

tion, such as:

for(;;)

{

mouse.Exclude(40,40,100,100);

do_some_processing();

if(!mouse.InBox(40,40,100,100))

mouse.show();

}

The  other problem with function 10h is that it only looks at  the  hot

spot  when checking the cursor's position, so part of the cursor  could

enter the exclusion area before the hot spot. The whole point of defin-

ing  an exclusion area is to turn the cursor off if it enters  the  de-

fined area. The Mouse++ Exclude() function is based on the InBox() fun-

ction, taking into account the hot spot location,  cursor image  height

and width, and cursor visibility.

Finally,  if  your program is performing a time-consuming  task  in

which mouse input is not needed, you may want to disable the mouse.  If

the  mouse  is not disabled,  the user might become frustrated  with  a

mouse  that does not respond, and the event buffer could become  filled

with useless events. Calling the function Disable() will turn the mouse

off and disable the interrupt handler. The mouse and interrupt  handler

are re-enabled with the next call to Enable(),  although the mouse cur-

sor must be turned on with an explicit call to Show():

...

mouse.Disable();

do_some_long_processing();

mouse.Enable();

mouse.Show();

...

-----------------------------------------------------------------------

Mouse Cursors

Mouse++ currently supports standard text and graphics cursors,  and

has rudimentary support for ColorGraphicsCursors. The mouse cursor type

and  shape are set by the function SetCursor().  In graphics mode,  the

cursor  is initially set to the familiar upper left pointing  arrow  by

the  mouse driver.  A standard graphics cursor is up to 16x16 pixels in

size  and moves in single pixel increments.  It is defined by a  screen

mask  (background),  an overlying cursor mask (foreground),  and a  hot

spot. Since the cursor is 16x16 pixels, a single-pixel hot spot must be

defined so that a unique pixel position can be determined for the  cur-

sor. The hot spot has x and y values with valid ranges of 0 to 15.

The two masks determine how the screen will appear where the cursor

is located.  The screen mask is first ANDed with the screen pixels, and

the  cursor mask is then XORed with the resulting screen.  If a  screen

mask bit is 0 it will set the underlying screen pixel to 0 (black), and

if  the screen mask bit is 1 the pixel color will not change.  A cursor

mask bit of 0 does nothing, and a cursor mask bit of 1 will invert  the

underlying pixel color.  Since the masks have only values of 1 or 0 for

each  pixel  location,  but each pixel can have  a  color,  you  cannot

achieve  complete control of the cursor color using a standard  cursor.

The  two  basic choices are a black and white cursor where  the  screen

mask bits are 0,  or a cursor that inverts the screen colors where  the

screen mask bits are 1.

Figure 1  shows the default cursor as an example.  The screen  mask

alone will set the screen pixels to black for the bits that are 0, cre-

ating a black arrow.  The cursor mask bits that are 1 will then  invert

the screen color, meaning part of the black arrow will become white. If

you lay the cursor mask directly over the screen mask,  you will notice

that  the  screen mask arrow extends one pixel beyond the  cursor  mask

arrow.  Because a screen mask bit 0 will create a black background  for

the cursor mask, this has the effect of putting a one pixel black  bor-

der around the cursor.  The reason for doing this is to make the cursor

visible  even where the screen background is white.  Making the  screen

mask  bits all 1's so that the cursor XOR's with the underlying  screen

is  useful in creating a CAD cursor (such as a "+" or a "x") that  must

be  one pixel wide.  The best way to get a better understanding of  the

graphics cursor is to try some out.  cursor.h defines several  graphics

cursors.

-----------------------------------------------------------------------

0011111111111111	0000000000000000

0001111111111111	0100000000000000

0000111111111111	0110000000000000

0000011111111111	0111000000000000

0000001111111111	0111100000000000

0000000111111111	0111110000000000

0000000011111111	0111111000000000

0000000001111111	0111111100000000

0000000000111111	0111111110000000

0000000000011111	0111111111000000

0000000111111111	0111110000000000

0001000011111111	0100011000000000

0011000011111111	0000011000000000

1111100001111111	0000001100000000

1111100001111111	0000001100000000

1111110000111111	0000000110000000

Screen Mask		  Cursor Mask

Figure 1: Example graphics cursor

-----------------------------------------------------------------------

The  graphics mask pair are defined together in a single  array  of

type unsigned integer.  For clarity,  comment fields were added in cur-

sor.h to show what the masks will actually produce. This is helpful not

only in identifying what a cursor looks like, but also in designing new

cursors.  The GraphicsCursor class constructor is used for declaring  a

cursor.  It takes as arguments the cursor hot spot,  mask array,  width

and height. The width and height are the actual pixel dimensions of the

image.  In Figure 1,  the width is 11 and the height is 16 -  these are

the  maximum  dimensions of the defined screen mask,  even  though  the

overall array is 16x16.  An instance of GraphicsCursor can be passed to

SetCursor() to make it the current cursor.

SetCursor() can also set the text cursor type and shape.  There are

two  cursor types in text mode.  The hardware cursor places the  normal

video  cursor (the cursor normally associated with the keyboard)  under

the  control of the mouse.  The software cursor is independent  of  the

video cursor and behaves similarly to the graphics cursor. The software

cursor is used in most applications so that the user is presented  with

the  normal video cursor for typing and a mouse cursor for menu  selec-

tion, etc.  The cursor type - 0 for software, 1 for hardware - is  part

of the TextCursor class.

Like the graphics cursor,  the text software cursor also requires a

screen and cursor mask,  but no hot spot since the cursor always  takes

up  a  whole character cell,  regardless of where the mouse  cursor  is

actually positioned in that cell. (If a hardware cursor is implemented,

the  starting  and ending scan lines for the cursor are required.)  The

two  masks operate only on a character cell,  not a 16x16 pixel  array.

Each cell consists of a one-byte character value and a one-byte charac-

ter attribute (which sets the foreground and background colors), so the

masks must each be two bytes long.  The upper byte masks the attribute,

and  the lower byte masks the character.  Like the graphics masks,  the

screen mask is XORed with the underlying character cell, and the cursor

mask is ANDed with the resulting value.  The difference is that in text

mode, both the character value and its color can be changed, whereas in

graphics mode, the pixel colors cannot be controlled.

To get a better understanding of what the masks will do, we need to

look at an example of a screen character:

6B24h = 0110 1011 0010 0100 b

|--- ---- ---------

| |   |       |__ character value  (24h)

| |   |__________ foreground color (11d)

| |______________ background color (6d)

|________________ blinking bit (1=blinking)

This  value will display a '$' (24h) with a background color  of  brown

(6) and a foreground color of lightcyan (11).  With the text cursor, we

can  mask each bit of the character and attribute,  so not only can  we

set the mouse cursor character,  but we can also control the foreground

and background colors (and even blinking).  The cursor masks default to

a reverse box, which is

Screen Mask = 77FFh = 0111 0111 1111 1111 b

Cursor Mask = 7700h = 0111 0111 0000 0000 b

The first byte of the screen mask,  77h in this example,  is ANDed with

the  underlying characters attribute.  The first 7h is ANDed  with  the

background  color  so that all colors but the blinking bit  are  passed

through.  Masking the blinking bit to 0 will allow the mouse cursor  to

rest on a blinking character without itself blinking.  The second 7h is

ANDed  with the foreground color so that those colors are also  passed,

except  that the high intensity colors will be converted to low  inten-

sity. The FFh also passes the screen character. In the cursor mask, the

77h  will invert both the foreground and background colors,  while  the

00h  creates an "empty" character, which is a character cell  with  no-

thing in it (a box).

If  you  want to create a cursor with a  particular  character  and

color,  you would set the screen mask to 0000h and the cursor  mask  to

whatever color and character values desired.  For example,

Screen Mask = 0000h = 0000 0000 0000 0000b

Cursor Mask = 0F23h = 0000 1111 0010 0011b

will set the cursor character to a '#' with a background color of black

and a foreground color of white.  More complex combinations are  possi-

ble,  such as setting the cursor's foreground color to a constant value

but allowing the screen's background color to show through.  Again, the

best  way to understand it is to try it,  and cursor.h defines  several

text cursors.

In graphics mode, the mouse driver takes care of drawing the cursor

and restoring the image under the cursor when it moves. One drawback to

this  is that you cannot use color cursors in graphics mode  using  the

mouse driver. You can instead create a color image, manually draw it to

the screen, and manually restore the underlying image.  This is exactly

how  the  ColorGraphicsCursors work.  If a CGC is installed  using  the

SetCursor()  command,  then the special drawing functions will  trigger

off of the event handler, thereby drawing the cursor whenever the mouse

moves. CGC cursors are defined just like standard graphics cursors, ex-

cept that the screen mask is split into the four video bit planes.  See

cursor.h for examples.

The color drawing functions in cgc.cpp were written purely in  C++,

with no assembly language.  The reason for this was to clearly show how

the  methods work.  The results are that the color cursor image  has  a

slight flicker, but overall the speed is acceptable.  Also, these func-

tions do not make any attempt to save and restore the video card state.

This could be a problem because the drawing functions are triggered  by

the  mouse event handler and could therefore be activated at any  time,

even  during another graphics draw.  You can avoid problems by  turning

the cursor off (using Hide()) before drawing any graphics,  which is  a

good  idea anyway.  Finally, these functions only support standard  EGA

and VGA resolutions.

A  better method is to use your favorite graphics library  to  draw

the cursor and restore the screen.  This will probably result in better

speed and might also support higher resolutions such as  SuperVGA/VESA.

(This approach also solves another drawback of the mouse drivers - they

also do not support high-res modes.) To implement another graphics  li-

brary, you need only modify the VideoRead(), VideoWrite(),  and Cursor-

Write() funtions. An example using the BGI is provided in cgc_bgi.cpp.

-----------------------------------------------------------------------

The Event Handler

There is another way to capture mouse events. The mouse event hand-

ler  is  similar to a TSR program in that it can be set up  to  capture

events in background.  The handler can be set to trigger off any combi-

nation of mouse events (Table 1),  and can then execute any user  code,

providing it does not issue a DOS or I/O call.  However, since the main

program  is suspended during the event handler execution,  the  handler

code  should  be fast so it does not to noticeably slow down  the  main

program.  When an event occurs,  certain mouse information is automati-

cally  placed  in the registers for the handler to  use.  The  function

Save() can be used to stuff these parameters into the event buffer  for

later use.  Any event handler routines you write must be declared as an

interrupt type and terminate with a far return.  The macro  EventExit()

provides  the proper exit code and should be the last statement in  the

handler function:

void interrupt lb_handler()

{

mouse.Save();

do_something();

EventExit();

}

You  can then use this function and an event mask to create an  in-

stance of a MouseEventHandler.  Installing it is accomplished  with the

function InstallHandler():

#include "mouse.h"

void interrupt lb_handler();

main()

{

unsigned char eventmask = LB_PRESSED | LB_RELEASED;

MouseEventHandler myHandler(eventmask, lb_handler);

if(mouse.Exists())

{

mouse.InstallHandler(myHandler);

mouse.Enable();

mouse.Show();

...

}

}

eventmask determines which mouse events will trigger the event handler.

In the example above, the handler will execute whenever the left button

is  pressed or released.  Because the event handler should be  kept  as

small  as possible,  you normally only want to call Save() for  storing

the mouse information and then exit.  The MouseEventHandler class  pro-

vides  such a handler by default if you do not specify a handler  func-

tion name:

main()

{

MouseEventHandler myHandler(LB_PRESSED | LB_RELEASED);

if(mouse.Exists())

{

mouse.InstallHandler(myHandler);

// Here's another way of doing it:

// mouse.InstallHandler(LB_PRESSED | LB_RELEASED);

mouse.Enable();

mouse.Show();

...

}

}

There  might be some cases where you want the handler to do more,  such

as when the program is waiting for user input and not doing much  other

processing.  In such cases you may want to load different handlers, de-

pending on what the program is doing.  A more functional handler can be

loaded for pulldown menus, for instance,  and a bare-bones handler  (or

none  at all) can be installed in speed critical areas where the  mouse

has little use.

The  Save() function called by the event handler stores  the  mouse

information in a buffer instead of writing it directly into the  class.

This allows events to be captured even while your program is busy doing

something else.  (The keyboard has a similar buffer that provides type-

ahead capability.)  Save() also calls the ColorGraphicsCursor funtions,

so  if you are using CGC's and also want to define custom  event  hand-

lers, you will need to consider this.  To get the event information out

of the buffer, you must call the function GetEvent():

for(;;)

{

mouse.GetEvent();		// get an event from the buffer

if(mouse.LB_Dn())		// check for left button down

{

if(mouse.x() > 320 || mouse.y() > 100)

do_something();

}

else

do_something_else();

}

When using the event handler you also have access to two other fea-

tures: MultiClick detection and the Repeater.  MultiClicks occur when a

mouse button is rapidly clicked more than once. You can set up the but-

tons  for MultiClicks by calling the SetClickThreshold()  function  and

then check them with the DoubleClick() and MultiClick() functions:

mouse.SetClickThreshold(250);	// set threshold to 250ms

for(;;)

{

mouse.GetEvent();

if(mouse.DoubleClick(LEFTBUTTON))	// check for 2 clicks

do_something();

if(mouse.MultiClick(LEFTBUTTON) == 3)	// check for 3 clicks

do_something_else();

}

When  using this feature, you should realize that multiple clicks  will

load  the buffer with several PRESSED and RELEASED events.  The  Multi-

Click  buffer does not get updated until a call to  GetEvent()  occurs.

Therefore,  it will take three passes through the loop above  before  a

triple click shows up. If the do_something() routine for a double click

clears the mouse buffer,  then the do_something_else() code will  never

execute because a triple click will never occur.

The Repeater allows a button to be pressed and held down to  gener-

ate multiple PRESSED events,  much like the keyboard does.  This  would

typically be used for a scrollbar routine.  Unlike the MultiClick  fea-

ture,  the  Repeater  can be set up for  individual  buttons.  Use  the

SetRepeatRate() function to set up the Repeater,  and then simply check

for events:

// This will set the intial delay to 250ms and the repeat rate

// to 100ms for the left button:

mouse.SetRepeatRate(LEFTBUTTON, 250, 100);

for(;;)

{

mouse.GetEvent();

if(mouse.LB_Dn())

do_something();

}

The mouse class installs the Repeater by hooking it to the system clock

interrupt which is called every 18ms.

Properly  used,  event handlers can add a whole new  dimension to a

mouse  driven interface.  However, there are some pitfalls to avoid. If

you set the event mask to trigger on mouse movement,  then the  handler

will execute quite often (every time the mouse is moved) and could  no-

ticeably  slow program execution during periods of  heavy  computation.

Reasons  to trigger on movement include updating a motion counter  such

as in the demo program,  having cursors that dynamically change  depen-

ding on the cursor position such as in a GUI,  or when using CGC's.  As

mentioned before, the handler should not issue a DOS, ROM, or I/O call.

The handler,  like a TSR,  is an interrupt routine that runs while  the

main DOS program is suspended.  DOS is not a re-entrant operating  sys-

tem  and issuing such a call will usually crash the system.  Also, some

standard C library routines make use of these system functions (such as

printf()), so often the only way of finding out is by trial and reboot.

-----------------------------------------------------------------------

0x01	MOUSE_MOVED

0x02	LB_PRESSED

0x04	LB_RELEASED

0x08	RB_PRESSED

0x10	RB_RELEASED

0x20	CB_PRESSED

0x40	CB_RELEASED

Table 1: Mouse events

-----------------------------------------------------------------------

A Demonstration

demo.cpp  is a demo program that utilizes most of the  mouse  func-

tions described.  main() installs the event handler,  runs a text  mode

demo, and a graphics mode demo.  The event mask is FFh which will trig-

ger the handler on any mouse event. Since that includes mouse movement,

the default handler that only calls Save() is used to store the parame-

ters and keep execution time to a minimum.

Both  demos  (textdemo() and graphicdemo()) call  a  function  that

draws the screen and another that processes mouse events.  textscreen()

assumes  a  color video card and draws a screen  with  eight  different

characters and colors to demonstrate how the cursor masks work.  Notice

that  the loop that draws the screen actually writes 50 lines of  video

even though the demo starts out with 25 lines.  A graphical representa-

tion  of a three-button mouse is drawn to show the cursor position  and

button status. After calling textscreen(), textdemo() prints the infor-

mation from the mouse.Info structure to the screen.  The mouse  y-limit

is  set to full screen (25 lines, which is the default) and the  cursor

is turned on with Show().  nexttdemo() is the processing loop for  text

mode  and is called with the name of text cursor structure and a  title

string.  nexttdemo() is called with several different cursors and  also

for 43/50 line mode.

nexttdemo() first sets the text cursor and prints the title string.

It then runs a loop that process mouse events. A counter that shows the

mouse position calls x() and y() which read variables that are  contin-

uously  updated by the event handler via GetEvent().  Button status  is

also  checked and displayed.  Released() is called to check for a  left

button release event which will terminate the loop if the cursor is in-

side the [Next] box at the time of release.  The loop will also be ter-

minated  if the right button is double-clicked anywhere on the  screen.

If the center button is double-clicked, the mouse is disabled until any

keyboard key is pressed.  Pressing the <Shift> key and the left  button

will  switch video pages (note that the alternate video page lacks  all

the  functionality of the main page).  Pressing the <Ctrl> key  and the

left button will set the global flag "done" which exits the rest of the

text demo and proceeds with the graphics demo.

graphicdemo()  works  the same way as textdemo().  In  addition  to

trying several cursors, it also tests different mickey-to-pixel ratios.

Notice that the ratio of 2 for the jet cursor will cause the cursor  to

locate  on every fourth pixel as it moves across the  screen.  graphic-

screen()  paints several background colors as well as black  and  white

boxes in the center. These boxes demonstrate the use of the cursor bor-

der created by the screen mask.  A graphical representation of a three-

button mouse is again drawn to show the cursor position and button sta-

tus.  nextgdemo()  sets the graphics cursor and  then  processes  mouse

events,  checking the position and button status.  The last cursor is a

ColorGraphicsCursor that is 9x39 pixels.

During  the  graphics demo any multiclick events are  displayed  as

different colors of the buttons on the screen (up to four).  The center

button  is  set up as a repeater button, so that holding it  down  will

generate multiple PRESSED events as well as multiclicks. A right button

triple-click  will advance to the next cursor,  and <Ctrl>-left  button

will exit the demo.

-----------------------------------------------------------------------

Supporting SuperVGA/VESA Modes

As mentioned before,  the mouse driver performs the draw &  restore

for the graphics cursor as it moves across the screen.  The drivers  do

not  currently support color cursors or video modes beyond  640x480x16.

The ColorGraphicsCursor feature of Mouse++ shows how color cursors  can

be supported by manually drawing the cursor.  This method can be exten-

ded to higher resolutions by modifying the VideoDraw() and CursorDraw()

functions.

If you are using a third party graphics library, the best method of

supporting color or high-res modes is to simply use the functions  that

are available in the library for manipulating bit images.  In the  BGI,

these are getimage() and putimage().  The file cgc_bgi.cpp contains the

code that uses the BGI to display the cursor.  This approach is  incom-

plete in that it does not properly mask the cursor at the edges of  the

screen (to display a partial cursor) nor does it attempt to insure that

the viewport is set to the entire screen.  The ColorGraphicsCursor con-

structor  in cgc_bgi.cpp converts the cgc image array to the BGI  image

format,  after which getimage() and putimage() are used to display  the

cursor.

The BGI,  however, also does not support SuperVGA or VESA modes  as

provided by Borland.  There is a set of VESA BGI drivers available that

extend the BGI up to 1024x768x256. They are shareware and are available

from the address listed below.  Using these drivers would allow cursors

to be displayed in any mode,  but you would still have to take care  of

masking the image at the screen edges and taking care of the viewport.

BGI VESA Drivers:

Jordan Powell Hargrave          Internet: jh5y@andrew.cmu.edu

1000 Morewood Ave, Box #3277      Bitnet: jh5y%andrew.cmu.edu@cmccvb

Pittsburgh, PA 15213                UUCP: uunet!andrew.cmu.edu!jh5y

(412) 268-4488                Compuserve: [72510,1143]

-----------------------------------------------------------------------

The Mouse++ Functions

-----------------------------------------------------------------------

Mouse::Exists()

Syntax: 	unsigned char Exists(void)

Description:	Returns the value of an internal flag that is set by

the Mouse constructor.

Return Value:	1 if a mouse was found, 0 otherwise.

-----------------------------------------------------------------------

Mouse::Visible()

Syntax: 	unsigned char Visible(void)

Description:	Returns the status of the cursor visibility.

Return Value:	1 if the cursor is visible, 0 otherwise.

-----------------------------------------------------------------------

Mouse::Buttons()

Syntax: 	unsigned char Buttons(void)

Description:	Returns the number of buttons for the mouse

Return Value:	2 for a 2-button mouse, 3 for a 3-button mouse.

-----------------------------------------------------------------------

Mouse::Button()

Syntax: 	unsigned char Button(void)

Description:	Returns the status of the mouse buttons and shift keys

that is stored internal to the Mouse class. Call

Position() before using this function to insure an

accurate value, or when using the event handler call

GetEvent().

Return Value:	A mask corresponding to the status of the mouse buttons

and the shift keys (down=1, up=0):

-------------------------------------------------------

0x01	Left button

0x02	Right button

0x04	Center button

0x08	Either shift key

0x10	Right shift key

0x20	Left shift key

0x40	Alt key

0x80	Ctrl key

-----------------------------------------------------------------------

Mouse::Enable()

Syntax: 	void Enable(void)

Description:	Enables the mouse. This function must be called before

the mouse can be used for the first time, or after a

call to Disable(). A separate call to Show() is neces-

sary to display the cursor.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::Disable()

Syntax: 	void Disable(void)

Description:	Disables the mouse and hides the cursor. If an event

handler has been installed, it is disabled and the

event buffer and multi-click buffer are cleared.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::Show()

Syntax: 	void Show(void)

Description:	Turns on the mouse cursor.

Return Value:	None. Sets the internal visible flag to 1.

-----------------------------------------------------------------------

Mouse::Hide()

Syntax: 	void Hide(void)

Description:	Turns off the mouse cursor. All other mouse functions

will continue to operate. You should hide the cursor

prior to any screen writes to avoid having the mouse

restore an incorrect background.

Return Value:	None. Sets the internal visible flag to 0.

-----------------------------------------------------------------------

Mouse::Position()

Syntax: 	void Position(void)

Description:	Reads the cursor position and saves the coordinates

in internal variables. Use x() & y() to read the inter-

nal variables. This function is not intended for use

with the event handler.

Return Value:	None. Sets internal class position variables and button

status.

-----------------------------------------------------------------------

Mouse::x()

Syntax: 	int x(void)

Description:	Returns the x-position of the mouse cursor that is

stored internal to the Mouse class. Call Position()

before using this function to insure an accurate

value, or when using the event handler call GetEvent().

Return Value:	An integer that corresponds to the pixel location of

the cursor, even if the screen is in text mode.

-----------------------------------------------------------------------

Mouse::y()

Syntax: 	int y(void)

Description:	Returns the y-position of the mouse cursor that is

stored internal to the Mouse class. Call Position()

before using this function to insure an accurate

value, or when using the event handler call GetEvent().

Return Value:	An integer that corresponds to the pixel location of

the cursor, even if the screen is in text mode.

-----------------------------------------------------------------------

Mouse::Motion()

Syntax: 	void Motion(void)

Description:	Reads the internal motion counters which give the dis-

tance (in mickeys) the mouse has moved since the last

call to this function, and stores the values in class

variables. Use xCount() & yCount() to read these vari-

ables. This function is not intended for use with the

event handler.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::xCount()

Syntax: 	int xCount(void)

Description:	Returns the distance the mouse has moved in the x-

direction. The distance is in mickeys, which may or may

not be the same as pixels, depending on the mickey-to-

pixel ratio. This function is useful for seeing if the

mouse has moved.

Return Value:	An integer that corresponds to the x distance in mic-

keys, regardless of screen mode.

-----------------------------------------------------------------------

Mouse::yCount()

Syntax: 	int yCount(void)

Description:	Returns the distance the mouse has moved in the y-

direction. The distance is in mickeys, which may or may

not be the same as pixels, depending on the mickey-to-

pixel ratio. This function is useful for seeing if the

mouse has moved.

Return Value:	An integer that corresponds to the y distance in mic-

keys, regardless of screen mode.

-----------------------------------------------------------------------

Mouse::Move()

Syntax: 	void Move(int x, int y)

Description:	Moves the cursor to a new position. x & y must be

pixel coordinates even if the screen is in text mode.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::Pressed()

Syntax: 	int Pressed(int button)

Description:	In manual mode, this checks to see if button has been

pressed since the last call to this command. In event

driven mode, this checks to see if the current event

was triggered by button being pressed. button can be 0

(LEFTBUTTON), 1 (RIGHTBUTTON), or 2 (CENTERBUTTON).

Return Value:	1 if button has a pressed event, 0 otherwise.

-----------------------------------------------------------------------

Mouse::Released()

Syntax:		int Released(int button)

Description:	In manual mode, this checks to see if button has been

released since the last call to this command. In event

driven mode, this checks to see if the current event

was triggered by button being released. button can be 0

(LEFTBUTTON), 1 (RIGHTBUTTON), or 2 (CENTERBUTTON).

Return Value:	1 if button has a released event, 0 otherwise.

-----------------------------------------------------------------------

Mouse::LB_Dn()

Syntax: 	unsigned char LB_Dn(void)

Description:	Returns the status of the left button.

Return Value:	1 if the left button is pressed, 0 otherwise.

-----------------------------------------------------------------------

Mouse::RB_Dn()

Syntax: 	unsigned char RB_Dn(void)

Description:	Returns the status of the right button.

Return Value:	1 if the right button is pressed, 0 otherwise.

-----------------------------------------------------------------------

Mouse::CB_Dn()

Syntax: 	unsigned char CB_Dn(void)

Description:	Returns the status of the center button.

Return Value:	1 if the center button is pressed, 0 otherwise.

-----------------------------------------------------------------------

Mouse::xLimit()

Syntax: 	void xLimit(int min, int max)

Description:	Sets the limit that the cursor can move in the x-

direction. min & max must be pixel values even if the

screen is in text mode.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::yLimit()

Syntax: 	void yLimit(int min, int max)

Description:	Sets the limit that the cursor can move in the y-

direction. min & max must be pixel values even if the

screen is in text mode.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::xyLimit()

Syntax: 	void xyLimit(int xmin, int xmax, int ymin, int ymax)

Description:	Sets the limits that the cursor can move in the x- &

y-direction. Limits must be pixel values even if the

screen is in text mode.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::GetVideoPage(void);

Syntax: 	int GetVideoPage(void)

Description:	Gets the video page number for which the mouse cursor

is currently active. This may or may not be the same

as the video page that is currently being displayed on

the screen.

Return Value:	The number of the mouse cursor's current video page.

-----------------------------------------------------------------------

Mouse::SetVideoPage(void);

Syntax:		void SetVideoPage(int page);

Description:	Sets the video page for which the mouse cursor will be

active. Normally this will be the same as the displayed

video page but it does not have to be.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::SetCursor()

Syntax: 	void SetCursor(TextCursor& cursor)

void SetCursor(GraphicsCursor& cursor)

void SetCursor(ColorGraphicsCursor& cursor)

Description:	Sets the text cursor as described by the TextCursor

class or the graphics cursor as described by the

GraphicsCursor or ColorGraphicsCursor class.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::MickToPix()

Syntax: 	void MickToPix(int horiz, int vert)

Description:	Sets the mickey-to-pixel ratio. A mickey is a motion

signal sent by the mouse and occurs every 1/200th inch

for most mice. horiz passes the number of mickeys re-

quired for an 8 pixel horizontal movement, and vert

passes the number of mickeys required for an 8 pixel

vertical movement. The constructor sets these to 8 and

16, respectively.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::SetSpeedThreshold()

Syntax: 	void SetSpeedThreshold(unsigned speed)

Defaults:	speed = 64

Description:	Sets the threshold at which the cursor speed doubles.

speed is in mickeys-per-second. Supposedly, this func-

tion is only available for Logitech mice.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::InBox()

Syntax: 	int InBox(int left, int top, int right, int bottom)

Description:	Checks if the mouse cursor is located within a rec-

tangle. Rectangle limits should be in pixels regardless

of screen mode.

Return Value:	1 if the cursor is in the rectangle, 0 otherwise.

-----------------------------------------------------------------------

Mouse::Exclude()

Syntax: 	void Exclude(int left, int top, int right, int bottom)

Description:	Sets up an exclusion area in which the mouse cursor

automatically turns itself off. This is not the same as

the exclude function that is built into the mouse, and

must be continuously called in a loop. Rectangle limits

should be in pixels regardless of screen mode.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::SetClickThreshold()

Syntax: 	void SetClickThreshold(unsigned time)

Defaults:	time = 250 (ms)

Description:	Sets the threshold at which sequential clicks are regi-

stered as multi-clicks. time is in milliseconds.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::MultiClick()

Syntax: 	int MultiClick(int button)

Description:	Checks for multi-clicks of a button. button can be 0

(LEFTBUTTON), 1 (RIGHTBUTTON), or 2 (CENTERBUTTON).

Return Value:	Current number of multi-clicks.

-----------------------------------------------------------------------

Mouse::DoubleClick()

Syntax: 	int DoubleClick(int button)

Description:	Checks for double-clicks of a button. button can be 0

(LEFTBUTTON), 1 (RIGHTBUTTON), or 2 (CENTERBUTTON).

Return Value:	1 if button has been double-clicked, 0 otherwise.

-----------------------------------------------------------------------

Mouse::ClearClick()

Syntax: 	void ClearClick(int button)

Defaults:	button = 7 (all buttons)

Description:	Resets the multi-click status of button. button can be

0 (LEFTBUTTON), 1 (RIGHTBUTTON), or 2 (CENTERBUTTON).

Return Value:	None.

-----------------------------------------------------------------------

Mouse::SetRepeatRate()

Syntax: 	void SetRepeatRate(unsigned char button,

unsigned delay, unsigned rate);

Defaults:	button = LEFTBUTTON

delay  = 250 (ms)

rate   = 150 (ms)

Description:	Sets the threshold at which sequential clicks are regi-

stered as multi-clicks. time is in milliseconds and de-

faults to 250.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::InstallHandler()

Syntax: 	void InstallHandler(unsigned mask,

void interrupt (*fn)(void))

Alt. Syntax:	void InstallHandler(MouseEventHandler handler)

Defaults:	fn = MouseHandler()

Description:	Installs an interrupt handler function which is automa-

tically run whenever an event occurs that is included

in the mask. The mouse driver loads the CPU registers

with mouse information before calling the function.

Therefore, the first action should be to save this in-

formation to an event buffer, which can be accomplished

by calling the Save() function. The remaining function

code should be kept to a minimum, and the function MUST

be terminated with the EventExit() macro. The mask bits

are described in Table X. If a function pointer is not

passed in the parameter list, then the default handler

MouseHandler() will be used.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::Save()

Syntax: 	void Save(int event, int button, int x, int y,

int xcount, int ycount)

Description:	Stores the passed parameters in the event buffer. This

should be the first function called from an event hand-

ler routine. See the default handler MouseHandler() for

details.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::GetEvent()

Syntax: 	void GetEvent(void)

Description:	Gets the next event from the event buffer and loads the

parameters into class variables.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::ClearEvent()

Syntax: 	void ClearEvent(void)

Description:	Clears the current event from the internal class varia-

bles.

Return Value:	None.

-----------------------------------------------------------------------

Mouse::ClearBuffer()

Syntax: 	void ClearBuffer(void)

Description:	Clears the event buffer.

Return Value:	None.

-----------------------------------------------------------------------

References

1.  Kent Porter, "Mouse Mysteries, Part I: Text", Turbo Technix, Vol.

1, No. 4, pp. 52-67, May/June 1988.

2.  Kent Porter, "Mouse Mysteries, Part II: Graphics", Turbo Technix,

Vol. 1, No. 5, pp. 42-53, July/August 1988.

3.  Terry Dettmann, DOS Programmers Reference, Part V: Reference, Mouse

Functions, pp. 717-739, Que, 1989.

4.  Ralf Brown, Interrupt List, Release 31, July 12, 1992. This is

available in the Public Domain as INTER31A.ZIP, INTER31B.ZIP, and

INTER31C.ZIP.