/�������������������� /��������������������\ /��������������������� '93

|                     |                    | |

|                     |                    | |

|___________          |____________________/ \____________________

|                     |                 \                         \

|                     |                  \                        |

|                     |                   \                       |

|                     |                    \ _____________________/

Instructions for using

the Small but useful FRS extension ( v2 should be a lot better )

Written by Ben Riley '93

Please note that this is the interpreter extension only

( if anyone could supply the compiler source, I would be grateful )

First of all I must say that I have only tested this extension on one other

ST besides my own STE and it worked ok but it should work on all ST's

except ones that are fitted with accelerators ( since the border removal

routines need exact timing ) and everything works okay on the Falcon except

for the scanfrequency commands ( i.e Hertz50 hertz60 ) which do nothing and

of course the border removal commands. The rvalid function also kept

giving back a value saying that there was a reset program in memory when

I tested it on my Falcon but I think this is because of an address change

or something like that. By the way if anyone knows how to rid the bug on

the Falcon in STOS which makes a character appear every now and again

then please tell me how to stop it, as it gets in the way quite often

when writing a program.

At the moment I am working on a second version in my spare time which will

include more versatile screen copying commands, the current ones may not

be very versatile but they are faster that equivalent commands in other

extensions. This version is very interesting but it does have some commands

which other extensions don't such as the border removal, frequency and

memory commands. Anyway enough of this chat-chit here is a current commands

list:-

1.	Hertz 50 - change to normal scan frequency

2.	Hertz 60 - change to a higher frequency ( only on colour monitors )

3. 	Mouse inter off - turn mouse off completely

4.	Mouse inter on - turn the mouse interrupt back on

5.	Screen wipe 'addr' - Clear a screen very fast

6.	Block copy 'addr1','addr2' - copy a screen

7.	Bottom inter on - remove bottom border

8.	Bottom inter off - put it back

9.	No sound - stop all sounds

10.	Set syscount 'value' - set the system timer

Now for some functions ( i.e. They return a value back to STOS )

1.	t=syscounter - get timer value

2.	r=rvalid - check for a reset program

3.	m=meg - check for a megabyte of memory

I would be grateful also if someone could supply the source for the compiler

extensions so these commands can be compiled.

1.	Hertz 50 - this is the scan frequency of a normal TV set and

can be used to set the frequency back to 50 Hz after

a change to 60 Hz.

example:-

10 cls

15 hertz 60

20 centre "Press any key to go back to 50 Hz..."

30 wait key

40 hertz 50

2.	Hertz 60 - this is used to increase the scan frequency to a higher

rate so as to improve screen quality. This command can

only be used on colour monitors as these can handle

the higher frequency. Do not try on a TV set as this

could damage it.

example:-

10 cls

20 hertz 50

30 centre "Press any key to change scan frequency..."

40 wait key

50 hertz 60

60 cls

70 centre "Press any key to return to normal frequency..."

80 wait key

90 hertz 50

3. 	Mouse inter off - this command turns of the mouse completely since

the normal 'hide' command draws a hidden mouse

pointer and uses up valuable processor time.

4.	Mouse inter on - this just restores the mouse after

mouse inter off has been entered.

5.	Screen wipe 'addr' - this clears a screen at the address 'addr'

it is much faster than cls.

example:-

10 sc=start(10)

20 screen wipe sc

would clear the screen bank 10

10 screen wipe physic

would clear the physical screen

6.	Block copy 'addr1','addr2' - this copies the screen at the address

'addr1' to the address at 'addr2'.

This command is very fast, faster

than commands in other extensions.

example:-

10 repeat

20 block copy physic,physic-160

30 wait vbl

40 until fire

note: the addresses 'addr1' and 'addr2' must be actual, that is

they must be 'start(10)' not just '10'

7.	Bottom inter on - this command removes the bottom border of the

screen thus allowing for a bigger screen.

example:-

10 erase 10

20 reserve as work 10,64000 : rem * 2 screens in length *

30 copy physic,physic+32000 to start(10)

40 copy physic,physic+32000 to start(10)+32000

50 old1=leek($45e)

60 old2=leek($44e) : rem * save old screen pointers *

70 loke $45e,start(10)

80 loke $44e,start(10)

90 bottom inter on : rem * remove bottom border *

100 wait key

110 loke $45e,old1

120 loke $44e,old2

130 bottom inter off : rem * restore old interrupts *

140 default

8.	Bottom inter off - this just puts the border back in after a

'bottom inter on'

9.      No sound - this just removes any sounds which haven't or won't

stop.

10.	Set syscount 'value' - this sets the system timer ( 200 Hz tick )

allows for better accuracy than the timer

command.

11.     t=syscounter - this gets the value of the system timer which is

returned in the variable 't'.

12.	r=rvalid - this checks to see if there is a reset program in

memory.

example:-

10 if rvalid then bell : print "Reset prog in memory"

13.	m=meg - this checks for a megabyte of memory

example:-

10 if meg then print "You've got a megabyte!"

Well that concludes the proceedings for today, I hope this small extension

is of use to you, its a bit late but I might as well do some greetz

Hi to Chris and Jeff H, Tony G, Lewis, Phil B, Kev, Col, Mick Wilson, Mark,

Flick, PV tubes ( Amiga 1200 and PC's - crap compared t' Falcon ! ),

everyone who helps make STOSSER, Joe, Mark R ( should have bought a

Falcon ), Lindsey, Emma, Cella, + all STOS coders.

If want to contact me for any reason write to:

Ben Riley,

96 Hollins Lane,

Accrington,

Lancashire,

BB5 2JS,

England