���Ŀ ���Ŀ ������Ŀ ڿ    ���Ŀ

��  � ��    ��  �  � ��  � ��

����� ����  ��  �  � ��  � ���¿

�� � ��    ��  �  � ��  �    ��

��  � ����� ��  �  � ����� �����

���������������������������������������������������������������������

Written by Roman Dolejsi (RDOS of DIMENSION)

January 1, 1997

Last update:   January 1, 1997

This is the documentation belonging to and explaining the use of

REMUS version 0.04e

Universal sound card emulator for MS-DOS

TOPICS covered in this document:

DISCLAIMER

APOLOGIZE

PURPOSE OF REMUS

REQUIREMENTS

INSTALLATION OF REMUS

REMUS IN ACTION

DIALOG LINE

AVAILABLE BASE CARDS

AVAILABLE EMULATIONS

GLOBAL EMULATION TABLE

SOUNDCARD AND EMULATION INFO

FUTURE PLANS

FREQUENTLY ASKED QUESTIONS

COPYRIGHT

WHERE TO FIND REMUS

THANX

CONTACTING ME

DISCLAIMER:

-----------

Although REMUS has been tested on several systems, I cannot guarantee that it

is bugless. Therefore I do not  take  responsibility for any damage directly or

indirectly caused by  REMUS as a result of known/unknown errors in it or errors

caused by its access to your devices.

APOLOGIZE:

----------

Sorry, english is not my  native language. However, I wrote this nice docy in

english without any help. I APOLOGIZE for any mistakes in the document. Take it

easy... Nobody is perfect.. :-)

PURPOSE OF REMUS:

-----------------

REMUS is a sound card emulator for IBM PC and compatibles running MS-DOS. By

emulation it will allow you to use products created for sound card you don't

have. It is programmed to be used on as many cards as possible, but it is still

in fairly alpha stage and don't expect any miracles.

REQUIREMENTS:

-------------

REMUS is coded in protected mode 386-assembler. You'll then need manager-free

MS-DOS environment (ie. without Emm386, Qemm, 386max  or such memory managers).

REMUS will run only  on computers with at least 2MB and less that 16MB RAM. I'm

really  sorry  for  the  higher  limit but currently my manager will freeze due

to unknown reason. You may also find useful to have some supported soundcard.

INSTALLATION OF REMUS:

-------------------------

- first  you have to make some directory on your drive. Then you have to unpack

there archive file RMSxyyz.ZIP.

(you probably did all this when you read these lines :-)

- REMUS pack should consist of these files:

REMUS.EXE   - Main file - OPL player

REMUS.DOC   - Documentation for REMUS (this file)

REMUS.REV   - REMUS changes/revision history file

FILE_ID.DIZ - quick info file

- now you have REMUS successfully installed :-)

REMUS IN ACTION:

----------------

For emulation are used protected mode features. When REMUS starts protected

mode memory manager is initialized and takes control over memory, I/O, IRQ, DMA

and Timer mechanisms. When REMUS is running, your sound card is used for

emulation of some other sound card. In that time your original sound card is

not available. Instead, you can even have emulated sound card on the ports

where normally sits your real card. It's because your card is internally

managed by REMUS's emulation routines and vm86 client's accesses are remaped

into REMUS. REMUS can't be stopped, you have to reboot the computer after

using it.

DIALOG LINE:

------------

Dialog line looks as following:

REMUS.EXE [/opt] base_card [/card_params] emu_card [/card_params]

options:

/b - list of supported base cards

/e - list of available emulations

Current list of base cards will tell you what to put instead base_card

parameter and same for emu_card.

When you want for example Gravis Ultrasound with 768KB on port 240

to be emulated on Sound Blaster on port 220, you have to use these params:

REMUS.EXE sblr /p220 gusc /p240 /m3

and so on...

AVAILABLE BASE CARDS:

---------------------

These cards can be used for playing:

[adlb] - Adlib (OPL2)

Sound is generated by simulating 5-bit D/A converter. Sometimes sounds

really weird.. accepts parameter /p (port), default value is 388.

[dacm] - D/A Converter

This is an 8-bit mono D/A converter. Can be created with minimal effords

and its quality is not so bad. It's even very fast board. Accepts /p,

default is 378.

[gusc] - Gravis Ultrasound

WaveTable board with noiseless sound. In REMUS are used only two voices

offering stereo 16-bit D/A. Wavetable will be used later. It's quite fast

board. Accepts /p (port + 100), default is 320.

[pcsp] - PC Speaker

Original beeper available on every system. REMUS uses it as primitive 6-bit

mono D/A converter. Accepts /p (port), default is 42.

[sblr] - Sound Blaster

Board offering 8/16-bit mono/stereo sound depending on type. Here it's used

as very slow mono 8-bit D/A converter (only minimal support now). Accepts /p

(port), default is 220.

AVAILABLE EMULATIONS:

---------------------

More info about REMUS's emulation capabilities will come later, but now list

of emulations would be fine:

[adlb] - Adlib (OPL2)      .. default: /p388

[dacm] - D/A Converter     .. default: /p378

[gusc] - Gravis Ultrasound .. default: /p220 /m4

* /mX   - memory size of emulated GUS in 256k banks

* on the base side there may be also /fX parameter for frequency

[pcsp] - PC Speaker        .. default: /p42

[sblr] - Sound Blaster     .. default: /p220

[ston] - Stereo-on-1 DAC   .. default: /p378

There is option common to all of the emulations:

/pXXX - port number for emulated card

GLOBAL EMULATION TABLE:

-----------------------

* - fully working (at least with most progs)

# - nearly fully working (port/irq) except DMA emulation

q - sound card is sometimes switched off and then no emulation works

w - emulation engine is in progress but is useless right now

��������������������������������������������������������������������������������������������������Ŀ

�what  � P � D � D � 2 � A � A � S � S � S � P � P � T � S � V � W � G � G � I � A � G � M � S � M �

�   ��>� C � / � / � x � D � D � B � B � B � A � A � U � N � O � S � C � U � W � W � M � T � C � A �

�      � S � A � A � D � L � L �   � P � 1 � S � S � R � D � C � S � O � S � A � E � I � 3 � C � U �

�      � P �   � S � / � I � G �   � R � 6 � 8 � 1 � T � M � M �   � D �   � V � 3 � D � 2 � 1 � I �

�where � K �   � T � A � B �   �   � O �   �   � 6 � B � 2 � S �   �   �   � E � 2 � I �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�PCSPK � *q� *q� *q�   � wq�   � #q�   �   �   �   �   �   �   �   �   � #q�   � wq�   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�D/A   � * � * � * �   � w �   � # �   �   �   �   �   �   �   �   �   � # �   � w �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�D/AST �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�2xD/A �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�ADLIB � * � * � * �   � * �   � # �   �   �   �   �   �   �   �   �   � # �   � w �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�ADLG  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�SB    � * � * � * �   � w �   � # �   �   �   �   �   �   �   �   �   � # �   � w �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�SBPRO �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�SB16  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�PAS8  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�PAS16 �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�TURTB �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�SNDM2 �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�VOCMS �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�WSS   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�GCODEC�   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�GUS   � * � * � * �   � w �   � # �   �   �   �   �   �   �   �   �   � # �   � w �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�IWAVE �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�AWE32 �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�GMIDI �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�MT-32 �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�SCC-1 �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

��������������������������������������������������������������������������������������������������Ĵ

�MAUI  �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �   �

����������������������������������������������������������������������������������������������������

SOUNDCARD AND EMULATION INFO:

-----------------------------

������ AWE 32 board ������

This is name of wavetable board created by a company Creative Labs, Inc.

It consists of standard Sound Blaster 16 board (refer to SB section) and

advanced wavetable processor EMU-8000 made by a renomed company E-MU, Inc.

Chip is capable of managing 32 16-bit independent voices, each with

its own 2 LFOs (low frequency oscillator), 2 EGs (envelope generator),

low pass filter and reverb and chorus effects. Sounds are created from

samples stored in internal memory space. EMU8000 can access up to 32MB of

memory, however in AWE32 there is only 1MB dedicated for ROM (expandable

to 4MB ??) and 512kB for RAM (expandable to 28MB). All voices are running

at constant speed 44.1kHz, while their LFO's speed may vary from 0.042Hz

to 10.72Hz and low pass filter's frequency from 100Hz to 8kHz. Whenever

EMU accessess RAM data or is drived to pass FM music from OPL chip, it uses

2 voices to refresh RAM memory and for FM throughpass. In that case only 30

voices are available for wavetable playing.

All effects for every voice and also some overall effects are made by

an effect engine, which is right now UNDOCUMENTED. These effects include

various Chorus, Hall and Reverb effects (for today, because effect engine

probably offers some programmability for myriads of new effects).

AWE 32 hasn't dedicated MIDI engine managing EMU-8000 and therefore it

has mechanisms for generating an NMI when something tries to access MIDI

ports. Software driver running in memory can then simulate missing MIDI

engine.

������ AWE 32 emulation engine specification ������

Right now only some very basic port mechanisms are emulated. No sound

yet nor any EMU-processor dependent function (EG/LFO/Filter counting etc.).

Apart of everything mentioned I can say effect engine will probably be

never supported because of lack of ANY documentation and this task will

also surely eat very much CPU power.

������ AWE 32 emulation engine compatibility ������

Name            Author      Type    Status      Checked by   Comp    Ver

������������������������������������������������������������������������

FXTweak         Infinity    util    ok run/quit RDOS         4/160  .03a

������ D/A converter on LPT port ������

This is very simple board offering one D/A converter accessed via

paralel port. It's not detectable and is simple to use. There are many

circuits which can serve as D/A converter compatible boards. One of them,

so-called "resistor DAC" is very very inexpensive. It's sound quality is

not so bad. It's probably the best solution for REMUS users without any

sound card. Its circuitry is described below.

Also some more advanced boards offer D/A compatibility. These boards are

sometimes detectable and some games use their detection for enabling sounds.

One of these boards is Covox Speech Thing and another one is called Disney

Sound Source (this one probably offers something more but I have not info

about it at all).

������ Circuitry of the D/A converter on LPT port ������

You need one plug for your amp, one for your parallel port and 9 20k

and 9 10K resistors. Use 1% resistors if possible � and 20k sound much

better than 22k. The 100nF capacitor may not be needed, try removing it

if this circuit sounds poor.

Paralel Port:

signal   pin        20k   20k

D0       2      >��������������0v

20k � 10k

D1       3      >������Ĵ

20k � 10k

D2       4      >������Ĵ

20k � 10k

D3       5      >������Ĵ

20k � 10k

D4       6      >������Ĵ

20k � 10k

D5       7      >������Ĵ

20k � 10k

D6       8      >������Ĵ

20k � 10k

D7       9      >������Ĵ

20k � 10k  100nF

������Ĵ�����> To amplifier

� 10k

GND      20     >��������������������>

0v

������ D/A converter emulation engine specification ������

Fully working D/A emulation. Standard init port is 378h (LPT1).

Some check functions of Disney Sound Source are implemented.

������ D/A converter emulation engine compatibility ������

Name            Author      Type    Status      Checked by   Comp    Ver

������������������������������������������������������������������������

DMP 3.01        Otto Chrons player  ok          RDOS         4/66   .02a

Power Replay    M. Grof     player  ok          RDOS         4/66   .02a

Another World   Delphine    game    ok          RDOS         4/66   .02a

AnotherW Intro  TDT         intro   ok          RDOS         4/66   .02a

Fascination     Koktel      game    ok          RDOS         4/66   .02a

Gobliins        Koktel      game    ok          RDOS         4/66   .02a

Visual Player   many ppl    player  ok          RDOS         4/160  .03a

ModPlay Pro     Mark J.Cox  player  ok          RDOS         4/160  .03a

������ Gravis UltraSound board ������

This is name of family of wavetable boards created by a company called

Advanced Gravis Computer Technology Ltd.

Onboard pipeline chip named GF1 is capable of managing up to 32 16-bit

independent voices (D/As) using samples stored in onboard RAM (256k-1MB).

Sampling frequency depends on number of used voiced (14..32) and may

vary from 44kHz down to 19kHz. Current voice position pointer in memory

has fraction part and resulting sample is interpolated using that number.

This leads to very high quality of produced sound even when using low

frequency samples. For transfering samples to/from GUS's RAM, board's

ports or DMA can be used. However, ports are not so fast as DMA that

many products prefer DMA uploading.

Basic board had no sound source mixer while later versions came with

ICS or other mixer. Later boards (Gravis UltraSound MAX) came also with

special chip named CODEC for Digital Signal Processing. The newest card

(Gravis UltraSound PnP) uses only one chip with built-in GF1, mixer and

CODEC chip. This is AMD's InterWave chip and offers except standard GUS

also up to 16MB ROM / 16MB RAM locally adressable memory, digital mixer

for mixing voices together, 64 LFOs for tremolo/vibrato effects, effects

processing for chorus/reverb/flange..., stable native frequency 44kHz.

PnP's CODEC has independent sampling/playing rates, freqs up to 48kHz,

very large FIFOs and more... GUS PnP (Pro) ships with 1MB ROM and

0kB (512kB) RAM expandable to 8MB (8,5MB) (via 30-pin SIMM slots).

All boards are not compatible with any other standard but are able to

generate NMI (Non Maskable Interrupt) when some program access permitted

ports. This process is used for emulating other platforms (namely

Adlib (OPL2), Sound Blaster DAC or Roland MT-32/SCC-1 and General MIDI).

������ Gravis UltraSound emulation engine specification ������

ALL functions of GF1 are emulated. Except this also onboard timer and

IRQ generator are emulated. There is no DMA emulation now (buggy REMUS

DMA manager !?!).

Sound is created via software GF1 pipeline emulation engine offering

all features of GF1 (16-bit, uni/bidirectional loops, volume ramping,

interpolation). Stopped voice is not emulated for speed reason (on

original GUS it may be used as a simple D/A converter - but nearly isn't).

Software mixing of all voices is done at the same frequency as GF1 or

at its multiplied derivate. Multiplication factor can be choosed during

REMUS startup and may vary from 1/4 to 9/4 (Freq 11kHz- to Freq 99kHz-).

Standard factor is 1=4/4 (Frequencies 44kHz - 19kHz like those found in GF1).

IRQ/DMA numbers are changeable using ULTRINIT (GUS utility). Standard

setting is IRQ 11, DMA 5 (note that DMA manager in REMUS doesn't work now!).

Standard GUS port number is 220h.

GUS Memory is selectable (1-4: 256k, 512k, 768k, 1MB) and is allocated

from available extended memory during REMUS inicialization. Standard value

is 1MB memory (value 4).

NMI is not emulated (MEGAEM / SBOS don't work).

ICS, CODEC or whole InterWave chip are not emulated.

������ Gravis UltraSound emulation engine compatibility ������

Name            Author      Type    Status      Checked by   Comp    Ver

������������������������������������������������������������������������

Scream Trck 3   FC          medit   ok          RDOS         4/160  .01b

GusPlay 1.5     Cascada     player  ok          RDOS         4/160  .01b

Cartoon         MiST        64k     ok          RDOS         4/160  .01b

Debut           DarkZone    demo    ok          RDOS         4/160  .01b

Bill G Force    Complex     64k     ok, slow!?  RDOS         4/160  .02a

Oops!           Elf         64k     ok          RDOS         4/160  .02a

Louise          Karma       64k     ok          RDOS         4/100  .02a

Second Reality  FC          demo    ok, 1/2 spd RDOS         4/160  .01b

Big Deal        Acme        demo    - vcpi/dpmi RDOS         4/160  .01b

Cyboman 3       Hnut Loop   demo    - vcpi/dpmi RDOS         4/160  .01b

Flight          Sin         demo    ok          RDOS         4/160  .01b

Arrow           Grid        64k     ugh, chip?! RDOS         4/160  .01b

Shadow Team     Hirmu!      64k     ok          RDOS         4/160  .01b

Res. Rangers    Live!       64k     ok          RDOS         4/160  .01b

Shamaan         Complex     demo    ok          RDOS         4/160  .01b

Mind Plr 1.0    Nght Brkr   plr     ok          RDOS         4/160  .01b

Amoeba Plr 0.9  Amoebasoft  plr     ok          RDOS         4/160  .01b

Disorder 2      Statix      trckr   - vcpi      RDOS         4/160  .01b

Extreme 0.97b   Extreme     trckr   - vcpi      RDOS         4/160  .01b

GusDrive II     CCS-prod.   ramdsk  - blockdev  RDOS         4/160  .01b

C64S Emulator   M.Peternel  emultr  - silent    RDOS         4/160  .01b

PolyPlay        AcmE        plr     ptrn skip   RDOS         4/100  .02a

Crystal Player  S.Grandjoux plr     ok          RDOS         4/100  .02a

DMP 4.01        Otto Chrons plr     ok, w/o dma RDOS         4/100  .02a

Digital Plr Pro             plr     - irq/dma   RDOS         4/100  .02a

Impulse Tracker Jeffrey Lim trckr   ugh, lo spd RDOS         4/100  .02a

SSS             Legend Des. plr     ok          RDOS         4/160  .02a

SidPlay 1.40a   M.Schwendt  plr     - Exception RDOS         4/160  .03a

BBStro          Dig.Design  intro   ok          RDOS         4/160  .03c

BNP             Blue Nose   intro   ok          RDOS         4/160  .03c

The One         Eclipse     intro   ok          RDOS         4/160  .04d

TDU Jam         Dream Dsgn  intro   ok          RDOS         4/160  .04d

Joe Mix         TPOLM       demo    ok          RDOS         4/160  .04e

������ PC Speaker ������

Although PC compatible systems are equipped with no sound board at all,

the simple thing that makes your PC beep, can also be programmed to

substitute weird D/A (6-bit) to produce digitized sound. Some programs

(the older ones) have used this capability to produce digitized sounds or

some music.

������ PC Speaker emulation engine specification ������

Emulated PC Speaker is always switched to digital which is fully emulated.

No beeping can be done. Digital PC Speaker emulation is not as bad as someone

may believe, because it's often done on real D/A converters and 6-bit Speaker

value is expanded to 8/16-bit equivalent instead.

������ PC Speaker emulation engine compatibility ������

Name            Author      Type    Status      Checked by   Comp    Ver

������������������������������������������������������������������������

DMP 3.01        Otto Chrons player  ok          RDOS         4/66   .04a

ModPlay Pro     Mark J.Cox  player  ok          RDOS         4/160  .04a

Inertia Player  Inertia     player  ok          RDOS         4/66   .04a

������ Sound Blaster board ������

This is name of family of boards created by Creative Labs, Inc.

Sounds are made via onboard D/A converter. To make things easier there

is a DSP (Digital Signal Processor) which accepts some commands. These

commands are used to set up some parameters on the card or send digital

data to the D/A converter. When multichannel music is needed, it's up to

the program to mix channels together and then send result to only ONE

D/A converter on the board. Some new boards of this type use two D/As,

every of them assigned to one side (left/right) for stereo music. Basic

board is capable of making 8-bit mono sound while the newest can cope

with 16-bit stereo samples (two 16-bit D/As). The board also supports

so-called DMA channel for sending data to D/A without CPU usage. 16-bit

board has to have two DMAs, one 8-bit and one 16-bit. For proper work

of DMAs, an IRQ is needed (It informs computer that DMA process has ended).

On 8-bit boards DMA and IRQ numbers are set with onboard jumpers while

on 16-bit boards DMA and IRQ numbers are runtime changeable.

On some 16-bit boards there is another chip called CSP (old ASP). It is

short term for Advanced Signal Processor which is mainly used to compress

recorded stream of data or for voice recognition. Documentation for this

chip has never been released and therefore it will never be support for it

in REMUS (probably).

Apart of this, there is also an onboard OPL2 chip for Adlib compatibility

(refer to FM/OPL emulation). On SB Pro 1 there were two OPL2 chips for stereo

music available, each of them for one side. Since SB Pro 2 there is only

one OPL3 stereo chip capable of more advanced, 4-operator FM music.

All boards are backward compatible down to Sound Blaster 1.0.

These board versions are known:

���� Name ����        ������ DAC specification ������     �� DSP ��

���������������������������������������������������������������������Ŀ

� Sound Blaster 1.x �   mono �  8-bit � up to 11kHz         �   1.0x  �

���������������������������������������������������������������������Ĵ

� Sound Blaster 2.0 �   mono �  8-bit � up to 22kHz         �   2.0x  �

���������������������������������������������������������������������Ĵ

� Sound Blaster Pro � stereo �  8-bit � up to 22kHz / 44kHz �   3.0x  �

���������������������������������������������������������������������Ĵ

� Sound Blaster 16  � stereo � 16-bit � up to 44kHz         �   4.xx  �

�����������������������������������������������������������������������

������ Sound Blaster DAC emulation engine specification ������

Right now only port and IRQ emulation works. Emulated DSP version is

1.01 with some check functions from 3.xx DSP (SBPro's DIAGNOSE works :-).

IRQ number is not selectable and is permanently set to IRQ10 (lower IRQs

do not work for some strange reason - ie. REMUS IRQ manager bug??).

Don't even think about DMA emulation. REMUS's DMA manager is unusable.

������ Sound Blaster DAC emulation engine compatibility ������

Name            Author      Type    Status      Checked by   Comp   Ver

�����������������������������������������������������������������������

SBPro Diagnose  Creat.Labs  util    ok (DMA-)   RDOS         4/33  .03a

Another World   Delphine    game    ok          RDOS         4/33  .03a

AnotherW Intro  TDT         intro   ok          RDOS         4/33  .03a

ModPlay Pro     Mark J.Cox  player  ok          RDOS         4/160 .03a

FUTURE PLANS (Global targets):

------------------------------

* EMS support

- memory paging manager for RPMMM needed first

* DPMI support

- probably 32-bit clients only

- minimal functions, but at least those needed by Tran's PMODE extender

- at the beginning buggy stack management (I don't understand it now)

* complete move in protected mode

- right now are base card inits in real mode (really messy, isn't it? :)

* total modularity

- one code for one emulation target at the binary level (now only on

source level, during compilation distributed throughout bases)

* online settings menu

- with runtime-changeable emulations, memory options, maybe disassembler

* coprocessor usability

- FPU is now detectable, but generates Divide by zero error when accessed

* Pro-Pack error solving

- now Pro-packed files definitely hang computer (why?! not under debugger!)

FUTURE PLANS (Emulation targets):

---------------------------------

* full Sound Blaster emulation

- DMA manager is not functional - DMA support is not possible now :-((

- stereo

- 16-bit

* Adlib (OPL2) emulation

- realtime FM digital emulation is very very hard thing to do :-)

* OPL3 emulation

- Depends on OPL2 emulation, would probably need the most CPU power

ever seen.

* Gravis Ultrasound DMA/IRQ handling emulation

- should work but doesn't now .. flame must go to REMUS DMA Manager

* Gravis UltraSound PnP emulation (AMD InterWave wavetable chip)

- synthesizer part will be probably first

- time for work... (have very complete documentation)

- its digital mixer is very interesting thing (and hard to emulate)

* Sound Blaster AWE32 emulation (EMU8000 wavetable chip)

- time for work... (have documentation for main pipeline processor)

- some documentation for EMU's effects engine needed

(dummy chaos of data arrays sent to card really does not tell anything)

- probably the hardest emulation I've ever seen

* Roland MT-32 / Roland Sound Canvas / General Midi emulation (wavetable chips)

- documentation for SYSEXes needed

- really explaining documentation for all the wheels and other effects

REMUS FREQUENTLY ASKED QUESTIONS:

---------------------------------

* REMUS hangs up after initialization

- if you have more than 16MB RAM, REMUS will not work (for now)

- if you have some memory manager incompatible with INT15 or HIMEM running,

REMUS will not work

- if you don't have XMS driver loaded and some application uses Xmemory

via INT15 BIOS services, computer hangs when REMUS starts and this

application will try to access that Xmemory (for example Norton Cache)

* REMUS does not want to run with PM driver (EMM386/QEMM/Win) installed in mem.

- REMUS is clean 32-bit PM manager. It needs completely free environment to

operate. Therefore other PM managers will NEVER be supported in REMUS.

REMUS is from the beginning meant as a complete memory manager so it's

internal functionality is absolutely incompatible with anything other

(even VCPI's ProtMode level 0).

* When I install REMUS there is no EMS memory in system. Some of my programs

don't want to run at all.

- EMS services are not yet supported by REMUS, sorry...

* My application told me I have no VCPI compatible environment and refused

to run when I started it under REMUS.

* My application refused to run with exit message about impossibility to enter

DPMI protected mode.

- VCPI/DPMI are 32-bit Protected mode interfaces allowing your applications

to run 16/32-bit protected mode code under other 32-bit PM environment

installed in memory. Although these ifaces are not yet supported under

REMUS, there exist light of hope that DPMI will be supported in far far

future. Right now DPMI interface is provided to ensure clients using it

won't hang computer via their lame PM detection (ie. Borland's dpmiload).

* How much system memory takes REMUS for its work ?

- REMUS takes about 1k of conventional mem. In fact, everything is now in

Xmemory (above 1MB) and takes there about 25kB-1050kB depending on current

running emulation. But Xmemory requirements probably don't bother anyone.

* GUS emulation is sometimes STRANGE !! often "ultraclicks" or noise occurs !!

- The noise is generated due GUS emulation sampling frequency. Try to use

at least 75% (33kHz) or more of GUS real sampling speed. Emulation engine

is rather old and I'll probably try to make completely new engine some day.

Maybe then also those painful ultraclicks disapper.

* emulations on SB are really slow, sometimes computer hangs up.

- Because of SB port speed emulations work fine up to about 20kHz. Above

this limit SB reply is too slow and machine often hangs up. This problem

will disappear after making DMA manager and DMA-using SB support.

* all emulations on PCSP are sometimes useles (quiet).

- In some programs PC Speaker is switched off and then no emulation works

(speaker is simply quiet).

- This is fault of my PCSP-driver and will be corrected in one of the next

versions of REMUS.

- example: Fascination game by Koktel vision.

* I've heard Disney Sound System sound card is something like D/A converter.

Is it right ? And what about Covox Speech Thing ?

- Those cards offer standard DAC on LPT compatibility, but they have

probably also some DMAs or other things for faster playback. I have no

documentation about them and also some good programs to test functionality.

- Already working example: Another World game by Delphine software (DAC LPT)

-     Not working example: Wolfenstein 3D game by ID software (DMA DAC ??)

* I've seen there are some strange emulations (for example: D/A on D/A). What

are they for ?

- these emulations are for people owning the right sound card but on another

port / IRQ / DMA they need...

* Could I use D/A -> D/A emulation for redirecting printer to another LPT ?

- No, generally not. Emulation works only for D/A output, LPT control port

emulation is designed to work moreless like Covox/Disney Sound Source's

LPT DACs, not like I/O ports of standard printer port.

* Some emulations are generally quite slow. What's the problem ?

- With some soundcards emulation could not be fast because of their speed.

- Some emulated sound cards are so hard to emulate (example: GUS) that the

emulation is getting to be quite slow.

- Protected Mode system exception handling eats some system resources

so even simple emulations are slower than should be.

- example: D/A converter on D/A converter.

* I've realised that all IRQ and DMA emulations don't work. Am I right ? Why ?

- yes, they don't work. Some nasty bugs in REMUS's IRQ/DMA manager cause

that all emulations which need some IRQ or IRQ/DMA don't partially or

completely work. IRQ/DMA is currently needed for emu_GUSC and emu_SB.

* Hey guy, where is my card in the list of supported soundcards ? How you dare

my soundcard is not supported ?

- That's not my fault. Your card will be supported if I get enough info

about it and some program which uses it (for testing). Then I would

probably include it in REMUS. Also one piece of that card borrowed would

greatly improve implementing.

* Ugh, why is some card listed in emulations but is not supported as base ?

- That's simple. I would probably not include any sound card support as base

until I get it for testing. But emulation fuctionality I can easily test

in available programs by setuping them to that card.

* Hey boy, I wanna emulation of XY$@#325 sound card and there is none. What

should I do for it ?

- First you should look in the FUTURE section to know the status of that

card. If you find no progress of that card there, you should look for

some programming info about that sound card and then you can send me this

info and the program which works with that sound card (for testing).

If you don't find any programming info you can still send me the program

that uses it and i'll try luck and debug that piece of code.

But in both cases don't expect fast response - I'm really really lazy :-)

Basic block of emulation takes me about 2-3 days to complete but really

powerfull and featured emulation takes me about next half year of my lazy

programming :-)

WHERE TO FIND REMUS:

-----------------------

Each time I have a new version of REMUS available, I'll upload it to:

ftp://sorry.vse.cz/dimension   (Dimension FTP site in Prague, Czech Republic)

http://sorry.vse.cz/dimension   (Dimension Inet HeadQuarters, Czech Republic)

REMUS will be probably also available at ftp.cdrom.com and ftp.sunet.se

FTP archives.

Ask your local Internet or FTP expert on how to reach them.

Another way is to visit REMUS's homepage at address:

http://sorry.vse.cz/dimension/rdos/remus   (Http site in Prague, CZE)

THANX:

------

Although I did all the programming, there are few friends which helped me with

some things about this program. Thanx go to:

Ondrej Stasek (Ondar/Dimension) - testing and suggestions

David Voracek (Assassin/Dimension) -     - || -

and other friends for their (mostly) betatesting and suggestions..

CONTACTING ME:

--------------

If you want to contact me, write to address listed below. If you want to send

me suggestions, tips or simply greets, send it freely. Also some money would be

good as a power and motivation for further work :-) If you find any new sndcard

and its programming info you can also send it to me (the info). Nice would be

also any report of success in functionality of REMUS. Don't forget many

reactions will bring me in new effort in this rather old project.

My address:                           E-Mail: (non-commercial)

Roman Dolejsi (RDOS)                    roman@sorry.vse.cz

Husova 419, Vcelna                      roman@romeo.pf.jcu.cz

37382 Ceske Budejovice

Czech Republic

Please do not send me E-mails longer than 150kB (because of our local smtp

server limit).

*** end of REMUS documentation ***