COM port API for JOYCE v1.22

----------------------------

WARNING: This is not tested, and no CP/M support is provided for it as yet.

If you want to use this interface, you will have to write a patch for your

modem program.

This API forms a thin wrapper round the SVASYNC serial comms library.

All calls are made by executing the instruction ED FE with A=13 (0Dh).

The function number is passed in C, and is:

C = 0: Empty buffers (SVAsyncClear)

C = 1: Set handshaking lines (SVAsyncHand). E=lines:

d0 => DTR

d1 => RTS

d2 => USER

d4 => LOOPBACK

C = 2: Set Baud rate & stop bits (SVAsyncSet)

HL=Baud rate, encoded.

1:50     2:75     3:110   4:134    5:150

6:300    7:600    8:1200  9:1800  10:2400

11:3600  12:4800  13:7200  14:9600

15:19200 16:14400 17:28800 18:33600

19:38400 20:57600 21:115200

E=settings:

d0 set for 8-bits, reset for 7

d1 = 1

d2 set for 2 stop bits, reset for 1

d3 set if parity, else 0

d4 set if parity even, clear if odd

C = 3: Character input status (SVAsyncInStat).

Returns A=0 if no character available, nonzero if there is.

C = 4: Poll for character, return it in A (SVAsyncIn). Returns A=0 if none.

C = 5: Character output status (SVAsyncOutStat). Returns A=0 if no, 1 if yes.

C = 6: Output character in E (SVAsyncOut).

C = 7: FID_EMS for COM port FID. Entered with:

B = FID option byte, currently unused

D = message number to expand:

0 => successful load, E=device number obtained

1 => no free memory / devices

2 => COM device is already present

3 => incorrect FID environment

HL = buffer for message

Returns A=0 to unload FID; 1 to remain resident.

C = 8: Switch to using COM port no. E (SVAsyncInit)

C = 9: Get current COM port no. into A