::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::::

DISK.DOC

---------

This diskette should contain 6 additional files:

CONTROLR.DEF        The "master" .DEF file for the Am29116

+ misc other parts in comma-positional

notation.

CONTROLR.SRC        A test SRC file (partial microword).

DISKCTLR.DEF        The .DEF file for the Am29116-Am9520

(BEP) based disk controller that is

discussed in the application note,

"A High Performance Intelligent Disk

Controller" by O. Tabler and B. Kitson.

This file is in DEF statement-overlay

format.

DISKCTLR.SRC        The source code for the disk controller.

(This file assembles to a 92K P2L file.)

Am29CPU.DEF         The "MASTER" .DEF file for RALU-based

designs (Am2901-2903 or 29203) RALUs

+ misc other parts in comma-positional

notation.

Am29203.SRC         Sample source code for an Am29203 CPU

(Am2910 sequencer). (Uses Am29CPU.DEF)

These  AMDASM.DEF  files  are provided as a  service  to  AmSYS29

customers.   They  are intended to provide a basis from which the

customer   may  select  those  equates,   subs  and   definitions

appropriate to a given design problem.   It is intended that  the

customer  edit  a copy of one or more .DEF files to  produce  the

desired .DEF file.

The  .SRC files are provided as a reference.   The microword will

probably not match the customer's requirements.

CONTROLR.DEF

-------------

Originally created as a master file for study, it has been edited

into one application, "DISKCTLR.DEF".   It is recommended that the

user   compare   these  two  .DEF  files  for  similarities   and

differences.

The  file includes a number of parts which may or may not  appear

in  a  controller.   The first page after  the  WORD  declaration

provides an effective index to the file.   The various EQU groups

of the Am29116 instructions are indexed ([i]) for cross reference

by  the  DEF statements.   Each instruction type has its own  DEF

statement.

The  Am2910 instructions,  without any of the additional  control

lines (OEy,  RLD,  CCEN) are presented with a SUB statement.  The

Am2910,  a  3-bit  conditional MUX field,  and a 12-bit  variable

address  field are part of each of the following DEF  statements.

By editing the SUB statement, all of the other DEF statements can

be altered.

This means that any of the three fields can be altered or  even

eliminated by editing one statement.

Control_lines

Several  of the control lines for the Am29116 are presented in  a

second  SUB statement.   Note that not all of the  Am29116  lines

appear (the Ti lines, for example, were not input).

This  manner of definition requires comma positional substitution

in  the  source  file.   These  lines  can  be  edited  into  DEF

statements  and  the  SUB deleted to  convert  the  control  line

specification  to  overlays.   The  required usage  and  personal

preference are the deciding factors in the approach.

Instructions

Pages 8-20 are the Am29116 instructions.   Each class is  defined

by first defining the mnemonics and then the DEF statement.   The

mnemonics  are taken directly from the Am29116 data  sheet.   For

example,  SOR,  the single operand-RAM instruction format (page 5

of  the Am29116 data sheet) has a choice of word-byte mode ([m]),

1  of 4 op codes ([1]),  1 of 10 source-destination pair  selects

([2]), and requires a RAM register address ([R]).

Am2914

The instruction set only is defined.

Am2940

The  instruction  set only is defined.   The instruction  set  is

flagged by ".40" to avoid duplicate mnemonics with the Am2942.

Am2942

The instruction set only is defined.

Am2904

The Am2904 instruction set is very powerful and extensive.   Some

ideas for mnemonics are given for shift control and various other

commands and  some sample DEF statements are included.

The user must define the Am2904 by defining only what is required

for a given design.  There are no standard mnemonics.

Am2925

Equates  for  cycle length codes plus  place-holders  (incomplete

definition)  for control lines are provided.   Again,  DEF state#

ments may be used instead.

Am2950/51

A comment only.

CONTROLR.SRC

------------

This source file is for debug of the preceding .DEF file.  It was

prepared primarily for study.

Page  3 provides calls to the single operand instructions.   SONR

requires  an  immediate data word and it is provided via  a  free

format (FF) statement.

Example:

SOR   ,,, W, MOVE, SORA, R10

SONR  ,,, W, MOVE, SOI, NRS

FF    H#E, 15X, H#0123, B#01000, 24X

Note the commas.  This can be avoided by the use of overlays.

(Note that this is an incomplete microword.)

CONTROLR.DEF_+_CONTROLR.SRC_MICROWORD

2910      COND       BR       29116

INSTR     MUX       ADDR      CTLs      OPEN

4        3         12        16        24    =   64 bits

Note  on  address  fields:   To avoid  problems,  use  $  as  the

attribute in address field for AMDASM, i.e. 12V$X, 6V$X, etc.

Note  on microword:   This incomplete microword was laid out  for

study  of  the Am29116 and will not necessarily match  an  actual

microword  format for a controller.   The 24 don't cares would be

replaced by other controls and the microword might be larger than

64 bits.

DISKCTLR.DEF

------------

An  AMD application note is being prepared which discusses a high

performance  disk controller.   The .DEF file for the  controller

was created by editing the "master" file.

The  Am29116 instructions were left basically intact;  the  major

change  was the deletion of the Am2910 and ZZZZZ SUB  statements.

The  length  of the microword was also increased from  64  to  80

bits.   A  global  substitution (under ED,  the editor)  replaced

Am2910 by nothing and ZZZZZ by 64X.

The instruction sets for all other parts were deleted.  Beginning

on  page  19,  EQUs and DEFs were added according to  the  design

requirements.   The choice here was to use overlay in the  source

file;  therefore,  all  Am2910  instructions were defined as  DEF

statements rather than equates.

Immediate_Operand

The  study  file used FF statements in the .SRC file  to  provide

data for immediate data operand instructions.   Here,  IMME is  a

DEF statement which can be overlayed with other DEF statements to

build a microword.

Control_Lines

Control  lines for the Am29116 are defined in DEF statements  for

overlay.

Am2910

The  Am2910 command are defined in DEF statements.   Note the use

of JS,  JP, RTN, IF and IF NOT which have been added to the basic

16 instructions.

Misc_Control_Signals

All of the remaining control signals appear as DEF statements for

overlay in the source file.

The_Microword_-_DISKCTLR.DEF

Bit_Position________#_________Function_____________________

0-15            16         Am29116 Instruction

0-15            16         Immediate Data Operand

16-19             4         CT Multiplexer Control (T#i# Lines)

20               1         Status Register Enable

21               1         Ouput Enable - Yi

22               1         Instruction Enable

23               1         Data Latch (D-I-Latch) Enable

24-27             4         Am2910 Instruction

28-37            10         Branch Address

38-42             5         Condition Code Selections

43               1         Parity

44               1         Address Mark Control

45               1         Memory Bus from Drive Control Bus

46               1         Memory Bus from Translate PROM

47               1         Memory Bus from 9403AS

48               1         Memory Bus from Am29116

49               1         Memory Bus from Am9520 - LSB

50               1         Memory Bus from Am9520 - MSB

51               1         (Disk) Bus Direction out from

Controller

52               1         Memory Bus to 9403AS

53               1         Memory Bus to Am29116

54               1         Memory Bus to Am9520 - LSB

55               1         Memory Bus to Am9520 - MSB

Bit_Position________#_________Function______________________

56               1         Memory Bus to Am9520 - Control

Information

57               1         Clock Enable Am9520 to Lower-

Byte Bus Int.

58               1         Clock Enable Memory Bus to

Am9520 Transfer

59               1         Clock Pulse (Actual Waveform)

for Am9520

60               1         Command Request

61               1         Input Serial Data to 9403AS

62-63              2         Jump Indirect Am29116 Register

62-63              2         No Indirect Jump

64               1         Memory Access

65               1         Memory Address

66               1         Memory Write

67               1         Output Serial Data from 9403AS

68               1         Parameter Enable

69               1         Set 9520 P Bits from 9520 PM Bits

70               1         Parallel Fetch from 9403AS

71               1         Parallel Load into 9403AS

72               1         Parameter Request

73               1         Read Gate

74               1         Reset FIFO

75               1         Select/Attention Strobe

76               1         Write Gate

77-79              3         Translate Prefix

Example_Source_Code_Lines

SOR  W, MOVE, SORY, R4  & NODLE  & NOIEN

\    & OEY  & NOSRE  & MADR  & NOJMPI  & CONT

;    MOVE R4 TO MAR

SOR  W, MOVE, SOZR, R0  & NODLE  & IEN

\    & NOOEY  & NOSRE  & NOJMPI  & RTN

;    Load 0 into R0 and return

DISKCTLR.SRC

--------------

This file, a very large one, was prepared for the AMD application

note,  "A High Performance Disk Controller".  The code is written

using  overlaid  DEF  statements  with  positional  substitutions

minimized (primarily appears in the Am29116  instructions).   The

code is commented.

Am29CPU.DEF

-------------

The   original   AmSYS29  software  included  a   library   file,

AM2900.LIB,  which  provided the user with a source of predefined

mnemonics for various parts.   The AM29CPU.DEF file is the latest

descendant of that original file.

The  AM29CPU.DEF  file provides mnemonics for a number  of  parts

considered to be likely to appear in a CPU design,  as opposed to

a  controller  design.   This includes  the  Am2901,  Am2903  and

Am29203  RALUs.    Flags  are  used  to  differentiate  duplicate

mnemonics.  (The Am29203 is given priority.)

The user must edit a version of this file to complete a .DEF file

for  a  particular application.   Most of the equate  groups  are

indexed ([i]) for cross reference from DEF and SUB statements.

Am2901

The oldest RALU, the function and destination fields are flagged, by .01.

Example  SUB statements are included  following  the   equates.

The mnemonics are from the data sheet.

Am2914

Only the instruction set is included

Am2930

Only the instruction set is included

Am2932

Only  the  instruction set is included.   The instruction set  is

flagged by ".32" to differentiate it from that of the Am2930.

Am2940

Only the instruction set is included.

Am2902_-_Am29203

The  instruction  set  is  flagged by ".03"  in  cases  where  it

duplicates  that of the Am29203.   Certain equate groups  (source

select  and  destination for example) are the same  for  the  two

parts.   The  differences are in the function set and the special

functions.   While many of the equates in these actually were the

same,  there  were  enough  differences  to  flag  that  separate

definitions were felt to be clearer.

Expanded_Memory

The Am2903 and Am29203 have expandable registers.   The  Am29705/

29707  2-port memories allow groups of 16 registers to be  added.

This changes the microword source fields.   Example mnemonics are

provided for a particular diagram.

Register [R]

All RALUs use 4-bit addresses for the on-board registers.

Carry

A  few  not-inclusive  definitions are  provided.   This  is  #not#

programming for the Am2904.

Am2910_-_Am29811

The  instruction  set for the Am2910 is  provided.   The  Am29811

differs only in the last mnemonic,  substituting JP for TWB.   An

example SUB statement is provided.

Am2925

Cycle  length  codes  and  an indication  of  control  lines  are

provided.  The control line definitions are incomplete.

Am2904

As  in the CONTROLR.DEF file -- incomplete as far as total Am2904

capabilities.   Intended  to provide ideas.   The  customer  must

create his or her own definitions for equates and DEF statements.

There are no standard mnemonics.

DEF_Statements

Comma  positional within the part and overlay are used to  create

the  microword.   The DEF statements are intended to  demonstrate

typical ALU and sequencer definitions.  The balance of the micro#

word is undefined.

Note  the use of commented default line plus the index  reference

[i].   The  reference  identifies equate mnemonics which  may  be

substituted  in  a given variable field in place of  the  default

value.

DEF  statements  for the Am2904 are given as well  as  statements

specific   to  a  particular  .SRC  file.    This  was  done  for

demonstration -- one DEF file may be used with more than one  SRC

file.   The  .DEF  file  is  then the  main  or  master  mnemonic

reference for a set of source programs.

Am29203.SRC

This  file,  developed from one that was originally prepared  for

the  Am2903  (AM2903.SRC)  was  prepared to  demonstrate  how  to

program the Am29203.

The microword is in a conceptual, standard format:

2 9 1 0              2     9     2    0   3                   2 9 0 4

2910  COND  BR  SOURCE  FUNC  DEST  CARRY  Ra   Rd  Ien  0ey   Shift  SE

INSTR MUX  ADDR    A      L     U     IN  ADDR ADDR       en   INSTR  en

4    3    12     3      4     4     2    4    4    1    1 14x  4    1 3x

47 bits + 17 don't cares = 64 bit word

Sample_Code_Lines

AM2910  RPCT,  ,LB & AM29203  ,SPECL, MULT, NOCARRY, R1, R0

AM2910  & AM29202  RAMAB,  ,RAM,,,R15,,OENDIS

AM2910  & AM29203  ,ADD,RAMQUPA,NOCARRY,R2,R2  & SHIFT SUR2Q2

Since the code was prepared for study,  it is more detailed  than

might  otherwise be necessary;  i.e.,  defaults are resubstituted

occasionally, etc.  Improvements in readability could be obtained

by  the  use of specialized DEF statements and increased  use  of

overlays.

###########################