================

** SYSBLD.DOC **

================

The following instructions are designed to help you build a

62K CP/M system.

STEP #1. Use MOVCPM to create a COM file with the basic CPM

system in it. Call it CPM62.COM.

STEP #2. Alter CDCIOS to fit your specific hardware configuration.

Assemble CDCIOS and produce an object file CDCIOS.HEX.

STEP #3. Repeat STEP #2 for CDCBOOT and CDCPUT.

STEP #4. Put all the pieces together to form a complete system.

Save that system on your diskette. It is done thusly:

A>DDT CPM62.COM		;Load DDT and CPM

DDT VERS 2.0

-ICDCPUT.HEX

-R			;Read CDCPUT into RAM at 100H

-ICDCBOOT.HEX

-R880			;Read CDCBOOT into RAM at 880H

-ICDCIOS.HEX

-R2D80			;Read CDCIOS into RAM at 1F80H

-G0			;Warm Boot CP/M

A>SAVE 40 CDC62.COM	;Save the system on your diskette

STEP #5. Assemble the disk format program CDCFMT. Use it to format

both the fixed and removable platters.

STEP #6. Write the system onto the removable disk pack. It is

done like this:

A>CDC62

STEP #7. Boot the system and test it.

NOTES ----

1. Because of the size of CDCIOS, the system requires an additional

2K bytes of memory. In other words, if you have 64K of memory

you'll have to build a 62K system. You will have to reduce the

size of the system an additional 1K if you are using a 96 Meg

Phoenix.

2. Those familiar with CP/M V1.4 will note that the SYSGEN memory

layout for BOOT, BDOS, & CBIOS has changed. BOOT starts 80H bytes

lower in RAM because the sector size of the hard disk is 256 bytes

in length (as opposed to a floppy disk which has a sector length

of 128 bytes).  CBIOS starts 100H bytes higher in RAM because BDOS

has grown.

BOOT	BDOS	CBIOS

1.4	900H	980H	1E80H

2.2	880H	980H	1F80H

##################################################################################