TIME

(Version 1)

Description

The TIME utility is a dual purpose program.  It is used to display -- or set

-- the current system time.

Use of the TIME utility requires three things:

o  AMPRO Z80 BIOS Version 3.7 (or later) be installed.

o  The Real Time Clock function within the BIOS must be enabled.

o  The LITTLE BOARD single board computer must be properly jumpered for

the system time to increment properly.

You can observe that the correct BIOS is installed and that the clock function

is enabled by observing the signon text after system reset, which should

include:

AMPRO BIOS Version 3.7                 <-- Version 3.7+ required.

(Clock enabled)

The LITTLE BOARD must be configured as follows, depending on which model is

present in the system:

Model 1B - "LITTLE BOARD/PLUS" - (AMPRO part number A60060 stamped on the

component side of the board.)  This assembly has jumper pins for

configuration of the Real Time Clock option.  Pins 1 and 2 of JMP2 must

be shorted, and pins 2 and 3 of JMP2 must be open.  Early assemblies of

this model had a trace shorting JMP2 pins 2 and 3 on the circuit side

(bottom) of the board, which must be cut if present.

Model 1A - "Original LITTLE BOARD" - (AMPRO part number A13001 stamped on

the component side of the board.)  This assembly requires that a trace on

the circuit side (bottom) of the board running to pin 20 of U6 (the Z80

CTC) be cut, and that a wire be added between U6 pins 9 and 20.

Operation

To set the TIME, type the program's name followed by the current time

(hh:mm:ss) at the operating system prompt.  The seconds (third set of digits),

if left out, are assumed to be 00.  Here are some examples:

A0>TIME 8:25:50<RETURN>       ...time set to 8:25:50 AM

A0>TIME 15:03:00<RETURN>      ...time set to 3:03:00 PM

A0>TIME 6:30<RETURN>          ...time set to 6:30:00 AM

To display the system time, simply enter the program's name at the operating

system prompt:

A0>TIME<RETURN>

################################################