DATE

(Version 1)

Description

The DATE utility is a dual purpose program.  It is used to display -- or set

-- the current system date.

Use of the DATE utility requires three things:

o  AMPRO Z80 BIOS Version 3.7 (or later) be installed.

o  The Real Time Clock function within the BIOS must be enabled.

o  The LITTLE BOARD single board computer must be properly jumpered for

the system date to increment properly.

You can observe that the correct BIOS is installed and that the clock function

is enabled by observing the signon text after system reset, which should

include:

AMPRO BIOS Version 3.7                 <-- Version 3.7+ required.

(Clock enabled)

The LITTLE BOARD must be configured as follows, depending on which model is

present in the system:

Model 1B - "LITTLE BOARD/PLUS" - (AMPRO part number A60060 stamped on the

component side of the board.)  This assembly has jumper pins for

configuration of the Real Time Clock option.  Pins 1 and 2 of JMP2 must

be shorted, and pins 2 and 3 of JMP2 must be open.  Early assemblies of

this model had a trace shorting JMP2 pins 2 and 3 on the circuit side

(bottom) of the board, which must be cut if present.

Model 1A - "Original LITTLE BOARD" - (AMPRO part number A13001 stamped on

the component side of the board.)  This assembly requires that a trace on

the circuit side (bottom) of the board running to pin 20 of U6 (the Z80

CTC) be cut, and that a wire be added between U6 pins 9 and 20.

Operation

To set the date, type the program's name followed by the current date

(mm-dd-yy) at the operating system prompt.  Here are some examples:

A0>DATE 4:7:86<RETURN>        ...date set to Apr 7, 1986

A0>DATE 12:25:86<RETURN>      ...date set to Dec 25, 1986

To display the system date, simply enter the program's name at the operating

system prompt:

A0>DATE<RETURN>

##########################################################################