Colex 800 series port addresses:

By Robert Harker

March 28, 1987

000H      SCC0C     Serial channel 0 conrtol port

001H      SCC1C     Serial channel 1 control port (console)

002H      SCC0D     Serial channel 0 data port

003H      SCC1D     Serial channel 1 data port (console)

TXRDY0    channel 0 TX ready mask       00100100B

CTSCH0    channel 0 CTS mask            00100000B

TXRDY1    channel 1 TX ready mask       00001100B

TXRDY1    channel 1 CTS mask            00001000B

RXRDY     RX ready mask                 00000001B

RSTIUS    reset intrupt under service   00111000B

004H      CIOPCD    CIO data port C

005H      CIOPCB    CIO data port B

006H      CIOPCA    CIO data port A

007H      CIOCTL    CIO control port

CH0RDY    Channel 0 ready          00000100B

CH0STB    Channel 0 status bit (?) 00001000B

CH1RDY    Channel 1 ready          00000001B

CH1STB    Channel 1 status bit (?) 00000010B

CTC chip (on STD-CPU2 board)

07CH                Base address of CTC chip

SASI hard disk controller board

0A0H      HDDATA    SASI data port

0A1H      HDSTAT    SASI control/status port

0A2H      CTRLA

0A3H      CTRLB

STD VT52 terminal control board

0DCH      CRTDAT    STD CRT data port

0DDH      CRTSTA    STD CRT status port

Floppy disk controller board

0E0H      FDCDMA    Address of Z80 DMA chip

0E3H      FDCSEL    Address of drive/size/etc port

0E4H      FDCCMD    WD1797 command register

0E4H      FDCSTA    WD1797 status register

0E5H      FDCTRK    WD1797 track register

0E6H      FDCSEC    WD1797 sector register

0E7H      FDCDAT    WD1797 data register

DRIVE0    Drive 0 select mask           00000001B

DRIVE1    Drive 1 select mask           00000010B           DRIVE2    Drive 2 select mask           00000100B

DRIVE3    Drive 3 select mask (8" only) 00001000B

Memory bank selector port

0FFH                Memory bank selector port