Documentation USEN1..ASM, USEN2.ASM, USEN3.ASM

These routines allow the output normally sent to the LST: device

to be sent instead to the SEN: device. The advantage is that

data is collected in 128-byte buffers in CP/M and then sent

to the PDP-11 128-bytes at a time. This results in less overhead

and faster printing, especially when console echo is used (ctrl-

P). This is a BIOS extension to the resident CP/M system. When

ctrl-P is used it is handled by the extension. When ctrl-P

enables echo the terminal beeps one time. Two times when

the echo is disabled.

This routine can be installed for manual or automatic use.

When used manually to run Palantir, for example the file

USEN.COM is created accordin to the instructions in USEN2.ASM

and the normal command line is preceeded by USEN

A>USEN WPVT100

When installed to run automatically, the resident system is

modified to run USEN.COM automatically, every time the system

boots. This is done by following the instructions in USEN3.ASM.

If the resident system is modified, then memory is wiped out

by the auto loading every time there is a warm boot and

operations which depend on memory remaining intact do not work.

To restore the resident system copy an unmodified copy from

another disk using GENSYS.

##cp buffer

mov	a,c	;count to move

stax	d	;to ccp buffer

inx	d	;point to message area of ccp buffer

mvl3:	mo