Software diskette #1

-----------------------------

1.Programmer Files :

EPP512.EXE   : EPROM programmer software.

2716 to 27512,

2516 to 2564,

8764 to 87256,

2816 to 2864A,

CYPRESS CY7C251,7C261,7C271...

NS NMC 27C16 to 27C512...

EPP1024B.EXE : 1 M bit EPROM include

BYTE wide(27010,27101,271000,271001)

PAGE mode(27513,27011..),

MASK ROM pinout EPROM(231024,27301,271000,271001),

BYTE WIDE 2M,4M bits EPROM(TC572000,574000).

EPP1024W.EXE : 1 M bit EPROM include

WORD wide(27210)

FPLP1.EXE    : FPLA series of SIGNETICS

PLS153/A,PLUS153,PLS173,PLUS173...

PLHS18P8A,PLHS16L8A/B.

FPLP2.EXE    : FPLA series of SIGNETICS.

SIGNETICS PLC42VA12.

2.REVISION of programmer software :

Hardware Revision on  :

VERSION 1 : 1st version hardware.4/15/1990.

current limit resistor from 22 ohm

to 10 ohm.

VERSION 2 : 2nd version hardware.12/1/1990.

Add GND control for PIN 30 to solve

40 PIN word-wide EPROM problom.

EPP512.EXE : V1.00 : 1st released version.

V1.01 : Software bug on 2564 programming is solved

V1.02 : Add a powerful screen editor to replace DEBUG.COM,

please ref. section 5.

V1.03 : NMC 27C256A problem solved.

V1.04 : NMC9817J-35 problem solved.

V3.00 : new version.

V3.10 : 1/1/91, Split EEPROM to EEP1.EXE.

V3.11 : 2/8/91, Add INTEL 87C257.

V3.12 : 3/1/91, Revision of programming algorithm and delay

time for NS Mfr. as well as program voltage

and delay time for CYPRESS.

V3.13 : 4/1/91, Update load file.

Add ATMEL 27C513,27C256R,27C512R,27C513R.

V3.14 : 6/20/91,Add SEEQ 2764,27128,27C256.

V3.15 : 7/1/91, Add MXIC 27C256,27C512,27C1000.

V3.16 : 10/1/91,Add MITSUBISHI M5M27C256AK-I,M5M27C512AK-I.

HITACHI HN27C256AG/HG,HN27C512G.

INTEL D27C256,TI 87C257,NEC D27256.

V3.18 : 12/20/91,Revision of programming algorithm for CYPRESS.

V3.19 : 1/15/91, Add GENERAL 27C513, MICROCHIP 27C513,

INTEL 27C513.

EPP1024B.EXE : V1.00 : 1st released version.

V1.01 : Add a powerful screen editor to replace DEBUG.COM,

please ref. section 5.

V1.02 : Add BYTE WIDE 2M,4M bits EPROM(TC572000,574000).

If memory size is not enough for extra 512K BYTES

allocation,this software can not program 4M bits

EPROM.

If memory size is not enough for extra 256K BYTES

allocation,this software can not program 2M bits

EPROM.

If memory size is not enough for extra 128K BYTES

allocation,this software can not program 1M bits

EPROM.

V1.03 : Bug on 27513 content reading is solved.

V3.00 : new version.

V3.08 : 1/1/91, Add new function "COPY file to EPROM",

"READ EPROM to file".

V3.10 : 2/8/91, Enhance S/W to calculate the check sum

while in "EPROM to File" function.

Enhance S/W to calculate the sheck sum

while in "File to EPROM" function.

V3.11 : 3/1/91, Retrieve programmer function.

V3.12 : 4/1/91, Update load file.

Add ATMEL 27C010/L,27C1024/L,27C011.

V3.13 : 5/1/91, Transfer file to EPROM by way of drive A or B

can be selected.

V3.14 : 6/1/91, Add ATMEL AT32C16,WSI 57C65,57C257,57C201F

V3.15 : 7/1/91, Add MXIC 27C1000.

V3.18 : 9/5/91, Add FUJISU 27C4001.

V3.20 : 10/1/91,Increase the range of "read EPROM to file".

Increase the range of "copy file to EPROM"

V3.23 : 11/13/91,Enhance S/W to calculate the check sum while

in "EPROM to file" function.

V3.25 : 1/15/92, Add "VERIFY file with EPROM" function.

Seperate special Byte wide portion from

EPP1024.EXE

EPP1024W.EXE : V3.25 : 1/15/92, Add "VERIFY file with EPROM" function.

Seperate special Word wide portion from

EPP1024.EXE

FPLP1.EXE    : V1.00 : 1st released version of FPLA SERIES.

FOR SIGNETICS PLS153/A,PLS173,PLUS153,PLUS173.

V1.01 : Add signetics PLHS18P8A,PLHS16L8A/B.

NOTE : PLHS18P8A,16L8A/B Blank fuses are opened.

In JEDEC fuse map, We define opened fuse to be

'1'.

But in PLHS18P8A, XOR fuse configuration are

inversed from JEDEC AND AMD18P8 configuration,

so we define Blank xor fuse to be '0' and at

low(or invert) output polarity.

V1.02 : Add AmdPAL18P8.

V3.00 : new version.

V3.03 : 1/1/91.

V3.06 : 7/1/91, Revison of text on the screen.

V3.08 : 10/1/91,Revision of text on the screen.

V3.11 : 12/6/91,Revision of LOAD file function.

V3.13 : 1/15/92,Revision of text on the screen.

Revision of PLS153A, PLHS18P8 programming

voltage.

FPLP2.EXE    : V3.00 : 1st released version of FPLA SERIES.

6/3/91, SIGNETICS PLC42VA12.

V3.02 : 10/1/91,Revision of text on the screen.

V3.04 : 12/19/91,Revision of LOAD file function.

Add RICOH EPL16P8B.

Add capacity for "Load file".

V3.05 : 1/15/92, Revision delay time for PLC42VA12.

3.NOTE : We will keep updating software for new programmable devices.

Special device will be arranged to be 1st priority, if user offer us

the programming information of that device.

4.Powerful screen editor function description :

Some users said that it is unconvenient to specify the address by segment

prefix form during DEBUG MODE .(such as F000:E000 represent address FE000).

User only want to enter the offset address in memory buffer.

According to upper reasons, we develop a simple screen editor that will

meet the requirment of users.

Although the DEBUG.COM has been replaced, but user still can invoke it

by SHELL( DOT ) command in this editor.

Editor command are almost the same as DEBUG.COM, such as

D,E,M,F. It is not necessary to enter the segment address before

the start address,end address. Only the offset address in memory buffer is

needed.

The extra commands are shown below.

E : Same as E command in DEBUG.COM, except that user can modified

data in full screen editing mode.

There are two editing fields, BINary and ASCII, you can edit

any datas on each field.

P : Same as D command,except that the data will be printed to the

printer.

C : The command will calculate the total sum between start and end

address.

S : Will search the ASCII string that are included between start and

end address.

B : Will search the BINARY data that are included between start and

end address.

. : Will execute the excutable file and pass argument to the file.

This command liked the BASIC SHELL command can invoke any file and

pass controll to it. So you can invoke your own editor to do more

powerful job.

Note : The actual segment address of memory buffer shown on

main menu is for referenced only. You can pass this parameter

to the invoked file for special purpose.

? : Will display the editor commands.

------------------------ END -----------------------------