FLOPCOPY.ASM     FLOPPY DISK COPY PROGRAM USING 1 DRIVE AND A HARD DISK

WRITTEN BY G. YOUNG, PO BOX 3218, NORTH HOLLYWOOD, CA 91609

FLOPCOPY IS A PROGRAM USED TO COPY ONE FLOPPY DISKETTE TO ANOTHER FLOPPY

IN SYSTEMS THAT HAVE ONLY ONE FLOPPY DRIVE BUT HAVE A HARD DISK.  IT WORKS

AS FOLLOWS:

1. READ THE DIRECTORY OF THE FLOPPY DISK USING THE MASK IN THE COMMAND

TO CREATE A LIST OF FILES TO BE COPIED (EXAMPLE: FLOPCOPY *.ASC TO

COPY ALL THE ASC FILES FROM ONE FLOPPY TO ANOTHER).  THEN, USING THE

LIST IN MEMORY, COPY THE FILES FROM THE FLOPPY TO THE HARD DISK.

2. IF THE FILE ALREADY EXISTS ON THE HARD DISK, ASK IF YOU WANT THE FILE

COPIED OVER WITH THE FLOPPY VERSION AND SET AN INDICATOR IN THE  FILE

LIST THAT THIS FILE WAS ALREADY THERE.  THEN THE PROGRAM WILL REQUEST

THE BLANK DISKETTE THAT WILL BE THE DESTINATION DISK TO BE MOUNTED.

3. USING THE FILE LIST NOW IN MEMORY, THE FILES ARE THEN COPIED FROM THE

HARD DISK TO THE BLANK FLOPPY.

4. WHEN ALL FILES HAVE BEEN COPIED, THE PROGRAM WILL ASK IF ADDITIONAL

COPIES ARE TO BE MADE.  IF NOT, THE FILES ARE DELETED FROM THE HARD

DISK.  IF THE FILE WAS ALREADY ON THE HARD DISK AS DETERMINED BY

INDICATOR SET PREVIOUSLY, THE FILE WILL NOT BE DELETED.  THE PROGRAM

IS SMART ENOUGH TO DETECT A FULL DISKETTE CONDITION.

############################################################