General discussion.

The CETAPE module is designed to allow programs written in C to effectively

access tape files for both input and output.  Before any tape commands are

issued, it should be initialized via function call 32.  Durring initialization

all available memory (under 640k) is allocated for tape buffers in order to

achieve the highest possible thoughput.  If the C program needs to have free

space (for "malloc", etc.), then it must be reserved when function 32 is called.

Read functions are processed as follows:

When the first read (function 2, 29 or 31) is processed, tape blocks are read

into memory until one of the following: 1) Memory is full, 2) A file mark is

read, or 3) a hard error occurs.  On this and following read calls any blocks

read correctly are passed back to the C program with good status codes.  When

there are no more blocks in memory, the above is repeated until terminated by

a file mark or an error.  After reads the C program should check the status

bits returned for Online, data overrun (data longer than the buffer provided),

hard error, and file mark.  When ANY other tape command is issued, it aborts

the streaming read and if file mark has not yet been detected, any data

remaining in the memory buffer will be flushed and in general the physical

position within the file will be unknown to the C program.

Write functions are processed as follows:

When the first write (function 1, 28, 30) is processed, the record/block is

simply placed in the memory buffer.  Following write calls continue to place

data in the memroy buffer until there is no more room.  At that time, all

blocks in the memory buffer are written to tape.  After each call, the C

program should check the status bits returned for Online, hard error,

and file protect.  When ANY other tape functions are issued, it first

causes all data from the memory buffer to be written to tape.  The only

exception to this is end of tape volume processing from within 'tapeov'.

See discussion of this below.  Note that each block of data is not immediately

written to tape when the write function is called.  For this reason if an

error occurs, there will in general be some number of blocks which already

have been passed to the module but not yet written to tape.  Hence, when the

error status is passed back to the C program, it does not know how many blocks

were correctly written or which block caused the error.  It is assumed that

when a fatal write error occurs (after many retries and having erased several

inches of tape looking for a good spot to write on) that the entire file will

have to be discarded and rewritten.

All other tape functions:

All other tape functions are executed immediately when recieved.

Tape unit handles.

Provision has been made to allow reading/writing from/to more than one tape

unit at a time.  To do this, the number of logical tape units required must

be specified via the initialize function (32).  It is assumed that each logical

unit is a different physical tape drive.  If not, your blood be upon your own

head!  Strange and wonderful things may happen.  If, for example, two units are

specified at initialization time.  This establishes unit handles 0 and 1 with

each being allocated one half of available memory.  The C code to initialize

two tape drives having physical addresses 0 and 1 could be as follows:

int addr;     /* physical tape address ranging from 0 to 7 */

int reserve;  /* Amout of main memory to be reserved for C */

int ts;       /* tape status returned from calls to cetape */

reserve = 0;  /* don't need to save any memory */

ts = cetape(2,32,&reserve);  /* init memory, establish handles 0 and 1 */

/* 'reserve' should be checked here to determine if there is enough memory

for the program to work.  The absolute minimum (for each unit) is twice

the size of the largest block to be read or written.

*/

addr = 0;

ts = cetape(0,27,&addr);     /* set handle 0 to access tape address 0  */

addr = 1;

ts = cetape(1,27,&addr);     /* set handle 1 to access tape address 1  */

Tape service routines.

A.  Calling format.

All tape service routines are called using the following format:

ts = cetape(handle,fun,&cnt,&buffer,blksize);

ts = Completion status from the tape drive.  This is bit sensitive.

15 - Online.  If not true after each operation, the unit has

gone off line.

14 - Speed/Density line from the tape drive.  Meaning may vary

depending on the tape drive.

13 - Data busy.  True while data is being transferred to or from

the tape.  Application program should ignore this bit.

12 - File protect.  True if the tape is write protected, that is,

if the write enable ring is not installed.

11 - Rewinding.  True while the tape is in the process of rewinding.

After issuing the rewind command, the user shuld loop rereading

status and checking this bit until it goes false to insure that

the tape has completed rewinding.

10 - Ready.  Tape is ready for another command.  The application

program may ignore this bit.

9 - Load point.  True if the tape is at load point.

8 - Formatter busy.  The application program may ignore this bit.

7 - DMA across 64k.  This error should not occur while using the

tape interface module.

6 - Command reject.  Invalid command received.

5 - N/A

4 - DMA request.  Must be true after each read.  If false, then

the record from tape was longer than the requested size.

3 - Hard error.  Unrecoverable read or write error.  Should be

checked after each read or write.

2 - End of tape.  Reflective spot at the end of tape has been

detected. Causes call to user's 'tapeov' routine.

1 - Corrected error.  The read or write operation had an error which

was correctable using the parity track.

handle = The logical tape unit to be used.  If multiple logical units have

not been initialized (see function 32), then this should be given

as zero.  This is a 16 bit integer value.

fun = Tape function to be performed as follows:

0 - Return current status.         cetape(h,0)

1 - Write a block.                 cetape(h,1,&cnt,&buffer)

2 - Read a block.                  cetape(h,2,&cnt,&buffer)

3 - Write in edit mode.            cetape(h,3,&cnt,&buffer)

4 - Read in reverse edit mode.     cetape(h,4,&cnt,&buffer)

5 - Read in reverse mode.          cetape(h,5,&cnt,&buffer)

6 - Write tape mark.               cetape(h,6)

7 - Forward space file.            cetape(h,7,&cnt)

8 - Back space file.               cetape(h,8,&cnt)

9 - Forward space block.           cetape(h,9,&cnt)

10 - Back space block.              cetape(h,10,&cnt)

11 - Rewind.                        cetape(h,11)

12 - Rewind & unload.               cetape(h,12)

13 - Set low density.               cetape(h,13)

14 - Set high density.              cetape(h,14)

15 - Set fast speed.                cetape(h,15)

16 - Set slow speed.                cetape(h,16)

17 - Set long interrecord gap.      cetape(h,17)

18 - Set short interrecord gap.     cetape(h,18)

19 - Erase fixed length (3 inches). cetape(h,19)

20 - Erase to end of tape.          cetape(h,20)

21 - Set the number of retries.     cetape(h,21,&cnt)

22 - Reset error counts.            cetape(h,22)

23 - Return read error count.       cetape(h,23,&cnt)

24 - Return write error count.      cetape(h,24,&cnt)

25 - Tape card present?             cetape(h,25,&cnt)

26 - Tape unit on line?             cetape(h,26,&cnt)

27 - Set tape address.              cetape(h,27,&cnt)

28 - Fixed block write.             cetape(h,28,&cnt,&buffer,blksize)

29 - Fixed block read.              cetape(h,29,&cnt,&buffer,blksize)

30 - Variable blocked write.        cetape(h,30,&cnt,&buffer,blksize)

31 - Variable blocked read.         cetape(h,31,&cnt,&buffer,blksize)

32 - Initialize memory.             cetape(n,32,&cnt)

'n' is the number of logical drives to be initialized.  When more

than one is requested (maximum is 4), an equal amount of memory is

allocated for each logical drive.  Notice that a logical drive

(handle) must be supplied for each tape function.  This is a

number from 0 to n-1.  Normally after initializing memory,

function 27 would be called for each handle to set the physical

tape drive associated with each handle.

'cnt' is the number of paragraphs (1 paragraph = 16 bytes) of

memory to be reserved.  All additional memory will be allocated

for tape I/O buffers.  The routine will return the number of

paragraphs of memory allocated for I/O buffers in 'cnt'.  If this

function is not called before calling a read or write function,

then all available memory will be allocated for buffers.

The value returned is the TOTAL number of paragraphs allocated.

The program should especially examine this to see if it is large

enough when allocating more than one handle.

33 - Rewind & wait for completion.  cetape(h,33)

34 - Forward space to logical end of tape.  cetape(h,34)

This starts from the current location on the tape, finds two

consecutive tape marks, and positions between them.  If the

current location on the tape is not certain, then a rewind should

be issued before this function.

35 - Write a short block without buffering.  cetape(h,35,&cnt,&buffer)

This writes up to a 500 byte block directly to tape.  This

routine may be used while processing the tape end of volume

without disturbing data already sent to the buffer but not

written yet.

36 - Read a short block without buffering.  cetape(h,36,&cnt,&buffer)

This will read up to a 500 byte block directly from tape.  This

routine may be used while processing the tape end of volume

without disturbing data already sent to the buffer but not

written yet.

37 - Return read/write block count.  cetape(h,37,&blkcnt)

The value returned is a 'long' integer (32 bits).  When a read

or write sequence is started, the module resets its internal

block count.  When a block is actually written out to tape or

read in from tape, this count is incremented.  Retrieving the

value with this call does NOT reset the count.  For example, if

a labeled tape is being written this call may be used from within

the 'tapeov' routine to determine the block count which must be

placed in the EOV trailer label.  It might also be accessed after

terminating an output file with the write tape mark call to

determine the number of physical blocks written to tape.  The

'direct' read and write functions (35, 36) do NOT affect this

count.

cnt = The count appropriate to the routine.  On write operations this is

the number of bytes to be written.  On reads the size (or maximum

size) expected should be placed in this field prior to the call and

the actual size is returned here.

buffer = The data area for read and write operations.

blksize = The block size for blocked reads & writes.  When writing this

is the size block which will be created.  When reading this is

the maximum size block expected.  For best use of memory space

this should not be set any larger than necessary.  This value

may not be altered durring a read or write sequence.  The

routine captures it from the first read or write, then ignores

it for the remainder of the sequence.

Variable length blocked read and write refer to "IBM" variable length

blocked format.  This has a four byte binary block length field at the

beginning of each block plus a four byte binary record length field at the

beginning of each record.  The first two bytes of a length field contain

the binary length of the block/record including the four bytes of the

length field with the most significant byte first.  The second two bytes

are always binary zero.

B.  Tape end of volume while writing.

The C program must define a subroutine 'int tapeov(h)'.  This routine

will be called if a write operation raises the EOV signal.  It should

be assumed that when this routine is called, one or more blocks of data

that are to be written to tape are still buffered in memory.  This routine

may  write tape marks, issue rewinds, etc.  In fact it may issue any

of the functions except the buffered reads or writes (functions 1 - 5, and

28, 29).  Functions 35 & 36 may used to handle volume, header, and trailer

records.  If this routine returns a zero (0), the buffered write which was

in progress will continue.  If a non zero value is returned, the buffered

write will be aborted.  The parameter passed to this routine is an integer

which contains the value of the handle which raised the EOV signal.

C.  Comments.

When reading or writing with these routines, the user should be aware

that the physical position of the tape does not correspond to the last

tape record read or written.  When reading or writting, issuing any

other command will terminate the sequence.   If writing, then this

causes all data in memory to be written out.  If reading, then any data

in memory that has not been retrieved will be lost.

After writing the following bits must be checked:

15 - Online.

12 - File protect.

3 - Hard error.

After reading the following bits must be checked:

15 - Online

4 - Data overrun (0 = overrun)

3 - Hard error.

1 - File mark (no data is returned)

Note that an 'include' file, tapedef.h, has been provided which defines

tests for all of the status bits and also defines macros for performing

some of the common functions.

Field conversion routines (NUMCON/NUMCONL).

Calling format for numeric conversions.

numcon(t1,l1,&f1,t2,l2,&f2)

unsigned char t1,t2;

int l1,l2;

f1 & f2 depending on the type field

t1 = Type of input field (see field types below)

l1 = Length of the input field in bytes.

f1 = Input data field

t2 = Type of output field (see field types below)

l2 = Length of the output field in bytes.

f2 = Output data field

Field types:

Z = EBCDIC zoned decimal.

L = Ascii numeric character string with leading sign (blank/-)

T = Ascii numeric character string with trailing sign (blank/-)

P = Packed decimal

C = Ascii numeric character string with trailing 'CR' for negative

G = Ascii numeric character string with trailing 'DB' for negative

B = Binary with most significant byte first

R = Binary with least significant byte first

Conversions are limited to values not exceeding 15 decimal digits.

The function returns an integer value as follows:

0 = Good conversion.

1 = Invalid input field length

2 = Invalid output field length

3 = Invalid input field type

4 = Invalid output field type

Ebcdic to Ascii conversion.

ebtoas(len,&inbuffer,&outbuffer)

int len;

Ascii to Ebcdic conversion.

astoeb(len,&inbuffer,&outbuffer)

The contents of 'inbuffer' is not changed.  The data is translated and placed

in 'outbuffer'.

Both of the character conversion routines use internal translation tables.

These tables may be edited in the final .EXE program using the INSUTIL program.

Memory models.

A.  Tape access module CETAPE.

There are two versions of this program.

CETAPE.OBJ  - For linking with small memory model C programs.

CETAPEL.OBJ - For linking with large memroy model C programs.

B.  Conversion module NUMCON/NUMCONL.

NUMCON.OBJ  - For linking with small memory model C programs.

NUMCONL.OBJ - For linking with large memory model C programs.