Using ZOOMSYSTo start, hit Reset or power up.  When CPM's (or RIO's) prompt (on the Z80)appears, do the following:(On RIO, substitute % for A> below.)A>SYS  (on RIO, this can and is recommended to be in OS.INIT, in which case it	would not be done here)  --  This downloads the Z8000 system.A>G  --  Starts up and transfers control to the Z8000 system.:  --  (Prompt on Z8000)	At this point, any filename that is typed will be loaded and run (if	it is of appropriate format), the same as on the Z80.  In addition,	multiple names may be separated by ";"s or ","s (in the latter case,	the preceding file is only loaded, not run).  In addition, typing "X"	causes the last loaded file to be executed, and "X n.nnnn" (where	"n.nnnn" is an address in hex, the first "n" being the segment number)	causes execution to begin at that address.  There can also be	input-output redirection (see below) and disk initialization (by	typing "I").:EE FILE1  --  Example of loading editor to edit FILE1.:Y FILE2   --  Example of loading Y and having it compile FILE2.://  --  Return to Z80.>A	(The Z8000 may be reentered by typing "G" again.)****************************************************************************			     Miscellaneous  1. Hitting 'ctl-C' causes a break on the Z8000.  Hitting 'ctl-S' stopsconsole output; it can be started up again by hitting any key.  2. If no other break handler is present, the system has a small monitor whichis entered upon a break.  This monitor has the following commands:(Below, "n.nnnn" refers to an address in hex.  The "n." is the segment number;if left off, it defaults to the segment of the PC prior to the break.  Also,the commands may be typed in either capital or small letters.)D n.nnnn,mm ... display mm lines of memory starting at address n.nnnn .  ",mm"		may be left off, with mm defaulting to 1.  If mm=0 or there is		a "," with nothing following, one line of memory is displayed		which may then be changed a word at a time by typing in the		new value for successive words in hex, each value separated		by a space from the preceding one, and hitting return when		finished.R ............. display memory where the registers prior to the break have		been saved.  The format is R0..R7 on line 1, R8..R15 on line 2,		and N4, FC, and PC on line 3.G ............. resume execution where left off (or at current value of the		stored PC)J n.nnnn ...... jump to address n.nnnn and resume executionSH ............ restart system and reenter shell, first closing files.  This is		a good way to get back to normal command mode after breaking		(with cntl-C) from a program.PR ............ quit to Z80 (closes files and shuts off timer) and jump to		boot prom in preparation for reloading SYS (Z8000 system) from		Z80 (not generally needed)  3. In order for a file to be loadable and runable on the Z8000, it must bean object file in the format described below, with header and segment tablesections at the beginning.  (The relocation information and symbol tablesections are not needed.)  The segment table specifies where the file is to beloaded; the entry_pt is specified in the header.  Exception: if the file ismarked as runnable in any Z8000 segment, the segment number given in theentry_pt is used for loading the file.     Programs are called as subroutines from the system, and are started upin segmented mode, with register RR12 and also the stack above the returnaddress containing a pointer to the rest of the command line (after thefilename) which invoked the program.  This can contain arguments to be passedto the program and is terminated by a carriage return (hex 0D).  (If additionalcommands follow on the command line, a carriage return is inserted at theend of the first program's argument list.)     As noted above, programs which are started up by the system must begin insegmented mode.  If they are to run nonsegmented at any time, they must firstset up a stack in the segment they are running in, possibly saving the originalstack pointer to switch back to before returning (also done in segmented mode).As an alternative to ending with a RET, the program may execute an EXIT systemcall (see below) or jump to the system reentry point at 0.800.  4. A 'shell script' file can also be executed.  This type of file containsa list of commands that would otherwise be typed in from the console.  Up to9 arguments provided upon invocation of the shell script can be referencedinside (referenced by '$n', n=1..9).  There can also be comments (indicated by'#' at the beginning of a line, and continuing to the end of the line).  NOTE: There MUST be a comment (or a single '#') on the first line of thefile. The system uses this to recognize the file as a shell script.  See this example:CMDFILE contains:#COPY $1.S $2; EE $2#this is a commentY $2Now, if the following is typed::CMDFILE ABC XYZthe following sequence of commands will be executed:COPY ABC.S XYZ; EE XYZ; Y XYZ  5. Input-Output Redirection:Whenever a program is started up by ZOOMSYS, 3 logical units are pre-openedand normally set to the console.  Logical unit 0 is considered standard input,and logical unit 1 is considered standard output.  These may both be redirectedto/from files or other devices instead of the console.  Logical unit 2 is usedas a convention for error or other special i-o for which it is desired alwaysto be connected to the console.To cause a program to start up with either the standard input or standardoutput or both redirected to a file or another device, the following syntaxis used on the command line:	:PROGNAME ARG1 ARG2 ..ARGN <FILEIN >FILEOUTThis opens FILEIN for input, first closing logical unit 0.  Since logical unit0 is then free, FILEIN gets logical unit 0 and becomes the standard input.In the same way, FILEOUT is opened for output after closing logical unit 1and thus gets logical unit 1 and becomes the standard output.  In the abovesyntax example, the arguments ARG1..ARGN are of course optional; the spacesbefore '<' and '>' are also optional.  If it is desired to only redirectstandard input, '>FILEOUT' would be left off (and vice versa).  6. When a filename has been entered, without a drive qualification, on thecommand line for the system to load, the system looks for it first on drive A(or 0), then on the currently-set default drive.-----------------------------------------------------------------------------			Object file formatHEADER:	word MAGICNO=0E007; long IMGSZ,UDATASZ; word SEGTABSZ; long ENTRY_PT	word HDRFLG,RELOCSZ,SYMTABSZ,UNUSED    where	MAGICNO=magic number	IMGSZ=image size	UDATASZ=size of uninitialized data	SEGTABSZ=size of following SEGTAB (segment table)	ENTRY_PT=entry point	HDRFLG=flag:	   bit 0=1 =>reloc. info + symtab stripped (execution addr fixed)	   bit 1=1 =>stripped plus may run in any Z8000 segment (uses segment			of ENTRY_PT)	RELOCSZ=size of relocation information	SYMTABSZ=size of symbol tableSEGTAB:	...segment table	...each entry=description of 1 code or uninit'd data region	long START_ADDR; word LENGTH,TYPE  ...TYPE=0 =>code, 1 =>uninit'd data	repeat entries until SEGTABSZ  (max 12 entries)IMAGE:  <-------------------> (length=IMGSZ=total of code lengths in SEGTAB)RELOC:	...relocation information	long ADDR; word FLAG	      bit FLAG,15=0 =>local symbol: bit 0=0 =>code ref, 1=>udata ref	   		 =1 =>external symbol: bit 12=1 =>initialization ref	   				       bits 0-11 = SYMTAB ordinal	repeat entries until RELOCSZSYMTAB:	...symbol table	long ADDR; byte TYPE; byte NAME[9]	   TYPE=0 =>undefined, 1=>absolute, 2=>proc, 3=>init'd data,			4=uninit'd data	   if name length>9, 1st byte=length|080, name follows, 0 padded		to whole entry	repeat entries until SYMTABSZ****************************************************************************			ZOOMSYS System Calls(Note: The following is valid for ZOOMSYS Version 1.00 of 11/17/82.)Unless otherwise stated, registers >=R6 are preserved through a system call.-------------------------------------------------------------------------		    General I/O -  (System Call 0)-------------------------------------------------------------------------Call with registers set as follows:  RL0: =command	0=OPEN	1=CLOSE	2=READ	3=WRITE  RH0: =open type if OPEN		 0=input (i.e. file must exist)		 1=output (i.e. create file)	Else it is the logical unit number (returned by OPEN)  R1:  =length of data to be transferred (may be adjusted; see below)  RR2: =pointer to memory where data is to be transferred 	(if R2=0FFFF, segment of caller will be used)  RR4: =On OPEN, points to a string containing a DEFT 'FILENAME' (possibly	qualified with the drive number); else not used	(if R4=0FFFF, segment of caller will be used)	Notes:	  String may also be in form 'string'+0	  DEFT 'string'  means a length byte (not counting itself) comes firstOn return:  RL0: =a completion code (=080 if successful, bit 6 set =>error (see list of	error codes below))  RH0: after OPEN, =logical unit number to use for subsequent operations  R1:  on READ or WRITE, =length read or written (may be less than requested			  amount if end-of-file reached, disk full, error, etc;			  can also be different for console read, as noted			  below)  Registers >=R6 are preserved.Notes:    Special Cases on READ from console:	    if requested length=0, console status will be tested.  If a		character is ready, RL0 will be set to 080 and the character		will be read in (note that you must have a space for it);		otherwise, RL0 will be set to 0.	    if requested length=1, 1 character will be returned as soon as		available.	    if requested length n>1, a maximum of n characters or up to a		carriage return will be returned, with input editing done on		the

line entered; also a control-D will terminate input at any		point.    On WRITE to console, currently CR's are echoed as CR/LF and LF's are    ignored.    Devices may also be opened and closed, although this is not usually    necessary since logical units 0, 1, and 2 are always pre-opened when a    program is started up, usually set to the console.  To open a device,    give its name which must be a name the system knows about.  (The name of    the console is "CON".)Example -- The following sequence will open a file, read it into a buffer, andclose the file:	LDL RR4,#0FFFF,FILNAM  ...FILNAM = address where have drive/filename	R1:=0; RH0:=0; RL0:=0; SC 0  ...open for input	LDL RR2,#0FFFF,BUFF	R1:=BUFLEN; RL0:=2; SC 0  ...read max. of BUFLEN bytes into BUFF	R1:=0; RL0:=1; SC 0  ...closeFurther note:    The library i-o functions described below (System Calls 040-047) may be    used instead of this system call and are generally easier to use.I-O Completion Codes:	(bit 6 set implies an error)    0:   no character ready on console read with request length 0    080: successful operation    0C0: general (unspecified) error    0C1: invalid request    0C2: device not ready    0C7: file not found    0C9: end of file reached before requested length was satisfied    0CB: file not open    0CF: too many files open (maximum 16 logical units; this also includes				those set to the console or other devices)    0D2: wrong file type or format    In addition, there may be other codes returned from the drivers or    operating system on the Z80.-----------------------------------------------------------------------------		 Set Periodic Interrupts - (System Call 1)-----------------------------------------------------------------------------Can vary rate of periodic interrupts of Z8000 and in addition set up for aspecified routine to be called on every interrupt.Call with registers set as follows:  RL0 =1 to 5 - set rate of periodic interrupts to approximately			default*(2**n-1)  where n=RL0  RL0 =0      - set rate of periodic interrupts to default and disable calls to		interrupt routine  RR2 =address of interrupt routine (if R2=0FFFF, caller's segment used)	if R3 is odd, then no interrupt routine is to be calledRegisters >=R6 are preserved.The interrupt routine should be written in segmented mode, preserve allregisters, and end with an IRET.(The default rate for periodic interrupts is on the order of 50/second butdepends on the particular hardware and system software.  It may also varysomewhat during i-o.  See the Get Interrupt Count system call below.)------------------------------------------------------------------------------		Activate/Deactivate Device (System Call 2)------------------------------------------------------------------------------(Devices must be able to handle OPEN, CLOSE, READ, and WRITE as described aboveunder the General I-O System Call (except OPEN is passed the logical unitnumber).  Exact registers passed at present slightly different: i.e., RR2=addr,R13=length, RL1=command, RH1=logical unit.)Call with registers set as follows:  RL0: =0FF if want to deactivate device; anything else if activate  RR2: =entry address of device driver  RR4: =pointer to DEFT 'DEVICE_NAME' or 'DEVICE_NAME'+0  (device name limited	  to <=11 characters)If R2 or R4=0FFFF, segment of caller will be used.Returns:  RL0=080 if succeed; =0C0 if not found/no room.------------------------------------------------------------------------------			Seek (System Call 5)------------------------------------------------------------------------------This call sets the read/write pointer (position for next read or write) for afile.Call with registers set as follows:  RL3: =logical unit number of file (as returned from OPEN)  RR4: =position or relative position in file (depending on R1) to set	read/write pointer to  R1: if =0, RR4=absolute position in file (in bytes)      if =1, RR4=relative offset to be added to current position      if =2, RR4=offset relative to end of fileReturns:  RR2=updated value of read/write pointer or -1 if error (logical unit not      open; RL5=code)------------------------------------------------------------------------------		      File Statistics (System Call 7)------------------------------------------------------------------------------Get file statistics of an open file, currently:		file type=regular file/device		file lengthCall with registers set as follows:  RL3: =logical unit number of file (as returned from OPEN)  RR4: =address of where to put the following 6-byte structure (if R4=0FFFF,	segment of caller is used):		word FTYPE  ...0=file, 1=device		long FLEN   ...file length in bytesReturns:  R3=0 if all okay or -1 if error (logical unit not open; RL5=code)------------------------------------------------------------------------------		     Get Interrupt Count (System Call 9)------------------------------------------------------------------------------Returns RR2=count of periodic interrupts which have occurred since systemstartup.This value can be used for timing (possibly with calibration) or randomnumbers.  (Note that the exact interval of periodic interrupts may varysomewhat during i-o.)------------------------------------------------------------------------------			  Exit (System Call 0D)------------------------------------------------------------------------------Closes all open files and does system reentry, restoring default standard i-odefinitions.  May be called to exit a program without restoring originalstack pointer and segmentation mode.This call doesn't return.------------------------------------------------------------------------------			Restart (System Call 0E)------------------------------------------------------------------------------Closes all open files, shuts off timer, and does system restart, simulatingstartup from Z80.  (This includes reinitializing a number of things.)This call doesn't return.------------------------------------------------------------------------------			  Quit (System Call 0F)------------------------------------------------------------------------------Return control to Z80, first closing files and shutting off timer.  Then goto either bootstrap in prom or infinite loop to await reactivation by Z80.If called with RL0=0, go to bootstrap to await system reboot.If RL0=1, go to infinite loop to await startup interrupt.This call doesn't return.-----------------------------------------------------------------------------		     Request Memory (System Call 010)-----------------------------------------------------------------------------Request a memory buffer.Call with R3=number of bytes requesting.Returns with RR2 at start of buffer, or R3=-1 if one big enough not available.-----------------------------------------------------------------------------		     Release Memory (System Call 011)-----------------------------------------------------------------------------Release a memory buffer.Call with RR2 pointing to start of buffer to be released (as returned fromRequest Memory system call).-----------------------------------------------------------------------------		    Read Break Routine (System Call 012)-----------------------------------------------------------------------------If R5=0, returns address of current break-handler in RR2 (will be odd ifsystem is taking breaks).If R5=4, returns address of current handler of system calls of value >=080(odd if none).-----------------------------------------------------------------------------		    Write Break Routine (System Call 013)-----------------------------------------------------------------------------If R5=0, the address in RR2 will be made the destination for breaks (if odd,system will again handle breaks).If R5=4, the address in RR2 will be made the handler of system calls ofvalue >=080 (if odd, then none).-----------------------------------------------------------------------------		Change Buffer Pool Location (System Call 016)-----------------------------------------------------------------------------Change location in Z8000 memory of system buffer pool (used with the RequestMemory and Release Memory System Calls above, as well as for system i-ofunctions).  The change takes effect upon the next system restart (either bybeing started up by the Z80 or by the Restart System Call).Call with registers set as follows:  RR2: =address of new buffer pool location  R5:  =size of new buffer pool, or, if =0, says want location and size of	current buffer pool returned insteadActually, the low address and high address of the new buffer pool area willbe modified to go on mod-16 boundaries within the region specified by RR2 andR5; also, if the specified region sits over top of part of the system, thelow address will be moved up so it doesn't.  In addition, it is required thatthe buffer pool be of a certain minimum size, around 1K bytes.Returns:  RR2=address of new buffer pool (current one if initial R5=0) or R3=-1 if      specified and (possibly) adjusted region too small  R5 =size of new buffer pool (current one if initial R5=0)Note that system disk i-o performance will degrade fairly significantly ifthe buffer pool size is made too small.-----------------------------------------------------------------------------		Library Functions (System Calls 040-058)-----------------------------------------------------------------------------SC 040 = OPEN:	On entry: R3=pointer to DEFT 'FILENAME' (i.e. length byte first) or		     'FILENAME'+0  (currently assumes in caller's segment),		  R5=open type (0=file must exist, 1=create file)	Returns:  R3=logical unit number for subsequent calls;

if error, R3=-1, RL5=error code	Preserves registers>=R6.SC 041 = CLOSE:	On entry: R3=logical unit	Returns:  R3=0 if successful, else R3=-1, RL5=error code	Preserves registers>=R6.SC 042 = READ:	On entry: R3=logical unit, RR4=data transfer address (if R4=0FFFF,		  segment of caller is used (currently also if called from		  nonsegmented mode)), R1=transfer length	Returns:  R3=length read; if end-of-file, R3=0;		  if error, R3=-1, RL5=error code	Preserves registers>=R6.	Also note special cases when reading from console:	  if request length 0, a character is returned if and only if one ready	    (the returned length indicates whether one was or not);	  if request length 1, 1 character is returned as soon as available;	  if request a length n>1, a maximum of n characters or up to a	    carriage return is returned, with input editing being done on the	    line entered; also a control-D will terminate input at any point	    (the control-D is not passed on); in any case, the actual number	    of characters read is returnedSC 043 = WRITE:	On entry: R3=logical unit, RR4=data transfer address (if R4=0FFFF,		  segment of caller is used (currently also if called from		  nonsegmented mode)), R1=transfer length	Returns:  R3=length written; if error, R3=-1, RL5=error code	Preserves registers>=R6.SC 044 = GETCHAR:	Returns:  R3=next character from standard input (-1 if end-of-file)	Preserves other registers.SC 045 = PUTCHAR:	On entry: RL3=a character which will be printed on standard output	Preserves all registers.SC 046 & 047 = PRINTF:	Formatted printing which preserves all registers.	(Used by "Y")  Example:	printf("string",arg1,arg2,etc.)	  ...puts out string; in string: "\n"=CR, "\t"=tab,	  ...if "%" in string, next character says format for next argument in	  ... list of arguments (use "%%" for "%"):		"%w": put out word in hex		"%b": put out byte in hex		"%c": put out ascii character		"%s": put out ascii string (DEFT pointed to)		"%d": put out word in decimal		"%f": put out floating-point number (uses 2 arguments=high and			low words of 32-bit floating-point number)	      (above small letters may be capital)	Specific example: printf("2+2=%w\n",2+2)	Note: floating-point format=ffffffee where ffffff=fraction part with		high-order bit=sign (1=neg) and ee=exponent (excess 128);		0=all 0	Code generated for printf:		SC 046  ...saves registers		expression to load arg1		PUSH arg1		expression to load arg2		PUSH arg2		...etc		PUSH size of above pushes		SC 047  ...does printing and restores registers		DEFT "string"  ...at return address of SC 047...............................................................................SCs 048-058 are floating point functions:  Argument(s) are in RR2 and RR4 (if 2 arguments), results are returned in RR2.  Registers R6 and above are preserved.  Floating-point format=ffffffee where ffffff=fraction part with high-order	bit=sign (1=neg) and ee=exponent (excess 128); 0=all 0SC 048 = Floating ADD:	RR2:=RR2+RR4SC 049 = Floating SUBTRACT:	RR2:=RR2-RR4SC 04A = Floating MULTIPLY:	RR2:=RR2+RR4SC 04B = Floating DIVIDE:	RR2:=RR2*RR4SC 04C = Floating COMPARE:	if RR2<RR4, RR2:=-1	if RR2=RR4, RR2:=0	if RR2>RR4, RR2:=1SC 04D = FLOAT:	Entered with RR2=integer; return RR2=equivalent floating-point number.SC 04E = FIX:	Entered with RR2=floating-point number; return RR2=integer (truncated).SC 04F = SQUARE ROOT:	RR2:=Sqrt(RR2)SC 050 = Ascii string-to-float conversion:	Enter with RR2 pointing to ascii string (either in DEFT (length byte	  first) form or in form "string"+0), e.g. "2.36E-4"	  (if R2=0FFFF, uses segment of caller)	Returns RR2=number in floating-point formSC 051 = Float-to-ascii string conversion:	Enter with RR2=floating-point number, RR4 pointing to buffer to put	  string (necessary length=14)	  (R4=0FFFF implies use caller's segment)	Puts ascii string representing number in buffer in form "string"+0SC 052 = SIN:	RR2:=sin(RR2)SC 053 = COS:	RR2:=cos(RR2)SC 054 = TAN:	RR2:=tan(RR2)SC 055 = ARCTAN:	RR2:=arctan(RR2)SC 056 = LOG:	RR2:=log(RR2)  (base e)SC 057 = EXP:	RR2:=exp(RR2)  (e raised to a power)SC 058 = POWER:	RR2:=RR2**RR4  (RR2 raised to the power RR4)�p(RR2)  (e raised to a power)SC 058 = POWER:	RR2:=RR2**RR4  (RR2 raised to the power RR4)���������#