IBM Corporation

IBM Enhanced 5250 Emulation Program Version 2.40

April 1994

==========================================================================

This file contains information that is not in the User's Guide about the

following topics:

Automatically restarting a printer session...................Page 1

Tool for displaying session configuration....................Page 1

New tools for isolating problems.............................Page 2

Considerations for landscape printing........................Page 3

Sample BASIC programs........................................Page 3

Command line parameters......................................Page 4

Switching between keyboard layers............................Page 4

Code page mappings...........................................Page 5

French keyboard profiles.....................................Page 5

==========================================================================

AUTOMATICALLY RESTARTING A PRINTER SESSION

------------------------------------------

After you clear an error on your PC-attached printer,

the Emulation Program automatically restarts your printer session.

For example, when there is a paper jam on a PC-attached printer, the

Emulation Program reports to the host that the printer encountered

an error, and printing stops.  After the paper jam is cleared and

the printer is ready again, the Emulation Program automatically

senses that the printer is ready and restarts your host printer

session job.

NOTE:  To make the restart totally automatic, you might need to set

an automatic reply on the host for some of the common printer error

messages.  On the AS/400, use the WRKRPYLE command to set automatic

replies to error messages.

==========================================================================

TOOL FOR DISPLAYING SESSION CONFIGURATION

-----------------------------------------

A program named CFGINFO.EXE is now supplied that allows you to easily

see the information in your DP5250.DAT file.  At the DOS prompt, enter:

CFGINFO

General information about your session configuration is displayed on

the first screen.  To see the second screen, which contains specific

information about the sessions, press any key.  To print the information

on one sheet of paper, print each screen by pressing the print screen

key.

IBM Enhanced 5250 Emulation Program Version 2.40                   Page 2

==========================================================================

(Tool for Displaying Session Configuration, Continued)

To save the information in a file instead of displaying it, use DOS

redirection.  For example, enter the following at the DOS prompt:

CFGINFO > CFGINFO.TXT

The information is saved in the current directory in a file that is

named CFGINFO.TXT.

If you need customer assistance, please run this program and have

your configuration information handy prior to calling customer

support at 1-800-237-5511.

==========================================================================

NEW TOOLS FOR ISOLATING PROBLEMS

--------------------------------

The Enhanced 5250 Emulation Program Version 2.40 uses external programs

to check and to enable your adapter.  There is a program for each

adapter type.  These programs:

- Verify that your adapter is installed

- List the interrupt number that the adapter is using

- List the memory address that the adapter is using

- List the I/O port address that the adapter is using

When you run DP5250, these programs are called automatically.

To isolate problems, you can run the program for your adapter from

the DOS prompt:

- If you have an Enhanced 5250 Emulation Adapter

(ISA/EISA bus), run the AT.EXE program.  The AT.EXE

program returns a default interrupt value of 5.  To

see the actual interrupt setting, run HWCONFIG.

- If you have a 5250 Emulation Adapter/A (micro-channel

bus), run the UCHAN.EXE program.

- If you have a 5250 Emulation PCMCIA Adapter, run the

ENB5250.EXE program.  (ENB5250.EXE was installed from

the Enabler/Diagnostic diskette.)

After you verify your adapter, run ENHDENB.EXE.  This program:

- Checks your line to determine system availability

- Checks your adapter cable

IBM Enhanced 5250 Emulation Program Version 2.40                   Page 3

==========================================================================

(New Tools for Isolating Problems, Continued)

- Checks for other devices on the line are using the

same address that your adapter is using

- Verifies that your ENH5250.DAT file (created by the

HWCONFIG.EXE program) is correct

If there are no errors, ENHDENB clears the screen and returns you to a

DOS prompt.  If there is an error, a message remains displayed on the

screen.

CONSIDERATIONS FOR LANDSCAPE PRINTING

-------------------------------------

Although the Emulation Program does not support landscape printing, you

might be able to print in landscape mode using the HPLASER2.PFT and

PCL5.PFT printer function tables.

To create a landscape option, change the commands for the Bottom Tray

Select and Feed as follows:

(from)  1B 26 6C 34 48 1B 26 6C 30 4F

(to)    1B 26 6C 34 48 1B 26 6C 31 4F

When you want to print in landscape mode, select drawer 2 from your host

application.

This suggestion might not work in all situations.

==========================================================================

SAMPLE BASIC PROGRAMS

---------------------

For computers that do not have the BASIC interpreter program, compiled

versions of the following sample BASIC programs are included on the

diskette:

UL.EXE

SS.EXE

SS2.EXE

NOTE:  The automatic signon program ASO.BAS is not supplied in a

compiled version, because logon-specific information must be compiled

into the program.

IBM Enhanced 5250 Emulation Program Version 2.40                   Page 4

==========================================================================

COMMAND LINE PARAMETERS

-----------------------

There are two new command line parameters, Q and R.  Usually, you do not

need to use either of these options.

Option Q   Applies to personal computers that allow interrupt sharing,

for example an IBM Model 95, which is a Micro Channel

architecture computer.  The Emulation Program defaults to no

interrupt sharing.  To turn interrupt sharing on, enter the

following command at a DOS prompt: DP5250 Q

NOTE: If you are using OS/2 2.1, you cannot use interrupt

sharing.

Option R   Versions prior to Enhanced 5250 Emulation Program Version

2.40 told the host that it had a 5250-typewriter-style

keyboard.  Version 2.40 tells the host that it has an

enhanced style keyboard, and the host expects to receive

enhanced keyboard scan codes.

Any programs that send scan codes to the host using the KEYO

interface of the Emulation Program send the old scan codes.

The Emulation Program automatically translates these old scan

codes to the appropriate enhanced scan codes.

If your application program can send the enhanced scan codes,

turn the translation off.  To turn the translation off, enter

the letter R on the command line when starting DP5250.

For information about the KEYO interface, see the IBM

Enhanced 5250 Emulation Program Version 2.3 Technical

Reference manual.

Options P and X are no longer valid.

==========================================================================

SWITCHING BETWEEN KEYBOARD LAYERS

---------------------------------

To switch to the base keyboard layer, define a key in the keyboard

profile to equal [baselayer].   For example:  def a-g = [baselayer]

To switch to the Latin keyboard layer, define a key in the keyboard

profile to equal [latinlayer].  For example:  def a-l = [latinlayer]

For example, if you are using the Greek keyboard:

To switch to Greek mode, press ALT+G.

To switch to Latin mode, press ALT+L.

IBM Enhanced 5250 Emulation Program Version 2.40                   Page 5

==========================================================================

CODE PAGE MAPPINGS

------------------

The first time that you run the session configuration program

(CONFIG.EXE), the language selections default to the code page

mappings shown below.  You can change these defaults during session

configuration.  The default code page mappings apply to both display

and printer sessions:

Host       Personal

Code       Computer

Country                 Page       Code Page

-----------------       ----       ----

US                      037   ->   437

Belgian                 500   ->   850

Brazilian               037   ->   850

Canadian French         037   ->   850

Danish                  277   ->   850

Dutch                   037   ->   850

French                  297   ->   850

German                  273   ->   850

Greek                   875   ->   869

Icelandic               871   ->   850

Italian                 280   ->   850

Latin American          284   ->   850

Norwegian               277   ->   850

Portuguese              037   ->   850

Spanish                 284   ->   850

Swedish/Finnish         278   ->   850

Swiss-French            500   ->   850

Swiss-German            500   ->   850

Turkish                 1026  ->   857

UK                      285   ->   850

==========================================================================

FRENCH KEYBOARD PROFILES

------------------------

When you select the FRENCH language option during session configuration

(CONFIG.EXE), the default keyboard profile is the 189 keyboard.  If you

want to use the enhanced French 120 keyboard, specify the following for

the User Defined profile:

FR120--E.PRO