AHA-2940 Family BIOS v1.23 Product Change Notice

Released:	March 18, 1996

Products Affected:	AHA-2940 with Autoterm, AHA-2940 with Rev. D chip, AHA-2940U and 	AHA-2940UW

BIOS Part Numbers:	AHA-2940 With Autoterm 	589214-00 Rev. D  Checksum EC00

AHA-2940D			589219-00 Rev. D  Checksum EC00

AHA-2940U			589217-00 Rev. C  Checksum 8800

AHA-2940UW			589217-00 Rev. C  Checksum 8800

The BIOS on the AHA-2940 family of products has been updated to incorporate the changes and corrections described below.  This affects all standard AHA-2940 products except the non-autoterm AHA-2940 Tray Bulk products (part numbers 591600 and 597000).

Functional Changes:

1.	Implemented New EBDA Usage Guidelines

The usage of EBDA (Extended BIOS Data Area) by BIOS v1.21 caused conflicts with some motherboards and add-in cards.  In particular, some multi-processor systems would hang, and other systems would not boot.  BIOS v1.23 implements new EBDA usage guidelines.  If no bootable CD-ROM is present, BIOS v1.23 does not utilize EBDA at all.  If a bootable CD-ROM is present, BIOS v1.23 carefully retains any previously stored EBDA data before allocating a segment for its own use.  Finally, the Int15 handler no longer traps the C0 function.

2.	Improved BIOS Decompression Algorithm (“Memory Error” On Warm Boot)

On a successful power-up boot, the decompression address is stored.  On a warm boot, this address is reused if BIOS v1.23 can not find enough free memory for decompression.

3.	Bootable CD-ROM Fixes

BIOS v1.22S was using incorrect floppy drive parameters, causing a hang condition.  Also, the Terminate Emulation function was not restoring the correct device ID before leaving, causing inconsistent behavior and system hangs.  BIOS v1.23 fixes both of these problems.

4.	Messaging (“BIOS Not Installed” & “Hit F1 To Continue”)

Changed the message that is output when no bootable SCSI device is found from “Int 13h Device Not Found” to “No SCSI Boot Device Found”.  Also, removed the error code returned when no bootable SCSI device is found.  The error code was causing some motherboards to print the message, “Hit F1 To Continue”.

5.	Changed AHA-2940 rev. D Board ID

BIOS v1.23 changes the AHA-2940 rev. D board ID to be the same as the ID of the AHA-2940 with the AIC-7870 rev. C chip.  Both boards will now be recognized by the same embedded OS drivers.  No software changes are required in the 7800 Family Manager Set.  This change affects the AHA-2940D Tray, the AHA-2940 Single and the AHA-2940 Kit.  Please note that the code on BIOS P/N 589214-00 and 589219-00 are identical, however we have retained the existing part numbers as customers will have already specificed these BIOS part numbers in their incoming goods inspections.

6.	Other Miscellaneous Changes

In BIOS v1.21, SCSI IDs greater than 7 were lost, affecting both the AHA-2940W and the AHA-2940UW boards.  BIOS v1.23 corrects this problem.  BIOS v1.23 also changes SCSISelect options to be more backwards compatible with prior version of SCSISelect.