OPSD BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97214

DATE:	January 6, 1997

FROM:	Intel Applications Support - Oregon

SUBJECT:	AP440FX Standard BIOS Release 6.0

About This Release

o	BIOS 1.00.06.CT1

Features/Errata Fixed In This Release

- Release 6.0

- Added support for French, German, Italian and Spanish in Standard BIOS.

- Fixed problem where Single bit ECC Error will cause system to lock up or WinNT to cause Kernel dump.

- Added support for Pentium® Pro B1 Stepping BIOS Update.

- Added support for S3 VirgeDX® BIOS.

- Release 5.0

- Fixed the issue where LDCM was incorrectly reporting a virus detected when a logo

- is installed.

- Added code to make DRAM timings more relaxed for booting.

- De-allocate the Heceta I/O ports (0x290-0x297) when Heceta  is not on board or is disabled in Setup.

- Fixed the issue of the 'Memory Error Detection  Setup” sometimes showing up on the Advanced Chipset Configuration screen when non-parity SIMMs are installed.

- Fixed the issue where the system would hang during POST if optional ROM switches processor modes and returns the processor back to the real mode, rather than Real Big mode, prior to giving control back to the BIOS. The Int 15h function 87 (Extended memory block  move) is an example that the ROM BIOS may call and which would result in a POST time system hang.

- Changed Floppy Disk Controller IO Port Allocation from 0x3F2, 0x3F4-0x3F5 to 0x3F2-0x3F5 for improved Win95* compliance.

- Added IRDA support to Serial Port 2.

- Corrected the operation of  Setup's "Plug and Play"  screen when using configuration mode  "Use BIOS Setup".  When Serial port 1 and Serial port 2 were both using IRQ 3, then IRQ 4 was NOT available on  the screen. When Serial port 1 and Serial port 2 were both using IRQ 4, then IRQ 3 was NOT available on the screen. The new operation allows either IRQ 4 or IRQ 3 to be shown on the screen if neither Serial port 1 or Serial port 2 is using the respective IRQ.

- Release 4.0

- Preserve all registers other than AX when the Read PentiumÒ Pro BIOS Update function is called.

- Added support for APM 1.2 resume timers.

- Changed APM BIOS postings for Hot key, Front Button and Inactivity timer from SUSPEND requests to STANDBY requests.

- Added Invalid Interrupt Service Routine in SMM.

- Resolved extended delay between ATA/ATAPI  initialization and floppy initialization during  POST.  Delay was four seconds if device(s) were  present on the primary master IDE controller and eight seconds if device(s) were present on primary and secondary master IDE controllers.

- Include Hard Disk Drive interrupts as system activity events.

- Removed code to clear all pending keystrokes whenever the user enters a new keystroke within Setup.

- Fixed a issue where APM Hot keys were ignored  by the APM driver when WINDOWS 95ä was shutdown and restarted in the MS-DOS Mode.

- Fixed an issue where WINDOWS 95ä would power down before it was finished with its initialization.

- Added code to clear any pending keystrokes on entry into Setup and whenever the user enters  a new keystroke within Setup.  This prevents possible keyboard buffer overflow when the key repeat speed is very fast.

- Relaxed checking during password entry in Setup to include all punctuation characters.

- Added bounds checking for the year field of the date entry window so  that the year will fall into the range, 1980-2099.

- Corrected a condition where the power management timer was not being checked for range when the user entered F10.

- Changed the implementation of the ISA Shared  Memory base and size fields on Setup's Advanced  Plug and Play Configuration screen. The choices for the ISA Shared Memory Size and Base address will be limited to options which do not cause the memory to overlap the current video BIOS ROM which will cause a static resource conflict and no video. This has come about because Video Option ROM’s  have grown beyond 32K.

- Added setup enhancement to prevent size of the ISA shared memory area from overlapping video  BIOS.

- IDE Translation Mode updated so that only 'Standard CHS' and 'Extended CHS' are offered if the 'IDE Device Configuration'  option is set to 'User Definable'.  This was done to synch up the Setup code with the drive detection code, which does not allow LBA translations when the drive is set to User Definable.

- Added support for up to 4 choices for serial port 2 IR mode.

- Declare the ECP Extended Registers as taken.

- Added display of Intel Copyright string during POST.

- Added an Invalid Interrupt Service Routine that is installed only while the system is in SMM.

- Enhanced method of ensuring that PCI gets at least 1 IRQ.

- Closed window between CMOS write and CMOS checksum to reduce possibility of a power down during POST causing a CMOS checksum error.

- Added additional DMI support for PCI slot reporting.  The BIOS will now indicate if each PCI slot is 'in-use' or available.

- Fixed issue where a PCI reference card failed.

- Enabled the numeric keypads key "4" for passwords.

- Fixed code to preserve CMOS scratch byte when defaults are loaded so that INT15 DA29 works  correctly.

- Added code to preserve CMOS scratch byte when defaults are loaded.

- Remove speaker enable/disable from Setup since it is not supported.

- Updated AP440FX DMI Information.

- Fix LDCM issues in Memory Upgrade Option, Cache Status, and # of PCI/ISA Slots.

- Included a fix for the PCI Latency Timer.

- Include a Date/Time Stamp in Flash..

- Include Intel Copyright Information.

- Fix VPD Display issue when invalid strings in Flash.

- Release 3.0

- Added support for PentiumÒ Pro BIOS Update INT 15 Interface.  This supports the Intel utility that will detect the stepping of the PentiumÒ Pro processor installed and will update the BIOS with a BIOS Update binary to match.

- Enhanced the DMI Event Logging to include the logging of  POST Device Initializer Manager (DIM) errors.

- Added support for setting the ISA VGA frame buffer region to a new PentiumÒ Pro memory type known as “Write Combine” (WC or USWC).

- Added support for Misty BIOS Recovery.

- Enhanced the PCI Latency Timer programming to allow the intelligent programming of PCI devices.

- Updated the basic Chipset register settings for the 440FX and the PIIX3 to match the latest recommended setting.

- Enabled refresh queue, set multi-transaction timer to 20h, set deturbo mode register to 0D0h.

- Added support for displaying the motherboard ID and serial number at boot time.

- Release 2.0

- Removed initialization of Heceta Head Hardware monitor and SMM logging support. The part is still programmed to use I/O address 290h but any initialization and setup will need to be done through an external driver or LDCM software.

- Removed support for boot Anti Virus check  including SMM logging support.

- Fixed mouse device node issue where if no mouse was installed on the device node 0, which is the PIC, is not assigned any resources.

- Fixed issue where ECC error detection would not function if APM was not enabled in setup. Moved enabling of EXTSMI signal to later in post.

- Fixed setup question for ISA LFB showing possible hole sizes of 2Mb and 4Mb. 1Mb hole should be only option.

- Updated DMI info strings for accuracy.

- Changed Slot 3 on the riser card IRQ routing table information to fix issue found with SCSI cards using this slot.

- Fixed issue with Multi bit errors not being logged or reporting type in NMI.

- Fixed INT15 cache services issue.

- Updated S3 video bios to latest version.

- Changes for WINDOWS 95ä supplement logo acceptance test:

- Made all ports indicate 16 bit decode

- Added non-standard I/O address selections 228 for LPT in ECP, EPP modes.

- Fixed a floppy issue where, when disabled, it returned message that it is consuming IRQ 0.

- Fixed issue with the video having the IRQ routed to INTD of the PIIX3 rather than to INTC where it is connected.

- Ported over to new BIOS code base including the following:

- Graphical setup interface with extended graphical logo support.

- More modular and easier to customize.

- Added setup questions for USB, Video IRQ, and Heceta hardware monitor.

- Added device node for Heceta motherboard device consuming I/O address space 290-297.

- Added boot Anti virus support in BIOS with setup question.

- Added new and more extensive event logging support code including logging for Post errors, ECC, Virus detection, and Heceta hardware monitor system events.

- DMI updates for clearing event log added.

- Fix for CrystalÒ 4232 coming up disabled in WINDOWS 95ä.