�����������������������������������������������������������������

�      PC TO TV CONVERTER PCV1000 Hardward Control Program      �

�                                                               �

�           Version 3.10  (PAL)  for IBM PC/XT/AT               �

�                                                               �

�      Copyright (c) 1992-95  de Vivanco & Co. (GmbH & Co)      �

�                       All Rights Reserved                     �

�                                                               �

�����������������������������������������������������������������

1. INSTALLATION:

In WINDOWS :

1) Insert the PCV1000 disk in drive A:  (or B:).

2) Activate Windows' Program Manager.

3) Run A:\SETUP.EXE (or Run B:\SETUP.EXE).

This  version has  added a Windows icon that  lets you use  PCV1000

control functions(screen positioning and display size toggle) under

under the Windows 3.X environment.

A setup  program is provided for installing the program in Windows.

It will create an PCV1000w  program  group and a  PCV1000w icon. Be

sure to run  PCV1000.EXE  before  entering  Windows to  enable  the

PCV1000w Windows control program.

In DOS :

1) Insert the PCV1000 disk in drive A: (or B:).

2) Change to drive A: (or B:) by entering the A: (or B:) command.

3) Execute INSTALL C:\PCV1000.

This batch file will copy the  related  files to the  C:\PCV1000

subdirectory.

2. Getting HELP:

In DOS :

Enter PCV1000/?

In WINDOWS :

Click the "HELP" button in the PCV1000w windows control program.

3. VGA card support

This version supports the following VGA cards and VGA modes.

CHIP VENDER    CHIP MODEL   CLOCK GENERATOR   DISPLAY MODE

------------   ----------   ---------------   ----------------------------

All suppliers  All kinds    All kinds         Standard VGA mode

Tseng Lab.     ET4000       ICS 2494-237      up to 800 X 600 X 64K colors

up to 640 X 480 X 16M colors

Tseng Lab.     ET4000/w32i  ICS 2494AM        up to 800 X 600 X 64K colors

up to 640 X 480 X 16M colors

Tseng Lab.     ET4000/w32p  ESI 5494-11E      up to 800 X 600 X 64K colors

up to 640 X 480 X 16M colors

Trident        TVGA 8900C   TCK9002           up to 800 X 600 X 64K colors

up to 640 X 480 X 16M colors

Trident        TGUI 9400CXI TCK9002           up to 800 X 600 X 64K colors

up to 640 X 480 X 16M colors

S3             86C801       ICS 2494-305      up to 800 X 600 X 64K colors

up to 640 X 480 X 16M colors

S3             86C805-p                       up to 800 X 600 X 64K colors

up to 640 X 480 X 16M colors

S3             Version 864                    up to 800 X 600 X 16M colors

up to 640 X 480 X 16M colors

S3             Version 964                    up to 800 X 600 X 16M colors

up to 640 X 480 X 16M colors

Cirrus         CL-GD5426                      up to 800 X 600 x 64K colors

up to 640 X 480 X 16M colors

Cirrus         CL-GD5434                      up to 800 X 600 x 64K colors

up to 640 X 480 X 16M colors

Cirrus         CL-GD6410                      up to 800 X 600 x 16  colors

up to 640 X 480 X 16  colors

Paradise       WDC90C33-ZZ  C9064             up to 800 X 600 x 64K colors

up to 640 X 480 X 16M colors

ATI            28800        ATI 18811-0       up to 800 X 600 x 32K colors

up to 640 X 480 X 16M colors

Actix System   ARK 1000                       up to 800 X 600 X 64K colors

up to 640 X 480 X 16M colors

Actix System   ARK 2000                       up to 800 X 600 X 64K colors

up to 640 X 480 X 16M colors

Note:

1) ICS is the abbreviation for Integrated Circuit Systems, Inc.

2) TCK9002 is TRIDENT's  model  number, TCK9002  is  compatible  with

CH9202. CH is the abbreviation for Chrontel Corporate.

3) ATI is the abbreviation for ATI TECHNOLOGIES INC.

4) Any clock generator that is software  compatible with those listed

above is also supported.

4. Operation notes for the supported VGA modes

1) VGA cards in a desktop personal computer

The PCV1000 DOS TSR, PCV1000.EXE, automatically  senses  what type

of VGA card is  currently used, and  then initializes the hardware

accordingly.

PCV1000.EXE supports  all  standard  VGA modes, and  supports  the

extended VGA modes (Resolution  modes beyond 640 X 480 X 16 Colors

are called extended VGA modes) for those VGA cards listed above.

If your VGA card is not included in the  list for the extended VGA

modes, please  contact your  dealer to double check whether or not

it is still being supported.

2) VGA chip in a book-size/laptop personal computer

Usually, a book-size/laptop  personal  computer does  not send VGA

signals to the computer's VGA connector after system power-up. The

computer requires users to do the following  opeartions as follows

in  order to get the VGA signals from the VGA connector.

I. Run a certain utility to switch the LCD display to CRT dislay.

II. Set the CRT display capability through the CMOS setup.

You MUST let  PCV1000 convert your VGA  display  to the TV screen.

Otherwise, you might fail to do the video conversion.

Follow the  steps below to  enable the  PCV1000 for use with  your

book-size/laptop personal computer.

I. Enable the CRT  display  capability  for your  book-size/laptop

personal computer.

II. Run PCV1000.EXE

III. Run your application software.

5. Details about hotkeys and the RESERVE command :

1) Ctrl + Shift + S

Toggle between character font size 8x8 and 9x16 in DOS text mode.

2) Ctrl + Shift + O

Based upon the hardware, this software supports both  overscan and

underscan display in horizontal direction.  Underscan lets all VGA

display  information appears  inside the TV  screen  without image

loss in the  horizontal  direction.  Overscan  fills the TV screen

with  VGA  display  information  in  the  horizontal  direction to

emulate what you typically see on a TV screen.

3) Ctrl + Shift + R : Reinitialize the VGA mode.

This hotkey is  useful for those  DOS and  WINDOWS  programs which

directly  change the  display  registers in the  VGA chip.  When a

program directly sets the  VGA chip, the TSR program, PCV1000.EXE,

can  not  recognize  which  VGA  mode  should  be  changed  for TV

conversion. In this case, the hotkey can remedy this problem.

In  DOS, when  the VGA  registers  are  changed by  some  programs

incidently and the  TV screen is lost,  you can use the hotkey to

correct it.

When the hotkey is invoked under Windows, the  current VGA display

will be reinitialized  for the TV screen.  For example, when using

the "full screen"  function of  Microsoft  Video for  Windows , we

need to reinitialize the VGA display to get the correct TV output.

In most cases, don't use this hotkey because it will reduce the

system performance.

4) Ctrl + Shift + G : Return to the default screen position.

When you move the  screen up, down, left  and right, and the screen

becomes distorted, you can hit this hotkey to let the screen return

to the default position.

5) Execution file RESERVE.EXE

If you have  adjusted the  screen to a  position that you like and

want to keep it as  the default value, you  can  execute the file,

RESERVE.EXE, to save the current position parameter. The next time

you  execute PCV1000.EXE, the  TV screen  will be at the  position

that you saved.

6. Programming Interface of PCV1000.EXE.

The following is a  summary of the services of Interrupt 61h Function

8642h for the PCV1000 control program :

(BX) = 01H - Display Move Left

(BX) = 02H - Display Move Right

(BX) = 03H - Display Move Up

(BX) = 04H - Display Move Down

(BX) = 05H - Toggling Screen Display Underscan Or Overscan

(BX) = 06H - Re_initialize

(BX) = 07H - Disable Re_initialize

(BX) = 08H - Check TSR Exist Or Not ?

(BX) = 09H - Goto Default Position

(BX) = 0AH - Change Font

(BX) = 15H - Reserve Parameter

(BX) = 16H - Get Screen Size(Overscan Or Underscan)

USAGE:(Using assembler code (80x86))

-----------------------------------------------------------------

INT 61h Function 8642h, Subfunction 01h

DISPLAY MOVE LEFT

Register at call:                       On Return:

AX = 8642H                              Display move left

BX =   01H

Details: The limit of "move left" is 8 steps or smaller.

-----------------------------------------------------------------

INT 61h Function 8642h, Subfunction 02h

DISPLAY MOVE RIGHT

Register at call:                       On Return:

AX = 8642H                              Display move right

BX =   02H

Details: The limit of "move right" is 8 steps or smaller.

-----------------------------------------------------------------

INT 61h Function 8642h, Subfunction 03h

DISPLAY MOVE UP

Register at call:                       On Return:

AX = 8642H                              Display move up

BX =   03H

Details: The limit of "move up" is 32 steps or smaller.

-----------------------------------------------------------------

INT 61h Function 8642h, Subfunction 04h

DISPLAY MOVE DOWN

Register at call:                       On Return:

AX = 8642H                              Display move down

BX =   04H

Details: The limit of "move down" is 32 steps or smaller.

-----------------------------------------------------------------

INT 61h Function 8642h, Subfunction 05h

TOGGLING SCREEN DISPLAY UNDERSCAN OR OVERSCAN

Register at call:                       On Return:

AX = 8642H                              Display Underscan or

BX =   05H                              Overscan

Details: The default display is underscan.

-----------------------------------------------------------------

INT 61h Function 8642h, Subfunction 06h

REINITIALIZE

Register at call:                       On Return:

AX = 8642H

BX =   06H

Details: Excuting this function, it will detect the Vertical sync.

automatically, but reducing the performance.

-----------------------------------------------------------------

INT 61h Function 8642h, Subfunction 07h

DISABLE REINITIALIZE

Register at call:                       On Return:

AX = 8642H                              Display Underscan or

BX =   07H                              Overscan

Details: Executing this function, it will be disabled to detect the

Vertical sync.

-----------------------------------------------------------------

INT 61h Function 8642h, Subfunction 08h

CHECK PCV1000.EXE TSR EXIST OR NOT

Register at call:                       On Return:

AX = 8642H                              BX = 4441H if successful

BX =   08H                              CX = 4144H if successful

Others, PCV1000 TSR not exist

-----------------------------------------------------------------

INT 61h Function 8642h, Subfunction 09h

GOTO DEFAULT POSITION

Register at call:                       On Return:

AX = 8642H                              The screen go back to

BX =   09H                              default position

-----------------------------------------------------------------

INT 61h Function 8642h, Subfunction 0Ah

CHANGE FONT

Register at call:                       On Return:

AX = 8642H                              8x8 <-> 9x16 FONTS

BX =   0AH

Details: It will be programmable, just only on the TEXT mode.

-----------------------------------------------------------------

INT 61h Function 8642h, Subfunction 15h

RESERVE PARAMATER

Register at call:                       On Return:

AX = 8642H                              CH = Left or Right offset

BX =   15H                                   value

CL = Up or Down offset

value

DH = Change font or not

DL = Overscan or Underscan

Details: The means of Register return value are :

CH : Default position : 80h

LEFT       : between 78h and 7Fh

RIGHT      : between 81h and 88h

CL : Default position : 80h

DOWN       : between 60h and 7Fh

RIGHT      : between 81h and A0h

DH : 80h = 8x8 font,  81h = 9x16 font

DL : 80h = UNDERSCAN, 81h = OVERSCAN

-----------------------------------------------------------------

INT 61h Function 8642h, Subfunction 16h

GET SCREEN SIZE(OVERSCAN OR UNDERSCAN)

Register at call:                       On Return:

AX = 8642H                              BX = 0, Underscan

BX =   16H                              BX = 1, Overscan

-----------------------------------------------------------------

EXAMPLE :

1. Check if the TSR program, PCV1000.EXE, exists or not.

mov ax,8642h

mov bx,08h

int 61h

cmp bx,4441h

jne not_exist

cmp cx,4144h

jne not_exist

...                   ; the PCV1000.EXE exist

not_exist:                       ; the PCV1000.EXE not exist

2. Display move left.

mov ax,8642h

mov bx,01h

int 61h

7. Miscellaneous :

1) About the Windows RESERVE function :

I. If you execute PCV1000w setup, the path of the PCV1000.EXE file

file will be written to WIN.INI file. Next time you execute the

RESERVE function of PCV1000 under windows, it will look for the

path of PCV1000.EXE from the WIN.INI.

II. Some  VGA modes  will share  the same  parameters  you excuted

RESERVE function.

2) When you change the VGA mode, "60Hz" vertical  scanning  frequency

is recommended. If you apply other frequencies, the display screen

may appear too small or distorted.

3) If your VGA card can not be  detected by  PCV1000.EXE, you can use

any of the VGA chips listed below:

( 0) ET4000/W32I/W32P

( 1) Trident/8900/9400CXi

( 2) Cirrus/6410/5426/5434

( 3) S3  801

( 4) ATI 28800

( 5) WD90C33

( 6) S3 805

( 7) S3 Version 964

( 8) S3 Version 864

( 9) ARK1000

(10) ARK2000

P.S. All trademarks are the properties of their respective holders.