# Chapter 5 - Telephone Answering Machine (TAM) Services

This chapter describes the telephony services available to application developers for the purpose of developing Mwave compatible TAM based applications.

## Mwave TAM Architecture

TAM functionality is provided with two separate MCI device drivers: Phone Line and Message.  The Phone Line driver is used for all operations involving the telephone line.  This includes making a call, answering a call and remotely (i.e. via a telephone call) reviewing or recording a message.  The Message driver is used only for those operations which do not involve use of the phone line (i.e. locally reviewing or recording messages).  This multiple driver approach allows simultaneous telephone answering and message review.  TAM can answer an incoming phone call at the same time a user is reviewing (i.e. listening to) his messages.

#### TAM Programming Environment

For many MCI devices, including TAM, the MCI controls are similar to those of a tape recorder (for example; record, play, stop, pause, and seek).  The MCI command message API specification for TAM applications begins with this conventional design, and adds enhancements such as voice compression and speakerphone operation.

The Phone Line driver can record and play through the telephone line only.  (for local message record and review, the Message driver is used).  The Phone Line driver can additionally be used to connect the phone line to the handset (as with a standard telephone) or the audio port (speaker/microphone).

TAM resembles a media recorder and player which can be connected to various telephony related voice channels.  Each of the telephony related voice channels has an audio input and output driver. All channels "connected" to the media are used for play or record operations. If multiple channels are connected to the media, they are also connected to each other, even if no play or record operation is in progress. The audio channels defined for use with the TAM device drivers are as follows:

·    MCI_TAM_AUDIO (speaker & microphone)	MSG

·    MCI_TAM_HANDSET		 MSG

·    MCI_TAM_PHONELINE		 PL

·    MCI_TAM_AUDIO_PHONELINE	PL

(speakerphone)

·    MCI_TAM_HANDSET_PHONELINE	PL

(standard phone operation)

·    MCI_TAM_SPEAKER_PHONELINE	PL

(answering machine w/ call screening)

Various telephony operations are achieved by configuring or "connecting" the TAM drivers via calls to MCI_SET.  For the Message driver, this includes the following:

·	Connect MCI_TAM_AUDIO to record a new announcement from the microphone or review messages on the speaker.

·	Connect MCI_TAM_HANDSET to record a new announcement or review messages through the local handset or desk telephone.

For the Phone Line driver, this includes the following:

·	Connect MCI_TAM_PHONELINE to play an announcement, record a new message, or review messages from a remote telephone.  This connection is required for all operations involving an outside phoneline.

·	Connect MCI_TAM_AUDIO_PHONELINE for speakerphone operation.  Note that with this operation, the media recorder/player is not available.  No record or playback can be done.  Setting speakerphone operation disables call discrimination based on calling tones.  Also DTMF key detection is also disabled.

·	Connect MCI_TAM_HANDSET_PHONELINE to allow the normal use of the desk phone.

·	Connect MCI_TAM_SPEAKER_PHONELINE to allow the user to screen calls.  Note that the microphone will not be connected.

As suggested above, both drivers can use the audio port (speaker/microphone) and telephone handset, but they cannot share them.  When either of these devices are in use by one of the drivers it is unavailable to the other.  Also, the handset must be available when opening the Phone Line driver and the audio port must be free when opening the Message driver.  Programmers must track the device connection status of the two drivers to avoid device conflict errors.

Once again, the actual operation of the TAM device is similar to a physical answering machine. The programming model incurs some complexity when implemented using a message driven architecture, but its similarities to a tape recorder remain. See the code example at the end of this chapter for more details.

### TAM File Formats

Sound files are notorious for their size. One of the more significant problems with the accumulation of large amounts of audio data is data storage. Although OS/2 MMPM and Microsoft Windows wave files support several different data formats, most MCI devices support only uncompressed PCM.

To efficiently deal with data storage, the TAM API specification allows a device driver to support a device dependent format (custom format tag), which allows the device driver to store data files in the most optimum format available. This removes the burden of data compression from the application writer, and at the same time, allows for increased functionality on the part of the device driver.

Support of the custom format tag is optional under this specification, and the application can still choose to use the standard PCM wave file format. For those device drivers which do not support the standard PCM format for play and record operations, a conversion command is available to convert standard wave files to the custom format used by the device. In general, if a device supports a custom format tag, it is to the advantage of the application (in terms of file storage) to use the custom tag in place of the standard file tag.  The custom format requires about 4K bytes per second of audio. (Note: Standard PCM wave format is supported by version 3.1 and above of the TAM drivers).

The programming example described at the end of this chapter uses the custom format tag and does not need to perform any file format conversions.

### Command Message Summary

The following table lists the MCI commands, most of which are used by the sample application. For more information on the actual command messages, see Chapter 7 of this document.

| MCI Command | Description |
| --- | --- |
| MCI_CLOSE | Close the device driver |
| MCI_CONVERT | Convert between device dependent and device independent files |
| MCI_DIAL | Dial the phone |
| MCI_GETDEVCAPS | Get capabilities of the device |
| MCI_INFO | Get device string identifier |
| MCI_LOAD | Load a voice file for playing |
| MCI_OPEN | Open the device driver |
| MCI_PAUSE | Pause the voice stream play or record |
| MCI_PLAY | Play a voice file |
| MCI_RECORD | Record a voice file |
| MCI_RESUME | Resume a paused voice stream |
| MCI_SAVE | Save a recorded voice stream |
| MCI_SEEK | Change current position of the media |
| MCI_SET | Configure the device |
| MCI_STATUS | Query device configuration |
| MCI_STOP | Stop a voice stream |

Table 5-1:  TAM Driver MCI Command Messages

Programmers familiar with MCI will note the new command messages; MCI_CONVERT and MCI_DIAL. For most TAM related applications, it is not  necessary to make use of the MCI_CONVERT message.  (MCI_CONVERT is supported in TAM drivers version 3.1 and above.)

### Event Message Summary

The following table is a summary of the MCI event messages which may be received from the TAM device driver. Most of these event messages are used in the sample application.  These messages are described in more detail in Chapter 7 of this document.

| Event Message | Description |
| --- | --- |
| PHONE_EVENT_ADVANCED_RING | Advanced format ring notification |
| PHONE_EVENT_CALL_PROGRESS | Call progress state has changed |
| PHONE_EVENT_CALL_TAM | Received an incoming voice telephone call |
| PHONE_EVENT_CALL_TERMINATED | Call has been terminated (supplies termination code ) |
| PHONE_EVENT_CALLER_ID | Caller ID string detected (supplies completion status) |
| PHONE_EVENT_DISTINCTIVE_RING | Distinctive ring detected |
| PHONE_EVENT_HANDSET | Change in handset status (supplies handset status) |
| PHONE_EVENT_HANDSET_KEY | Keypad press from handset (supplies character) |
| PHONE_EVENT_LINE | Change in phone line hook status (supplies status) |
| PHONE_EVENT_LINE_KEY | Keypad press from phone line (supplies character) |
| PHONE_EVENT_RING | Telephone ring status change (supplies ring on/off) |

Table 5-2:  TAM Driver Event Messages0

Event messages are received by the event message handler which is declared to the TAM device driver using MCI_SET. For more information on the event handler, and how it relates to the TAM device driver, refer to Chapter 3 on Telephony Services.

## Developing an Mwave TAM Application

This section describes how to develop an application, which calls the TAM API to access the Mwave TAM device drivers, providing telephone answering capabilities.

### Handset/Speakerphone Interactions

The following describes the interactions and application source required for changing from speakerphone to handset and back again.

Scenario:

Assume the application is connected to speakerphone, a call has come in and the application has gone off hook.

User Lifts Handset:

-App receives PHONE_EVENT_HANDSET with dwHandsetStatus = 1 (off hook)

-App issues set connect to MCI_TAM_HANDSET_PHONELINE

-App issues an onhook

Note:  The application should issue on hook because connecting handset creates 2 extensions on the phone line.  When the application issues the “onhook” the line will only have one extension.  (Disconnecting TAIO from the line allows the handset to act as a normal phone) .  For example, if the application had not done the onhook after connecting to handset then when the user put the phone down the call would still be offhook (because TAIO is still connected).

User Requests Speakerphone:

-App issues off hook (turn on second extension before connecting to that extension)

-App issues set phoneline/audio

User wants to do conversation record:

The app must be in either normal phone or speakerphone mode.

-App issues off hook (if in normal phone mode the app needs to reconnect the computer (TAIO) to the phoneline by issuing and off hook) This will allow the remote side to be recorded.

-App issues Mciphone record (only voi supported for conversation record)

### Sample Application Definition

The first step in developing a solid application, is the definition of its intended functionality. For the purpose of demonstration, the application defined here is modeled after a simple telephone answering machine. Defined functionality includes: playing announcements, recording messages, and reviewing messages from the control panel or a remote telephone. Below is a diagram of how an incoming telephone call is handled.

Figure 5-1:  Answering Machine Model

Once placed in command mode, the touch-tone keypad of the remote telephone can be used for an unlimited number of functions. Using keypad entries and voice menus, the remote telephone may be used for anything from reviewing messages to requesting FAX documents. Keypad commands can consist of one or multiple key entries. A list of the actual keypad commands used in the sample application can be found later in this chapter.

The following sections assumes you are familiar with the operation of the TAM sample application included on the companion diskette. See "Using the TAM Sample Application" on page 1-23 for complete details.

### Sample Code Design

The primary goal of this application example is to illustrate the operation of an event (message) driven TAM device, which is able to execute as a background task.

The TAM device has been defined as a finite state machine, which is mainly driven by the MM_MCINOTIFY message issued on a MCI command completion. To implement the state concept, a single variable is defined:

short wTamState;		\\ State of TAM device

This variable can have any of the following states:

·	TS_IDLE

·	TS_PLAY_ANNOUNCEMENT

·	TS_RECORD_MESSAGE

·	TS_COMMAND_MODE

·	TS_PLAY_MESSAGE

·	TS_REMOTE_PLAY

·	TS_ARCHIVE_PLAY

These states correspond roughly to the diagram shown in the previous section. The 'idle' state, not shown in the diagram, is the state where the application is waiting to answer a call. The 'COMMAND_MODE' state is the state of the TAM device when in message review mode. The state of the TAM device is used to determine the next operation after receipt of successful completion messages in the NOTIFY section of the event handler.

Some of the additional variables used in the program are listed below. Most of these values are user definable, and are read from the application INI file when the program is first executed.

| Global Variables | (* Stored in INI file) |
| --- | --- |
| wActiveMessages | Number of active messages on disk |
| * wCommandCode | digit command code |
| wCurrentMessage | Index of current message being played |
| * wMonitor | Incoming call monitoring (0-Off 1-On) |
| wMsgDate | Current message data in MSDOS format |
| * dwMsgIndex | Index of next message to be saved |
| wMsgTime | Current message time in MSDOS format |
| wNewMessages | New messages since last MSG review flag |
| wPlaySpeed | Message playback speed (0-Slow 1-Norm 2-Fast) |
| * wRingCount1 | Ring count with messages |
| * wRingCount2 | Ring count with out messages |
| wTamState | State of TAM system (See TAM States above) |
| * wVolume | Speaker volume level |
| wExclusive | Set to '1' when app can not answer the telephone |
| wBFE | Error code used by error_box() |

| Global Strings | (* Stored in INI file) |
| --- | --- |
| CurrentTimeStamp | String containing time stamp of current file |
| MsgFileSpec | Full path & filename of current message file |
| * MsgPath | Path to message file storage on system |
| Table 5-3: Selected Global Variables and Strings |  |

Because this example is targeted to show TAM operation and not necessarily efficient file management, the file storage system for the voice messages is defined using a simple prefix and suffix system. Under this system, all active messages contain a single letter prefix ('M'), a seven-digit suffix, and the extension '.TAM'. For example, the first message stored in this system is 'M0000000.TAM'.

Recording and Playing the Announcement - The announcement file is stored as 'ANNOUNCE.TAM' in the message directory. When recording a new announcement, a temporary filename is used until the application user chooses to accept the new announcement.

Recording a New Message - New messages are recorded to a filename consisting of the letter 'M', followed by a seven-digit suffix which is the current value of dwMsgIndex (master message index). The extension '.TAM' is added to the filename, and the master message index is incremented. This master message index is stored in the INI file, and is never reset. This allows for the creation of 10 million unique message filenames.

Playing Active Messages - To play the active messages, all files are searched and those matching the 'M???????.TAM' pattern are sorted alphabetically (also chronologically) and stored in a list. The list can then be 'walked' forwards or backwards.

Erasing a Message - To erase a message, the message file is simply deleted.

The TAM device driver is obviously not dependent on any single file management system, and an application programmer might want to use a more sophisticated system in the implementation of a more complex TAM application.

### TAM State Machine Operation

Most programmers familiar with MCI agree that programming to the MCI interface is not a difficult task. The main 'trick' involved in programming a TAM application is writing the application in an event driven fashion, so that when the system is idle, it consumes minimal processor time.

As mentioned above, the application is implemented as a state machine. The application proceeds from state to state based on messages received by the event handler routine. Although most state changes occur as a result of the MM_MCINOTIFY message, the MM_MCIEVENT message is also of interest. Most of the application logic is executed based on event messages.

#### MCI Event Message Handling

MCI event messages are sent as a direct result of an external telephony event detected by the device driver. All event messages are for notification purposes only, and the application is not required to perform any action to 'handle' these events. The event messages are very useful however for writing event driven applications. The messages that are handled in the TAM application example are listed below:

·	PHONE_EVENT_CALL_TAM - This message initiates the TAM state machine execution. The application starts by playing the announcement. Before the call has come in, the application has already connected to the phone line (via MCI_SET_CONNECT) and loaded the announcement (via MCI_LOAD). (Both these operations are performed by the ExitExclusive() function.) At this point, the application executes an MCI_PLAY call to begin playing the announcement. The MCI_NOTIFY flag is supplied with the PLAY call so that the event handler is notified when the play is complete, and it is time to start incoming message recording. The TAM state (dwTamState) is set to TS_PLAY_ANNOUNCEMENT.

·	PHONE_EVENT_CALL_TERMINATED - Here a call has been terminated because the caller has hung up the telephone. If a message record was in progress, the message is saved. The ExitExclusive() function is called to stop any current operation, connect the driver to the phone line, load the announcement file, and set the TAM state (dwTamState) to TS_IDLE. At this point, the application is ready to answer another call.

·	PHONE_EVENT_CALL_PROGRESS - This message is used to detect a 'hang-up' condition that does not produce a 'CALL_TERMINATED' message. If dialtone is detected, the CallTerminated() is called to perform the same actions produced by a 'CALL_TERMINATED' message.

·	PHONE_EVENT_HANDSET - This message indicates that the handset on the local telephone has been either picked-up, or replaced. Picking up the handset  causes different effects at different times. When the system is IDLE, picking up the handset auto-switches from speakerphone to normal phone mode. When reviewing messages to the speaker, picking up the handset disables the speaker and continues play to the handset. If the handset is picked up while the system is recording a message, the record operation is aborted and the system is placed into normal phone mode (connecting the phone line to the handset).

*** This functionality is not implemented in the sample application!***

·	PHONE_EVENT_LINE_KEY - This message sends the ASCII character of the telephone key which has been pressed on the incoming telephone line. The actions taken on receipt of this key vary according to the current state of the TAM device.

If the TAM state machine is in command mode, this message is the potential gateway into remote review mode. It tracks the keys that have been pressed, and if the correct 3 digit sequence has been entered, the current operation is stopped, the TAM state is set to TS_REMOTE_PLAY, and the remote review announcement is played ("You have messages...").

If the TAM state machine is already in remote review mode when this message is received, the key is interpreted as a new command. Key commands can be used to skip, erase, save, repeat, and control the playback of messages stored on the system.

A second state variable (wRemoteState) is used to track the state of the remote message review.

#### MCI Notification Message Handling

The MCI notification message system (MM_MCINOTIFY) is the standard method for MCI to notify an application that a driver action has been completed. For the purposes of our application, we  need to be notified when a message play or record command has been completed. In most cases, the successful completion of a play or record operation requires the execution of another event, and sometimes advances the state of the TAM state machine.

The types of notification messages possible, as well as how they are treated, is as follows:

·	MCI_NOTIFY_ABORTED - There are cases when a play or record operation will be aborted. The most common being when the user picks-up the telephone handset in order to talk 'live' to the caller. Since aborting an operation is not a normal part of the TAM state machine, this command does not examine or alter the state of the TAM device.  *** This functionality is not implemented in the Mwave TAM driver! ***

·	MCI_NOTIFY_SUCCESSFUL - The successful completion of an event is the only automatic method to drive the state change in the TAM state machine. The application should use the NOTIFY flag only for API calls requiring significant execution time.  These include playing and recording voice files. Below is a state change table based on the completion of a play or record operation.

| Current State | Next State |
| --- | --- |
| TS_PLAY_ANNOUNCEMENT | TS_RECORD_MESSAGE |
| TS_RECORD_MESSAGE | TS_IDLE |
| TS_PLAY_MESSAGE | TS_COMMAND_MODE |
| TS_ARCHIVE_PLAY | TS_COMMAND_MODE |
| TS_COMMAND_MODE | TS_COMMAND_MODE |
| TS_REMOTE_PLAY | TS_REMOTE_PLAY |
| Table 5-4 |  |

For example, when the machine is in the PLAY ANNOUNCEMENT state, the completion message indicates that it is time to start recording an incoming message. The message handler starts the record operation. Because detecting the command mode code digit entry is not handled by the event handler, this routine assumes that the RECORD MESSAGE state always follows completion of the PLAY ANNOUNCEMENT state. Similarly, the completion of the RECORD MESSAGE state is always followed by the IDLE state and call termination.

·	MCI_NOTIFY_SUPERSEDED - This message should not occur under normal operating conditions since all play and record operations using the NOTIFY flag are invoked as a result of the notification that the previous play or record command has been completed. In the event that a record or play operation is aborted due to user interruption, the MCI_NOTIFY_ABORT message will be received. This message is treated the same as the successful notification message.

·	MCI_NOTIFY_FAILURE - This message is treated the same as the successful notification message. For debug purposes, it  generates an error message, but the example program supplied with the companion diskette does not attempt to correct for errors.

#### Remote Message Review

As mentioned above, the remote message playback feature of the application uses a separate state system than the main program logic. When in remote message review mode, the TAM state (wTamState) is set to TS_REMOTE_PLAY, and the remote state variable (wRemoteState) determines the state of the remote playback operation. Possible remote state values are as follows:

·	RS_WAIT

·	RS_WAITING

·	RS_PLAYMENU

·	RS_PLAYEND

These states determine the current action, or the next action to be taken when the current play is complete.

When the command key sequence for remote play is first entered, and messages are available, wRemoteState is set to RS_WAIT, and a greeting file is played ("You have messages..."). When the MCI_NOTIFY message is received indicating the end of the greeting, wRemoteState is changed to RS_WAITING, and the system does nothing until a key is pressed. If no key is pressed in a set amount of time, the application disconnects the telephone and resets the system.

If there are no messages available when the command sequence is entered, wRemoteState is set to RS_PLAYEND, and an exit message is played ("You have no messages..."). When the MCI_NOTIFY message is received indicating the end of the exit greeting, the telephone is disconnected, and the system is reset.

For normal message reviewing, wRemoteState is set to RS_PLAYMENU before a the message is played indicating that when the play has completed that the system should then play the verbal menu greeting ("Press 1 for next, 2 for erase..."). Before this menu is played, wRemoteState is set to RS_WAIT, indicating that the system should wait for a key press after playing the verbal menu.

The physical transition table for these states is as follows:

| Sample Application Source Code Current State | Next State |
| --- | --- |
| RS_WAIT | RS_WAITING (wait for a key) |
| RS_WAITING | undefined(continue waiting) |
| RS_PLAYMENU | RS_WAIT (play verbal menu) |
| RS_PLAYEND | Telephone disconnect & system reset |
| Table 5-5 |  |

To better illustrate the concepts of the TAM state machine introduced in this section, the source code to a sample application using the state machine is provided  in the "\tam" subdirectory on the companion diskette.

This section documents the structure of the sample application source code, and explains some of the more interesting routines.

#### Source Code Organization

The TAM sample application is more complex than the average sample application. The purpose of this is to fully demonstrate all the available functionality of the TAM MCI device drivers. The following is a short synopsis of the files included on the companion diskette:

| Source file | Description |
| --- | --- |
| MAKEFILE | Application makefile (for use with MS NMAKE.EXE) |
| MCIFTDD.H | Mwave MCI FAXTAM include file |
| TAM.H | TAM include - Global variable references |
| TAM.DEF | Windows definition file for use with the linker |
| TAM.RC | TAM resource source file. Contains system menus & dialog box definitions |
| TAMDEFS.C | Counterpart of TAM.H, containing global variable definitions |
| TAM.C | Program entry point. Contains initialization logic, and user interface code. |
| TAMFST.C | Main TAM state logic. Contains event handler & the majority of code which actually commands the MCI TAM driver. |
| DIALOG.C | Contains dialog procs for controlling all the dialog boxes used in the TAM application. |
| Table 5-6 |  |

In addition to the above, the diskette contains files with a .TAM extension.  These are audio files used during the operation of the sample.

#### Initializing the TAM driver environment

A startup example is supplied in the chapter on Telephony Services. The function shown below has been taken from the startup example, and modified to perform a simple TAM environment initialization. The steps to initialize the driver include:

1.	Open the driver and register the event message handler.

2.	Verifying minimum TAM capabilities (CAN_PLAY, CAN_RECORD, and CAN_SAVE). Exit if any are not supported.

3.	Check for optional speakerphone capability.

4.	Check the supported file formats, and use the custom format tag when available.

5.	Set the TAM call filter to have TAM auto answer incoming voice calls and route them to our application. Exit if another application is screening voice calls.

These initialization steps have been simplified in the sample code below, because we know the capabilities of the Mwave driver. This code would have to be adjusted for a more complex TAM application.

Initializing the TAM Driver - Windows

// InitDriverEnv

//

// This routine is called to initialize the TAM driver environment

//

static InitDriverEnv()

{

// Open the MCI TPL Driver

mciOpenParms.dwCallback = hEventHandler; // Always required on OPEN and CLOSE

mciOpenParms.lpstrDeviceType = "Mwavetpl";

if( dwBFE = mciSendCommand(0,MCI_OPEN,MCI_WAIT | MCI_OPEN_TYPE,

(DWORD)(LPVOID)&mciOpenParms) )

{

error_box();

return(0);

}

// Get the device ID & register the Event Handler for TPL so incoming phone

//  calls are sent to the application's event handler

wTplDeviceID = mciOpenParms.wDeviceID;

mciSetParms.dwCallback = hEventHandler;

mciSetParms.dwItem     = MCI_TAM_SET_EVENT_HANDLER;

mciSetParms.dwSetData  = hEventHandler;

mciSendCommand( wTplDeviceID,MCI_SET,MCI_WAIT | MCI_SET_ITEM,

(DWORD)(LPVOID) &mciSetParms);

// Open the MCI TPS Driver

mciOpenParms.dwCallback = hEventHandler;

mciOpenParms.lpstrDeviceType = "Mwavetps";

if( dwBFE = mciSendCommand(0,MCI_OPEN,MCI_WAIT | MCI_OPEN_TYPE,

(DWORD)(LPVOID)&mciOpenParms) )

{

error_box();

//although not in sample, should close TPL here

return(0);

}

// Get the device ID & register the Event Handler for TPS

wTpsDeviceID = mciOpenParms.wDeviceID;

mciSetParms.dwCallback = hEventHandler;

mciSetParms.dwItem     = MCI_TAM_SET_EVENT_HANDLER;

mciSetParms.dwSetData  = hEventHandler;

mciSendCommand( wTpsDeviceID,MCI_SET,MCI_WAIT | MCI_SET_ITEM,

(DWORD)(LPVOID) &mciSetParms);

// Set to auto answer incoming phone calls

mciSetParms.dwItem    = MCI_TAM_SET_CALL_FILTER;

mciSetParms.dwSetData = 1;

mciSendCommand( wTplDeviceID,MCI_SET,MCI_WAIT | MCI_SET_ITEM,

(DWORD)(LPVOID) &mciSetParms);

return(wTplDeviceID);

}

NOTE:	With Windows, the handle of the window procedure responsible for processing MM_MCINOTIFY messages MUST be specified by assigning it to mciOpenParms.dwCallback prior to calling the MCI_OPEN command, regardless of whether the MCI_WAIT or MCI_NOTIFY flag is specified in the MCI_OPEN call. Failure to do so when using versions earlier than 2.1 will result in erratic behavior of the device driver.

Initializing the TAM Driver - OS/2

tam.c: InitDriverEnv

// InitDriverEnv

//

// This routine is called to initialize the TAM driver

//

static int InitDriverEnv(void)

{

// Open the MCI TPL Driver

mciOpenParms.dwCallback = hEventHandler;

mciOpenParms.lpstrDeviceType = (INT *) "Mwavetpl";

dwBFE = mciSendCommand(0,

MCI_OPEN,

MCI_WAIT | MCI_OPEN_TYPE,

(DWORD)&mciOpenParms,

mci_cmd_ctr++);

if( dwBFE )

{

error_box();

return(0);

}

// Get the device ID & register Event Handler for TPL

wTplDeviceID = mciOpenParms.wDeviceID;

mciSetParms.dwCallback = hEventHandler;

mciSetParms.dwItem     = MCI_TAM_SET_EVENT_HANDLER;

mciSetParms.dwSetData  = hEventHandler;

mciSendCommand( wTplDeviceID,

MCI_SET,

MCI_WAIT | MCI_SET_ITEM,

(DWORD)&mciSetParms, mci_cmd_ctr++);

// Open the MCI TPS Driver

mciOpenParms.dwCallback = hEventHandler;

mciOpenParms.lpstrDeviceType = (INT *)"Mwavetps";

dwBFE = mciSendCommand(0,

MCI_OPEN,

MCI_WAIT | MCI_OPEN_TYPE,

(DWORD)&mciOpenParms,

mci_cmd_ctr++);

if( dwBFE )

{

error_box();

return(0);

}

// Get the device ID & register Event Handler for TPS

wTpsDeviceID = mciOpenParms.wDeviceID;

mciSetParms.dwCallback = hEventHandler;

mciSetParms.dwItem     = MCI_TAM_SET_EVENT_HANDLER;

mciSetParms.dwSetData  = hEventHandler;

mciSendCommand( wTpsDeviceID,

MCI_SET,

MCI_WAIT | MCI_SET_ITEM,

(DWORD)&mciSetParms,

mci_cmd_ctr++);

// Set to receive TAM phone calls

mciSetParms.dwItem    = MCI_TAM_SET_CALL_FILTER;

mciSetParms.dwSetData = 1;

mciSendCommand( wTplDeviceID,

MCI_SET,

MCI_WAIT | MCI_SET_ITEM,

(DWORD)&mciSetParms,

mci_cmd_ctr++);

return(wTplDeviceID);

}

#### Implementation of the Event Handler

The event handler routine is the core of the TAM state machine. All  state changes are a result of a NOTIFY or EVENT message sent to this routine. Note that although this routine has been isolated into its own procedure, the event handler code could be easily merged into the main window procedure, eliminating the need to create a separate window. It is also possible to implement a design where NOTIFY and EVENT messages are posted to different message procedures in the same application.

This module is the heart of the TAM application, and is the key for  understanding the various operations of the TAM state machine.

Handling Events - Windows

//

/	EventHandler

//

//	This function is called whenever a message is sent from the MCI TAM

//	driver. These messages drive new states of the TAM state machine.

//

long FAR PASCAL EventHandler(hWnd, message, wParam, lParam)

HWND hWnd;

unsigned message;

WPARAM wParam;

LPARAM lParam;

{

static UINT		uMCIMessage = 0xffff; // Initialize to invalid value

static short		wKeys[3];       	// Last 3 keys entered

static short		wQuiet;        	// Count for QUIET messages

static short		wKeysPressed;     	// Count for 3 key command

static short		wCmdKey;       	// Flag for 5-x play ctrl

unsigned short	wEvent;

unsigned long		dwEventData;

switch (message)

{

case WM_CREATE:

// Register the message we wish to look for

uMCIMessage = RegisterWindowMessage("MM_MCIEVENT");

break;

case MM_MCINOTIFY:

// *** Received a NOTIFY message indicating earlier call

//     w/ MCI_NOTIFY has completed

switch( wParam )

{

case MCI_NOTIFY_FAILURE:

case MCI_NOTIFY_SUCCESSFUL:

case MCI_NOTIFY_SUPERSEDED:

case MCI_NOTIFY_ABORTED:

switch( wTamState )

{

case TS_COMMAND_MODE:

case TS_PLAY_MESSAGE:

PlayComplete();

break;

case TS_REMOTE_PLAY:

ContinueRemote();

wQuiet = 0;

break;

case TS_PLAY_ANNOUNCEMENT:

RecordMessage();

wQuiet = 0;

break;

case TS_RECORD_MESSAGE:

SaveMessage();

break;

case TS_ARCHIVE_PLAY:

PlayComplete();

SendMessage(hMainWnd,WM_COMMAND,IDM_ARCHIVE,0l);

break;

default:

break;

}

break;

}

break;

default:

if( message == uMCIMessage )

{

// *** Received an EVENT message ***

// Isolate the message parameters

MCI_EVENT_PARMS far *mep = (MCI_EVENT_PARMS far *)lParam;

wEvent   = LOWORD( mep->dwDataParam1 ); // or wParam

dwEventData = mep->dwEventData;

switch( wEvent )

{

case PHONE_EVENT_CALL_TAM:

wKeysPressed = 0;

AnswerCall();

break;

case PHONE_EVENT_CALL_TERMINATED:

CallTerminated();

break;

case PHONE_EVENT_CALL_PROGRESS:

if( wTamState == TS_RECORD_MESSAGE ||

(wTamState==TS_REMOTE_PLAY &&

wRemoteState==RS_WAITING))

switch( dwEventData )

{

case DIALTONE:

case SLOWBUSY:

case FASTBUSY:

CallTerminated();

break;

}

break;

case PHONE_EVENT_LINE_KEY:

if( wTamState == TS_REMOTE_PLAY )

{

if( wCmdKey == 5 ) // Check for play ctrl sequence

{

wCmdKey = -1;

switch( dwEventData )

{

case 1:    // (51) Seek back 5 seconds

SeekMessage(TB_BACK);

break;

case 2:    // (52) Pause (or resume)

if(!(wPause^=1))

mciSendCommand( wOurDeviceID,

MCI_RESUME, MCI_WAIT, 0);

else

mciSendCommand( wOurDeviceID,

MCI_PAUSE, MCI_WAIT, 0);

break;

case 3:    // (53) Seek ahead 5 seconds

SeekMessage(TB_FORWARD);

break;

}

}

else        // Standard Remote Play command

{

switch( dwEventData )

{

case 1:    // (1) Play first / next

RemoteNext();

break;

case 2:    // (2) Remove current message

RemoteRemove();

break;

case 3:    // (3) Repeat current message

RemoteRepeat();

break;

case 4:    // (4) Archive current message

RemoteArchive();

break;

case 5:    // Initiate 2 key (5x) sequence

wCmdKey = (short)dwEventData;

break;

}

}

}

else         // Check for 3 digit command code

{

wKeys[2] = wKeys[1];

wKeys[1] = wKeys[0];

wKeys[0] = (short)dwEventData;

if( ++wKeysPressed > 2 )

{

if((wKeys[2]*100+wKeys[1]*10+wKeys[0])==wCommandCode)

{

BeginRemote(); // Initiate remote playback

wCmdKey = -1;  // Reset command key status

}

}

}

break;

default:

break;

}

}

else

return (DefWindowProc(hWnd, message, wParam, lParam));

}

return (NULL);

}

Handling Events - OS/2

MRESULT EXPENTRY MyWindowProc ( HWND hwnd,

ULONG msg,

MPARAM mp1,

MPARAM mp2 )

{

HDC hdc;

static int          InitEnv    = 0;

static short        wKeys[3];               // Last 3 keys entered

static short        wQuiet;                 // Count for QUIET messages

static short        wKeysPressed;           // Count for 3 key command

static short        wCmdKey;                // Flag for 5-x play ctrl

unsigned short      wEvent;

unsigned long       dwEventData;

static int          FlashState = 0;

switch( msg )

{

case MM_MCINOTIFY:

switch( SHORT1FROMMP(mp1) )

{

case MCI_NOTIFY_FAILURE:

case MCI_NOTIFY_SUCCESSFUL:

case MCI_NOTIFY_SUPERSEDED:

case MCI_NOTIFY_ABORTED:

switch( wTamState )

{

case TS_COMMAND_MODE:

case TS_PLAY_MESSAGE:

PlayComplete();

break;

case TS_REMOTE_PLAY:

ContinueRemote();

wQuiet = 0;

break;

case TS_PLAY_ANNOUNCEMENT:

RecordMessage();

wQuiet = 0;

break;

case TS_RECORD_MESSAGE:

SaveMessage();

break;

case TS_ARCHIVE_PLAY:

PlayComplete();

break;

default:

break;

}

break;

}

break;

case MM_MCIEVENT:

mep = (LPMCI_EVENT_PARMS)mp2;

wEvent = LOWORD( mep->dwDataParam1 );  // or wParam

dwEventData = mep->dwEventData;

if (dwEventData >= '0')

dwEventData -= '0';

else if (dwEventData == '#')

dwEventData = 35;

else if (dwEventData == '*')

dwEventData == 42;

switch(wEvent)

{

case PHONE_EVENT_CALL_TAM:

wKeysPressed = 0;

AnswerCall();

break;

case PHONE_EVENT_CALL_TERMINATED:

CallTerminated();

break;

case PHONE_EVENT_CALL_PROGRESS:

if( wTamState == TS_RECORD_MESSAGE ||

(wTamState==TS_REMOTE_PLAY && wRemoteState==RS_WAITING) )

switch( dwEventData )

{

case DIALTONE:

case SLOWBUSY:

case FASTBUSY:

CallTerminated();

break;

}

break;

case PHONE_EVENT_LINE_KEY:

if( wTamState == TS_REMOTE_PLAY )

{

if( wCmdKey == 5 )  // Check for play ctrl sequence

{

wCmdKey = -1;

switch( dwEventData )

{

case 1:

SeekMessage(TB_BACK);

break;

case 2:

if(!(wPause^=1))

mciSendCommand( wTpsDeviceID, MCI_RESUME, MCI_WAIT,

(DWORD)&mciGenericParms, mci_cmd_ctr++);

else

mciSendCommand( wTpsDeviceID, MCI_PAUSE, MCI_WAIT,

(DWORD)&mciGenericParms, mci_cmd_ctr++);

break;

case 3:

SeekMessage(TB_FORWARD);

break;

}

}

else                // Standard Remote Play command

{

switch( dwEventData)

{

case 1:

RemoteNext();

break;

case 2:

RemoteRemove();

break;

case 3:

RemoteRepeat();

break;

case 4:

RemoteArchive();

break;

case 5:       // Initiate play ctrl sequence

wCmdKey = (short)dwEventData;

break;

}

}

}

else                    // Check for 3 digit command code

{

wKeys[2] = wKeys[1];

wKeys[1] = wKeys[0];

wKeys[0] = (short)dwEventData;

if( ++wKeysPressed > 2 )

{

if((wKeys[2]*100+wKeys[1]*10+wKeys[0])==wCommandCode)

{

BeginRemote();

wCmdKey = -1;

}

}

}

break;

default:

break;

}

## Using the TAM Sample Application

The sample application included on the companion diskette is   designed primarily to illustrate some of the concepts behind the creation of an event driven application using the Mwave TAM API. Although the sample is also a functional telephone answering machine, it doesn't contain the error recovery  or feature set required of a robust application.

The TAM example applet requires  the following hardware in addition to the base Mwave hardware:

·	A telephone handset attached to the Mwave adapter telephone port (if you want to try the handset functions)

·	An analog phone line attached to the public switch network (if you want to initiate and receive real calls)

·	A microphone attached to the Mwave adapter microphone input (if you want to use the microphone functions)

·	One or two speakers attached to the Mwave adapter speaker ports (if you want to use the speaker functions)

The TAM applet is designed to operate both as a standard telephone and a telephone answering system.  When using either the telephone to take a call, or the answering system to review messages, the application allows the user to select either a desk telephone handset or an external microphone/speaker as an input/output device.  This allows for private reviewing of messages through the handset, and adds speakerphone capability to a standard telephone through the microphone and speaker devices.

To implement this dual functionality, the application operates in two distinct modes, a 'Telephone' mode and a 'Message Review' mode.  The operating mode is set by the user, through the 'Mode' pulldown menu.

In 'Telephone' mode, the applet answers incoming calls, plays an announcement, and records messages.  When system output is set to 'Handset', the desk telephone is connected directly to the  telephone line and on-hook off-hook is .  When the output is set to 'Speaker', the telephone is taken off hook and the system microphone and speaker are enabled.

In 'Message Review' mode, the application can play recorded messages to either the speaker or telephone handset.  All message play controls are located under the 'Play Control' menu.  When message reviewing is taking place, the system does not answer  incoming calls.

The handset volume is not adjustable with the applet, but the speaker volume (used for both speakerphone and message review operations) can be set using the 'Volume' menu.

### System Setup

The TAM applet is pre-loaded with a default announcement greeting and some other default settings, but there are some initialization steps to perform if you wish to tailor it to your requirements.  Below are specific instructions for the various initialization procedures.

#### Recording an Announcement

To record a new announcement, select the 'Message Review' mode option under the 'Mode' menu, and then select 'Record Announcement...' under the 'Configure' menu.  A three part dialog box is displayed.

The top portion of the dialog box is used for recording.  The 'Record from...' box in the upper left hand corner of the dialog box determines the input recording device.  The default device is set to 'Microphone' so if you wish to record your announcement from the telephone handset, first select the 'Telephone Handset' button in the 'Record from...' box.

After the input device has been chosen, click on the 'Begin Recording' button and start speaking (once you hear a beep) into the input device you have chosen.  When done, click on the 'End Recording' button.  To listen to the announcement you just recorded, select an output device in the 'Play to...' box, and click on the 'Play Announcement' button.  The announcement plays to the output device selected.  If you wish to use the newly recorded announcement, click on the 'OK' button.  Otherwise, to keep the old

announcement, click on the 'Cancel' button.

After closing the dialog box,  place the application back into telephone mode by selecting the 'Telephone' mode option in the 'Mode' pulldown menu (to enable the system to take messages)

#### Setting the Ring Count

The ring count determines the number of rings before which the system answers the telephone to record an incoming message.  The TAM application has two ring counts, one for when messages are available, and one for when no messages are available.  A common ' toll saver' feature is to set the device to answer on the first ring when new messages are available, but not to answer before the fourth ring when there are no new messages.  This allows the user to hang-up when calling remotely before the system answers when there are no messages available.

To set the ring count, select the 'Set Ring Count...' item in the 'Configure' menu.  A dialog box is displayed, prompting for the two types of ring counts.  After entering a new ring count for when messages are available and one for when no messages available, press the 'OK' button to use the new counts, or press the 'Cancel' button to abort any changes.

#### Setting the Command Code

The TAM application allows the user to retrieve messages from a remote telephone, by calling the device, and when prompted to record a message, entering instead a 3-digit command code on the touch-tone keypad.  The command code is configurable, and can be changed any time.  To set a new command code, select the 'Set Command Code...' item on the 'Configure' menu. A dialog box prompting for a new command code is displayed.  After entering a new command code, press the 'OK' button to accept the change, or press the 'Cancel' button to abort any changes and keep the original code.

### Using the Speakerphone

In addition to providing the answering machine function, the TAM application turns a speaker/microphone connected to the Mwave Adapter into a speakerphone.

#### Initiating a Speakerphone Call

Before initiating a call for use with the speakerphone, verify that the application is in telephone mode by selecting the 'Telephone' option under the 'Mode' pulldown menu, and that the system is connected to the handset by selecting the 'Handset' item under the 'Mode' pulldown menu.

Note: These two options should always be set when you are not reviewing messages.  Otherwise, the system will not take messages

The speakerphone call is initiated by picking up the handset and dialing the number r using your standard desk phone.  After the number has been dialed, the speakerphone is initiated by selecting the 'Speaker' option under the 'Mode' pulldown menu.  After this has been done, you can hang-up the telephone handset.

#### Speakerphone Volume Control

While the speakerphone call is in progress, the output volume of the speaker can be adjusted by selecting a volume level from the 'Volume' pulldown menu.

#### Terminating a Speakerphone Call

To terminate a speakerphone call, leave the desk phone handset on hook, and place the system back into handset connect mode by selecting the 'Handset' item under the 'Mode' pulldown menu.  This also places the phone 'on-hook'.

### Reviewing Messages Locally

The number of active messages on the system is shown in the application window whenever the application is open on the desktop.  If new messages arrive while the application is in icon form(i.e. minimized), new messages can be detected by the flashing of the icon text (either on the desktop or in the Minimized Window Viewer).  If the TAM application window is open, its title bar flashes if new messages have arrived since the last message review.

#### Playing Recorded Messages

To review messages, select the 'Message Review' mode from the 'Mode' pulldown menu.  Incoming calls are not answered while you are in this mode.

Next,  select the output device to which to play your messages.  You can play messages to either the telephone handset or the external speaker.  To use the handset, select the 'Handset' option on the 'Mode' pulldown menu.  To use an external speaker, select the 'Speaker' option on the 'Mode' pulldown menu.  When using the speaker, you can adjust the speaker volume be selecting a new volume level from the 'Speaker Volume' pulldown menu.

When in message review mode, some of the entries on the 'Play Control' pulldown menu become visible.  To start reviewing messages, select the 'First' item on the 'Play Control' pulldown menu.  This prompts the system to play the first active message.  If the 'First' item is grayed on the menu,  you have no active messages.

After the first message has played, you have a choice of either replaying the message, keeping the message and playing the next message (if any), or erasing the message and playing the next message (if any).  To replay the message, select the 'Repeat' option on the 'Play Control' pulldown menu.  To keep this message

and play the next message, select the 'Next' option.  To erase this message and play the next message (if any), select the 'Erase' option.

After playing one of multiple  messages, you can go back and play the previous message by selecting the 'Previous' option.

Note: When you have finished reviewing messages,  re-enable call receiving by selecting the 'Telephone' mode option under the 'Mode' pulldown menu.

#### Message Positioning Controls

While a message is playing, you can step back 5 seconds in the message, step forward 5 seconds in the message, or pause the message playback.  When paused, the playback will remain stopped until unpaused (pause selected a second time).  These options are performed by selecting the 'Back 5 seconds', 'Ahead 5 seconds', or 'Pause' menu entries under the 'Play Control' pulldown menu.

#### Message Speed Controls

As well as being able to skip around in a message using 'Step Back' and 'Skip Forward', the TAM application  allows you to set the play speed of the message, so that you can play back messages at an accelerated or decelerated rate.  To change the message play speed, select a new speed (either 'Play Slow', 'Play Normal', or 'Play Fast') from the 'Play Control' pulldown menu.  The speed setting you choose remains in effect until you exit the message review mode at which point it reverts to 'Play Normal'.

### Reviewing Messages Remotely

Message review is also possible using a touch-tone telephone, when calling from a remote location.  To gain access to the message review mode of the application, first call the system and let the answering machine answer the call.  While  the announcement is playing, enter the 3 digit command code you selected during the application setup process.  The system now tells you how many active messages you have.  At this point, you can begin entering touch-tone keypad commands to review messages as described in the previous section.  Below is a list of the available keypad command options:

Message Play Commands:

Press '1' to play first/next active message

Press '2' to erase current message and play next active message

Press '3' to replay or restart the current message