Release 1.00

OS/2 PM Utility Disk   README.DOC

May 10, 1991

This file, README.DOC, contains information that is more

recent than that contained in the Software Guide for

OS/2 Presentation Manager.  Check this file for information

you may need to set-up or use your Renaissance Intelligent

Graphics Controller with OS/2 PM.

***IMPORTANT***

When you install the Renaissance OS/2 PM driver, the installation

will overwrite the existing config.sys file.

If you should ever want to run PM with your old driver again, you

need to copy the config.sys file before you start our installation.

We recommend that you do this.  To copy the file, go to an OS/2 window

or the DOS box and do the following:

copy config.sys config.old

Renaissance Set-up Notes

-------------------

*** Hewlett-Packard Vectra RS/20 PC

The HP Vectra RS/20 386 PC will not boot if an RVGA

daughterboard is installed on the Renaissance Graphics

Controller.  There is no problem if the RVGA daughterboard is

NOT installed.   Consequently, for VGA compatibility, a

separate VGA (either passed-through to the Renaissance Graphics

Controller or connected to its own monitor) is required in the system.

====End README.DOC==========================================================