Graphics Vision 1024/Rendition IIe  Release 1.02		README.DOC

July 2, 1992

This file, README.DOC, contains information that is more recent

than that contained in the User Guide and the Software Driver Guide.

Check this file for information you may need to set-up or use your

Intelligent Graphics Controller.  If you should need

more assistance, the Appian Technical Support staff will gladly

assist you Monday through Thursday between the hours of 8:00 AM

and 5:00 PM Pacific Time and Friday from 8:00 AM to 4:00 PM Pacific

Time.  You may reach them at (206) 649-8086 or send your concerns to:

Appian Technology Inc.

2405 140th Ave. NE

Bellevue, WA 98005

FAX (206) 649-5360

8514/AI

-------

In order to run 8514/AI applications and to insure their compatibility with

the Graphics Vision 1024, be certain the unit you have purchased is the

256-color version which comes with 1 megabyte of VRAM.  Additionally, you

will have to purchase 512K DRAM.  The reason for the additional DRAM is that

some 8514/AI applications have taken advantage of a paging technique.  This

paging technique requires a full 1 megabyte of memory with no memory left for

program space.  Therefore, the additional DRAM is also required.  Contact

your dealer to obtain additional memory for your GV1024.

TIGA-340

--------

The TIGA-340 Interface Driver for TIGA version 2.05 needs 1M of VRAM.

It will be not work with only 512K of VRAM and 0K of DRAM.  To upgrade

to 1M of VRAM you MUST purchase a Memory Upgrade Board.  Installation

instructions for the Memory Upgrade Board will be shipped with the

Memory Upgrade Kit.  To obtain the Memory Upgrade Kit contact your

dealer.

Intelligent Graphics Controller Setup Notes

------------------

*** Mitsubishi HL6605 Monitor

If you have a Mitsubishi HL6605 connected to your Intelligent

Graphics Controller, select the Mitsubishi HL6905 when you run

the Intelligent Graphics Controller INSTALL program.

*** Dell Model 220

The VGA built into the Dell Model 220 does not have an

IBM standard feature connector, consequently it cannot

be used for VGA pass-through to the Intelligent Graphics

Controller.  There are two possible solutions to this:

1)  Connect a separate monitor to the 220's VGA and do

not pass through the VGA to the Intelligent Graphics

Controller.

2)  Disable the 220's built-in VGA and use the RVGA

daughterboard for VGA compatibility.

*** Hewlett-Packard Vectra RS/20 PC

The HP Vectra RS/20 386 PC will not boot if an RVGA

daughterboard is installed on Intelligent Graphics

Controller.  There is no problem if the RVGA daughterboard is

NOT installed.   Consequently, for VGA compatibility, a

separate VGA (either passed-through to the Intelligent

Graphics Controller or to its own monitor) is required in the system.

Board Diagnostic Utility

------------------------

It is always a good idea to check your Intelligent Graphics

Controller the first time you install it with the Board

Diagnostic Utility, UDIAG.  The User Guide has

detailed instructions for running this utility.  If you

have a monochrome monitor connected to your board, the

"video color bars" test will not display colors which

correspond to the text shown (it will display shades of

gray, instead).  If you are using Single Test Mode, simply

press "Y" to continue to the next test.  All the other

diagnostics are valid for monochrome monitors.  In addition,

if you have a monitor which is not VGA compatible, the color

bars test may not sync properly.

====End README.DOC===================================================