How to Install the Packet Driver

======================================

The ETHER-32V/P Packet Driver for the PCnet architecture (backwards compatible

to the Am2100/Am1500T) is based on the packet driver from Clarkson

University.  The Driver Diskette contains the executable code for

the DOS version of the Packet Driver.

Copy the ETHER-32V/P Packet Driver, PCNTPK.COM, in a directory. The packet

driver can be loaded from the DOS prompt or from the AUTOEXEC.BAT

file.The command line format for Packet Driver is:

PCNTPK [-n] [-d] [-w] <INT=packet_int_no> [IRQ=int_no]

[IOADDR=io_addr] [DMA=dma_no]

If the [IRQ=int_no], [IOADDR=io_addr], and [DMA=dma_no] are not

specified for the ETHER-32V/P adapter card, the default values are

assumed by the Packet Driver.

Additional common keywords may be specified.  Please refer to the

Software Keyword section for the available keywords on README of root.