MMRST.COM     MM Format Reset Utility

Copyright Summagraphics Corporation 1990, 1991.   All rights reserved.

MMRST transmits a reset command (ASCII NULL) to the tablet out the selected

serial port.

Usage: MMRST [ options ]

/1  (default) selects serial port 1 {PC, XT, AT, PS/2}

/2  selects serial port 2 {PC, XT, AT, PS/2}

/3  selects serial port 3 {PS/2 only}

/4  selects serial port 4 {PS/2 only}

/5  selects serial port 5 {PS/2 only}

/6  selects serial port 6 {PS/2 only}

/7  selects serial port 7 {PS/2 only}

/8  selects serial port 8 {PS/2 only}

/BAxxxx  selects serial port with base address of xxxx (in HEX)

/N  selects no parity configuration

/? or /H  displays help screen

Notes: -When using the BA option (Base Address) all four digits must be

specified.  (example: for COM 1 - /BA03F8)

-This program transmits in the SummaSketch/MM format default serial

communication configuration which is 9600 baud, 8 data bits,

odd parity, and 1 stop bit.

-Use the /BA switch only when the serial port is located at a non-

standard address within the computer.

-A summary of command line switches is always available by starting

the program with switch /? or /H.

Serial Port Address Table

Command Line	Serial	   Serial Port	 System

Switch		Port	   Address

------------------------------------------------------------------------------

/1		1	   03F8 Hex	 All (PC, XT, AT, 386, and any PS/2)

/2		2	   02F8 Hex	 All (PC, XT, AT, 386, and any PS/2)

/3		3	   3220 Hex	 Any PS/2

/4		4	   3228 Hex	 Any PS/2

/5		5	   4220 Hex    	 Any PS/2

/6		6	   4228 Hex	 Any PS/2

/7		7	   5220 Hex	 Any PS/2

/8		8	   5228 Hex	 Any PS/2

------------------------------------------------------------------------------

*********************** 87-1009-001 **********************************