Installation des CD56E/CD58E unter WINDOWS 95

Installation of CD56E/CD58E under WINDOWS 95 (see below)

1. 	In der AUTOEXEC.BAT darf auf keinen Fall ein DOS Treiber

(MSCDEX bzw. CORELCDX) geladen werden, wenn das Laufwerk

als 32-BIT Gerät unter WINDOWS95 betrieben werden soll.

2.	Falls das Laufwerk nach dem Beenden von WINDOWS95 in DOS

zur Verfügung stehen soll, müssen Sie in der CONFIG.SYS

folgenden Eintrag machen.

DEVICE=C:\TEAC\TEAC_CDI.SYS /D:CDI /P:S-M

Wobei \TEAC das Verzeichnis ist, in den sich die Treiber

des CD56E/CD58E befinden, und das Lufwerk als MASTER am SEKUNDÄREN

Port angeschlossen ist. Diesen Eintrag müssen Sie entsprechend

Ihrer Konfiguration ändern, wobei die Abkürzungen folgende

Bedeutungen haben:

/P: (P)rimär, (S)ekundär,(T)ertiär,(Q)uartiär - (M)aster,(S)lave

In der Datei C:\WINDOWS\DOSSTART.BAT muß dann noch der High Level

Treiber nachgeladen werden, dazu müssen Sie folgenden Eintrag in

diese Datei einfügen:

C:\WINDOWS\COMMAND\MSCDEX.EXE /D:CDI /M:18

3.	Starten Sie nun WINDOWS95 und führen Sie eine automatische Hardware-

erkennung durch.

Nach dem Neustart von WINDOWS95 steht Ihnen das Laufwerk als 32-BIT

Gerät zur Verfügung.

4.	Falls Sie das Gerät in der ursprünglichen DOS Umgebung nutzen wollen

müssen Sie entprechend die Dateien AUTOEXEC.DOS und CONFIG.DOS

editieren.

1.	Please do not load any DOS drivers (such as MSCDEX or CORELCDX)

in your autoexec.bat) if you are trying to use CD56E/CD58E as a 32-BIT

device.

2.	If you want to use your drive under DOS after you have closed your

WINDWOS session, please modify the mentioned files as listed below:

CONFIG.SYS:

DEVICE=C:\TEAC\TEAC_CDI.SYS /D:CDI /P:S-M

here \TEAC is the directory which contains the driver files for the

CD56E/CD58E and the drive is connected to the secondary port as MASTER.

please change this settings to your configuration.

/P:(P)rimary,(S)econdary,(T)ertiary,(Q)uartiary - (M)aster,(S)lave

C:\WINDWOS\DOSSTART.BAT

C:\WINDOWS\COMMAND\MSCDEX.EXE /D:CDI /M:18

3.	Start WINDOWS95 and run the hardware detection, after rebooting your

system the CD56E/CD58E will apear as a 32-BIT device.

4.	If you want to use the CD56E/CD58E in your old DOS enviroment you have to

edit the files AUTOEXEC.DOS and CONFIG.DOS as mentioned above.

**** Regarding TEAC CD-56E/CD58E Trition Chip Driver Beta Version *****

The CD-56ECD/58E Triton Chip Driver Beta Version is the Windows 95 CD-ROM device driver utilizing the FAST IDE Interface function of Triton Chip (82371FB) produced by Intel Corporation. High-speed transfer (13.3MHZ) realized by DMA Bus Master format improves the CPU usage rate to 5-10%. This is fully suitable for playback of moving picture, etc.

Note that the Beta version has following restrictions at present:

1. Restricted Items

a) This is the driver exclusively for the Secondary IDE CD-ROM of Triton Chip.

b) Only CD-ROM can be connected to the Secondary IDE (Up to two units can be

connected)

c) Some of Triton Chip mother boards cannot be used for this driver.

d) This is the Windows 95 device driver exclusively for TEAC CD-ROM CD-56E/CD58E.

2. Installing Method

Installing Intel Zapper/Endever mother board

a) Click the System icon in the Control Panel.

b) Click the Device Manager.

c) Click the Hard disc controllers.

d) Two Standard IDE/ESDI Hard disc controllers are indicated.

e) Double click one of controllers indicated in order to confirm which is the Secondary IDE.

f) Click the Resources as the property of the controller is indicated.

g) The Secondary IDE is the one that uses 1F0-1F7 for Input/Output Range in the

infomation of Resources. Confirm which controller uses this Input/Output Range.

h) After confirming the Standard IDE/ESDI Hard disc controller, get out of the Property by

clicking „Cancel“ and delete the driver that uses the Secondary IDE by clicking

„Remove“.

i) Restart Windows 95 after deleting the driver.

j) „Unknown Device“ is detected at starting Windows 95 and „New Hardware Found“ is

indicated in the dialogue box.

k) Choose „Driver from disk provided by hardware manufacturer“ and click „OK“.

l) Insert the folppy disc of TEAC CD-56E/CD58E Triton Chip Driver Beta Version into the drive and start installing by choosing „Have Disc...“.

m) The CD-ROM device driver becomes valid by rebooting after installing.

Installing AsusTek/Micronics/Air mother board

a) Choose „Run...“ from „Start“ and input „REGEDIT“ to execute.

b) Click HKEY_LOCAL_MACHIN directory in the REGEDIT display.

c) Start clicking Enum\MF\CHILD0001 in the directory and click

\PCI&VEN_8086&DEV_1230&BUS_00&DEV_07&FUNC_01.

d) Take a note of figures (000?) in data „hdc\000?“ of „Driver“ in the Name/Data section

of REGEDIT.

e) Return to HKEY_LOCAL_MACHIN directory and start clicking

System\CurrentControlSet\Service\Class\hdc.

f) Double click „000?“ noted in procedure d).

g) Change Data ESDI_506.pdr of „PortDriver“ in the Name/Data section into

TEAC6X.MPD.

h) Terminate REGEDIT after completing the above change.

i) Copy TEAC6X.MPD to C:\Windows\System\Iosubsys directory from the Beta Version

floppy disc using My Computer.

j) CD-ROM device driver becomes valid by rebooting afer * completing copy.

3. TEAC Corporation License Agreement

Before using this software, read following sentences carefully. It is regarded that users agree to be restricted by articles of this agreement by using this software.

a) License

Ownership of a disc that has TEAC software shall belong to a user.

However, all rights and ownership concerning TEAC software shall belong to TEAC. TEAC software can only be utilized for personal use and making a copy except for backup use shall be prohibited.

b) Restrictions

TEAC software contains copyright material, confidential material and patent. In order to protect those, it is prohibited to translate, change, disassemble TEAC software or part of it or deform it to the form the third party can recognize. It is prohibited to change whole or part of TEAC software, use it in any network, lease, lend, resell for profit, distribute and make secondary copyright material based on this software. It is also prohibited to transmit TEAC software from one computer to another via telephone line or network.

c) Disclaimer of Warranty on TEAC Software

Users shall owe all responsibility for using TEAC software. TEAC software shall be used „as it is“ and there shall be no warrant for contents. TEAC shall not warrant this software as product, nor conformance for specified purpose. TEAC shall not warrant that TEAC software does not fail and all the failure in TEAC software is repairable. Furthermore, TEAC shall not warrant or make a proof for the use of this software, its operation and result of operation.

Information or advice by TEAC or authorized representatives of TEAC shall not warrant anything or expand the scope of this agreement. Even if the TEAC software proves to be defective, users shall owe all the expenses for required service and repair or mending. TEAC and authorized representatives of TEAC shall not owe any burden.

d) Limitation of Liability

TEAC shall not owe any resposiblility for any hindrance or problem caused by using the TEAC software without regard to articles of this agreement.

e) Completion of Agreement

Articles of this agreement stipulate complete agreement concerning use of the TEAC software. Any modification or change of these articles shall not be binding without consent of duly authorized representatives of TEAC.

Copyright C 1995 TEAC Corporation Ltd. All Rights Reserved.

Triton is registered trademark of Intel Corporation.

Zapper and Endever are registered trademark of Intel Corporation.

Windows 95 is registered trademark of Microsoft Corporation.