| Field Note: 113 |  | Distribute freely |
| --- | --- | --- |

Subject:  	Changing the monitor list

Date Created: 	July 29, 2026

Product: 	928Movie

This field note describes the format of the 928Movie monitor list and how to add your own monitors to the list.

The capabilities and preset modes of each monitor in the 928Movie list are stored in 2 files:

928S1MB.CAP

928S2MB.CAP

The monitor capabilities should be identical for each file.  The first file has the best preset modes for the 1MB VRAM card and the latter for the 2MB card.  We have separate files because the 2MB card can display 16.7 million colours and higher resolution modes and so the best presets are different.

When the Windows display driver runs, it selects ONE of the files according the amount of VRAM detected during boot-up.

The files named 928M1MB.CAP and 928M2MB.CAP are capability files for the 928Movie card itself and must NOT be changed.  Do not confuse them with the 928S*.CAP files.

## File format

The files are ordinary ACSII text and can be edited with DOS Edit or Windows Notepad.  Here is the format:-

; comment in any line preceded with a semi-colon

VGA monitor, 14, 60, 60, 0, 0, 0, 0, 0, 0, 0, 0<cr>

640, 480,  16, 60, S<cr>

640, 480,  16, 60, S<cr>

640, 480,  16, 60, S<cr>

mon 2,size,640R1,640R2,800R1,800R2,1024R1,1024R2,1152R1,1152R2,1280R1,1280R2<cr>

Preset mode 1<cr>

Preset mode 2<cr>

Preset mode 3<cr>

.

mon 3,size,640R1,640R2,800R1,800R2,1024R1,1024R2,1152R1,1152R2,1280R1,1280R2<cr>

Preset mode 1<cr>

Preset mode 2<cr>

Preset mode 3<cr>

.

.

.

mon X,size,640R1,640R2,800R1,800R2,1024R1,1024R2,1152R1,1152R2,1280R1,1280R2<cr>

Preset mode 1<cr>

Preset mode 2<cr>

Preset mode 3<cr>

syntax of mon X, the monitor's name, is ASCII characters but not   + / - . , " ;

syntax of size is 2 digit number in inches of the screen diagonal (CRT)

640R1  is lowest refresh rate supported in 640x480 resolution for that monitor

640R2  is highest refresh rate supported in 640x480 resolution for that monitor

800R1  is lowest refresh rate supported in 800x600 resolution for that monitor

800R2  is highest refresh rate supported in 800x600 resolution for that monitor

... and so on up to 1280 resolution

Where refresh rate is number (of Hertz) between 0 and 255

Note that refresh rate of 0 denotes that the resolution is not supported at all

Preset mode syntax is:

X-resolution,Y-resolution,bits per pixel,refresh mode,font size

where 	bits per pixel is 4/8/16/32  see table below

refresh mode is 43/60/72/75 Hz for 928Movie

font size is S/L (small or large)

Preset mode 1 is the default when a monitor is selected with the 928Movie Display applet.  Three presets must be defined and no more.

The refresh mode and colour depth MUST be supported at that resolution by 928Movie for any preset.  The latest modes supported by 928Movie are listed in the file 928M1MB.CAP for 1MB cards and in the file 928M2MB.CAP for 2MB cards.  Note that refresh mode 43 is interlaced, all others are non-interlaced

| bits per pixel | number of colours |
| --- | --- |
| 4 | 16 |
| 8 | 256 |
| 16 | 65 thousand |
| 32 | 16.7 million |

The first monitor is the default that is selected during 928Movie Setup and should be VGA in order to match the settings chosen if the display driver is added manually using Microsoft Setup.

Carriage returns <cr> and comment lines can be added anywhere next to a carriage return.

Spaces can be added next to any comma.

Here is an example for the IBM 17X monitor, for the 1MB and the 2MB cards respectively:

;                    [ 640 ] [ 800 ] [1024]  [1152]  [1280] comments<cr>

IBM 17X, 17,         60, 75, 60, 75, 43, 75, 60, 72, 43, 72<cr>

800, 600, 16, 75, S<cr>

1152, 864,  8, 72, S<cr>

1280,1024,  4, 60, L<cr>

IBM 17X, 17,         60, 75, 60, 75, 43, 75, 60, 72, 43, 72<cr>

1152, 864, 16, 72, S<cr>

1280,1024,  8, 60, L<cr>

800, 600, 32, 75, S<cr>

This shows that the 17" monitor can support 1024x768 resolution between 43 and 75Hz and the default preset is 800x600 16 bits per pixel at 75Hz, small fonts for a 1MB card.

## How to add a monitor to the list

The data sheet for the new monitor is required from the manufacturer.  This will show the maximum refresh rate supported for each resolution and the maximum resolution for the screen.  If the resolution (such as 1152x864) is not described, then the refresh rate can be calculated as follows:

- Find the horizontal scan frequency range for the monitor, e.g. 31.5 - 64 kHz

- Find the 928Movie Display modes in the Technical details section in the User Manual

- Look at the horizontal scan frequency for a vertical refresh rate, e.g. 54kHz for 60Hz refresh

- If the horizontal scan frequency given if LESS THAN OR EQUAL TO the monitor's maximum

horizontal scan frequency then this refresh rate is supported OK.

E.G.  A 31.5 - 65 kHz horizontal scan frequency range monitor supports 1152x864 at 60Hz refresh (54kHz scan) but NOT at 72Hz refresh (66kHz scan)

The presets are chosen so that the highest resolution and refresh rate for different colour depths are easily available.  Mode 1 is the default and is chosen to be 65k colours.  Any conventional monitor will support colour depths from 16 up to 16.7 million because the signal to the monitor is analogue.

Use any text editor, e.g. Notepad, to add new monitors to the files 928S1MB.CAP and 928S2MB.CAP using the syntax described above.  Save the original files first in case of problems.  New monitors must be added to BOTH files so that they are supported for both the 1MB and 2MB versions of 928Movie.  I suggest adding a monitor and then running the Display applet to check that the monitor is included correctly.

An easy way to add a monitor is to copy the 4 lines for an identical monitor, paste them in the file (keep the alphabetical order of manufacturer) and then change the monitor name to the new model.

When the 928Movie Display applet (928DIS.EXE) is run, it checks the syntax of the capability file and alerts the user to any error in any monitor description or if any preset modes are invalid for that version of 928Movie.  The monitors are numbered from 1 to X.  The screen size in inches is not used at present but may be used in the future to choose small or large fonts when the user selects Auto fonts in the Display applet.

The applet's Edit mode only allows modes that both the monitor and the graphics card can support.  If the presets are edited in Edit mode then the new presets are stored in the 928MOVIE.INI file.

If a monitor is reselected, then the original default presets are loaded from the capability file.

The refresh rates in the capability file can be a higher figure than those used by 928Movie because the applet checks if the rate requested is lower than or equal to the monitor's capability.  This allows other VideoLogic graphics cards, such as PCI Movie, to use the same monitor capability information.

When the files are complete, copy them to the 928MOVIE directory.  Do NOT compress them or add them to the 928Movie Setup for Windows floppy disks because this would stop the 928Movie Setup from running.

Please check that the new monitors can support the modes you enter in the capability files.  You can damage a monitor by choosing a mode that it cannot support.

Steve Everitt

July, 26