| Field Note: 116 |  | Distribute freely |
| --- | --- | --- |

Subject:  	EDRAM cache PCs

Date Created: 	July 30, 2026

Product: 	928Movie - VESA bus

A new memory technology called EDRAM  (Enhanced Dynamic Random Access Memory) is used in some high performance PCs to provide high speed cache memory in place of conventional DRAM.  These PCs have a QuickCache or EDRAM label on the outside and a EDRAM controller chip on the motherboard.

One model of motherboard is the '486-EL' with AMI BIOS, far east origin.

The 928Movie VESA local bus card conflicts with the controller because it uses the same local memory address range as the 928Movie default high linear address.  The result is that Windows does not run and the PC is left on a flashing DOS prompt and requires a re-boot.

The solution is to select the alternative address for 928Movie by editing (in DOS) SYSTEM.INI in the Windows directory as follows:-

find this section at the end of the file:

[928Movie]

HighMemoryBase=0

ApertureBase=0

and change it to:

HighMemoryBase=1

ApertureBase=1

Windows should now run correctly.  If the problem still occurs, then try removing the 928Movie high linear addressing for local bus by removing or commenting out the 2 lines as follows:-

;HighMemoryBase=1

;ApertureBase=1

Windows graphics can run slower when the linear addressing is disabled in this way.  ISA bus 928Movie cards have NO such problem.

Steve Everitt

July, 26