| Field Note: 124 |  | Distribute freely |
| --- | --- | --- |

Subject:  	Accelerated Video for Windows formats

Date Created: 	July 30, 2026

Product: 	928Movie v1.1

This table shows the functionality (Digital Movie Acceleration or SmoothScale) that is available using 928Movie v1.1 for various AVI formats:

| Number of display colours | 16 | 256 | 65k | 16.7M |
| --- | --- | --- | --- | --- |
| Digital Movie Acceleration | None | Full frames 8-bit MS Video 1 8-bit Mediamatics M-JPEG In Full screen only:- MS Video 1 16-bit MS Video 1 24-bit Indeo R2.5 Indeo R3.1 and R3.2 CinePac 1.5 MS RLE | Full frames 8-bit Full frames 16-bit Full frames 24-bit MS Video 1 8-bit MS Video 1 16-bit MS Video 1 24-bit Indeo R2.5 Indeo R3.1and R3.2 CinePac 1.5 and 1.6 Captivator VLV1 Mediamatics M-JPEG | Full frames 8-bit Full frames 16-bit Full frames 24-bit MS Video 1 8-bit MS Video 1 16-bit MS Video 1 24-bit Indeo R2.5 Indeo R3.1 and R3.2 CinePac 1.5 and 1.6 Captivator VLV1 Mediamatics M-JPEG |
| SmoothScale | None | None | Full frames 8-bit Full frames 16-bit Full frames 24-bit MS Video 1 8-bit MS Video 1 16-bit MS Video 1 24-bit Indeo R2.5 Indeo R3.1 and R3.2 CinePac 1.5 and 1.6 Captivator VLV1 Mediamatics M-JPEG | Full frames 8-bit Full frames 16-bit Full frames 24-bit MS Video 1 8-bit MS Video 1 16-bit MS Video 1 24-bit Indeo R2.5 Indeo R3.1 and R3.2 CinePac 1.5 and 1.6 Captivator VLV1 Mediamatics M-JPEG |

Note 1:  Microsoft RLE (8-bit) movies cannot be accelerated or SmoothScaled, except when played in Full-screen mode.  This is due to a Microsoft problem when stretching RLE movies.

Note 2:  Other MCI devices such as Quicktime (.MOV), Fractal, MPEG and Autodesk animation (.FLC or .FLI) movies are not AVI format and cannot be accelerated or SmoothScaled.

Note 3: Motion-JPEG files created using VideoLogic's MediaSpace can be played and accelerated using the MediaMatics codec.  The movie will be jerky if the data rate is too fast for the CPU.

Note 4: MS Video 1 movies are not accelerated or SmoothScaled if they are stretched to exactly x2 (either manually or if the x2 box is checked in Device Configure) in any display mode.

If both the x2 and the Full Screen boxes are checked then MS video 1 movies are slow (although SmoothScaled), compared to just checking the Full Screen box alone.  This is a feature of Video for Windows which attempts to speed up MS Video 1 movies when stretched.

Note 5:  Indeo R3.2 movies require the 3.2 codec and are fully compatible with PowerPlay.  This codec is installed with 928Movie v1.1 and has improved Full-screen performance with Indeo R3.1 movies.

## Digital movie acceleration

This is the ability to stretch a movie to a larger size, even full-screen, without dropping any more video frames than when played at the original size.  928Movie can do this up to 30 frames per second.  It CANNOT invent any more frames than are in the original movie.

## SmoothScale

When a movie is stretched, the normal method is to just repeat pixels to make it larger.  This makes the movie appear blocky because it is easy to see groups of repeated pixels with the same colour.  With SmoothScale, the movie is better quality with no blockiness because the image data is interpolated to put new pixel colours in between existing pixels.

## How to detect the AVI format

Load an .AVI file with Media Player, then select Device Configure to get Information about the movie.  Here are some examples of each format:-

Full frames (uncompressed) 256 colours:

Data Rate: 419k/sec

Video: DIB  180x135x8 (none) 15.004 fps

Keyframe: every frame

Full frames (uncompressed) 65K colours:

Data Rate: 779k/sec

Video: DIB  180x135x16 (none) 15.004 fps

Full frames (uncompressed) 16.7M colours:

Data Rate:   k/sec

Video: DIB  180x135x24 (none) 15.004 fps

Full frames is just a series of bitmaps and so has no keyframes for inter-frame compression.

Microsoft Video 1 (CRAM)  256 colours:

Data Rate: 142k/sec

Video: MSVC 160x116x8 (CRAM) 15.226 fps

Keyframe: every 15 frame(s)

Microsoft Video 1 (CRAM)  65K colours:

Data Rate: 406k/sec

Video: msvc 180x132x16 (CRAM) 15.004 fps

Microsoft Run Length Encoded 256 colours (suitable for animations):

Data Rate: 45k/sec

Video: RLE  160x120x8 (RLE ) 30.000 fps

Keyframe: every 8 frame(s)

SuperMac Cinepak codec v1.5:

Data Rate: 141k/sec

Video: cvid 320x240x24 (cvid) 15.000 fps

Keyframe: every 14 frame(s)

Intel Indeo Video R3.1:

Data Rate: 140k/sec

Video: iv31 320x240x24 (IV31) 15.000 fps

Audio: Mono 11.025KHz 8bit

Keyframe: every 4 frame(s)

Intel Indeo Video R3.2:

Data Rate: 140k/sec

Video: IV32 320x240x24 (IV32) 15.000 fps

Keyframe: every 4 frame(s)

Cinepac and Indeo use YUV encoding to produce full colour movies and are referenced as 24-bit colour.

Intel Indeo Video R2.1:

Data Rate: 129k/sec

Video: RT21 160x120x16 (RT21) 16.130 fps

Keyframe: every 3 frame(s)

Intel Indeo Video R2.1 raw (suitable for capture only):

Data Rate: 638k/sec

Video: yvu9 308x120x24 (YVU9) 15.153 fps

Keyframe: every frame

VideoLogic Captivator YUV (suitable for capture only):

Data Rate: 303k/sec

Video: vlv1 160x120x8 (VLV1) 15.053 fps

Keyframe: every frame

Corel 5 CorelMove RLE format:

Video: RLE  480x360x8 (RLE ) 18.000 fps

Some other examples of different data rates:

Data Rate: 105k/sec

Video: iv31 160x120x24 (IV31) 15.000 fps

Keyframe: every frame

Data Rate: 1400k/sec

Video: DIB  480x360x16 (none) 4.000 fps

Keyframe: every frame

Data Rate: 1330k/sec

Video: DIB  240x180x16 (none) 15.004 fps

Keyframe: every frame

Steve Everitt

July, 26