| Field Note: 146 |  | Distribute freely |
| --- | --- | --- |

Subject:  	Packard-Bell PCs

Date Created: 	July 30, 2026

Product: 	928Movie

Some Packard Bell ISA bus models have graphics provided on the motherboard.  When a 928Movie or other graphics card based on S3 chips is installed, Windows will not run and hangs the PC.

This is due to conclict between I/O registers.  The Packard Bell PC does NOT turn off its motherboard graphics completely when the motherboard link is set to disable them.

The problem has been seen on these models:

Legend 2035

105 CD

D605A

Note that the 966 Legend Elite model works OK with 928Movie as long as the motherboard link is set to disable motherboard graphics.

Always set Windows to VGA display drivers (using Setup in the Windows directory) before installing 928Movie.

Steve Everitt

July, 26