| Field Note: 147 |  | Distribute freely |
| --- | --- | --- |

Subject:  	Using 800x600 resolutions at 56Hz

Date Created: 	July 29, 2026

Product: 	928Movie

The 928Movie packaging and User Manual specify that 800 x 600 resolution (at different colour settings) can run at 56Hz refresh rate.  These modes are NOT available in Windows, but can be selected for DOS applications such as AutoCAD.

A patch is now available, number 89, that sets 56Hz refresh rate when an 800 x 600 60Hz mode is selected in Windows or DOS.  This is for users whose monitors support 56Hz but not 60Hz.  These monitors are typically a few years old and have a horizontal line rate of 35.5kHz maximum.

Steve Everitt

July, 26