| Field Note: 0217 |  | Distribute freely |
| --- | --- | --- |

Subject:  GrafixStar 600 Refresh Utility v1.0

Date Created: July 30, 2026

Product: GrafixStar 600

CONTROL.COM is a program which allows you to alter the refresh for various modes.  This program is designed for DOS operation, and is not necessary in Windows 3.1, Windows 95 and OS/2 drivers due to their built in mechanisms for changing refresh rate control. The program is resident in memory and occupies 1,664 Kbytes of memory space.  At anytime, CONTROL.COM can be released from memory (see instruction below for more information).

Please note: This program is not suitable for use with a fixed frequency monitor.

Supported modes

Refresh rate’s can be selected from the modes listed in the table below:-

| Resolution | Colors |  |  |
| --- | --- | --- | --- |
| 320x200 | 256 | N/A | N/A |
| 640x400 | 256 | 65k | 16.7m |
| 640x480 | 256 | 65k | 16.7m |
| 800x600 | 256 | 65k | 16.7m |
| 1024x768 | 256 | 65k | N/A |
| 1152x864 | 256 | 65k | N/A |
| 1280x1024 | 256 | 65k | N/A |

Background

In order to safely use the refresh rate program, it is vital that you know the following information with regards to your monitor.

The monitors maximum resolution capabilities

Please use resolutions listed from the ‘supported resolutions’ list above.  If your monitor’s maximum resolution capability does not match a resolution on the list, you must select the nearest mode below your monitors maximum capabilities.  eg. For a monitor with a max resolution of 1152x1024, you would select 1152x864.

Maximum Horizontal scan rate

This describes the rate in which the monitor can display pixels on a line.  Otherwise known as the line rate.  This is typically measured in units of thousands of Hertz (kHz).

Maximum Vertical scan rate

This describes the rate in which the monitor can display screens.  Otherwise known as the refresh rate.  This is typically measured in units or Hertz (Hz).

This information is usually available within the documentation that was supplied with your monitor in a section detailing specifications or capabilities.  If this is not available, please contact your monitor supplier for more information regarding this area.

Installation

The refresh rate program can automatically sense the capabilities of a DDC (Plug’n’Play) monitor.  If you monitor is not DDC compliant, you will need to supply some additional data to the refresh rate control program to allow correct operation.  The data required has already been been mentioned above (ie. Max resolution, Max refresh rate, and Max line rate).

Warning: using settings which exceed the limits of your monitor may damage your monitor permanently. Please see the specifications in your monitor documentation or ask your monitor supplier for further details regarding the capabilities of the monitor.

Syntax

CONTROL  [-i] [-u] [-vnnn] [-hnnn]

| where |  |  |
| --- | --- | --- |
| i | Show mode information (Verbose mode) |  |
| u | Remove from memory |  |
| vnnn | Set maximum refresh rate |  |
| hnnn | Set maximum line rate |  |

Note:

The maximum refresh rate input can accept values in the range 60 - 250 Hz.

The maximum line rate input can accept values in the range 28 - 150kHz

(or 28000 - 150000 Hz).

For example: to setup the refresh rate control program to use a monitor capable of 48kHz and a maximum refresh rate of 90Hz, use the following command line.

CONTROL -i -v90 -h48

or

CONTROL -i -v90 -h48000

The -i option can be omitted from the command line, but proves useful for mode

information.

Uninstall

To uninstall the refresh rate control program, use the [-u] option.  This unloads the program from memory.  The refresh rate control program can also be removed by rebooting your system.

For example: to uninstall the refresh rate control program from system memory, use the following command line.

CONTROL -u

Please Note: Once the refresh rate control program is loaded with monitor settings, to redefine the settings, the program needs to be uninstalled before the new settings are used.

VideoLogic Support Team

July, 26