| System Unit | Dimensions (HxWxD) | 1.49 x 11.4 x 10.2 in/3.78 x 28.96 x 25.95 cm |  |
| --- | --- | --- | --- |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Weight |  |  |
|  | Standard Configuration Slimline Configuration Multimedia Configuration | 6.2 lb/2.83 kg 5.3 lb/2.42 kg 8.3 lb/3.81 kg |  |
|  | Stand-Alone Power Requirements |  |  |
|  | Nominal Operating Voltage | 14.2 VDC |  |
|  | Maximum Operating Power | 35 W |  |
|  | Peak Operating Power | 40 W |  |
|  | Temperature |  |  |
|  | Operating | 5 to 95° F/10 to 35° C |  |
|  | Nonoperating | -4 to 140° F/-20 to 60° C |  |
|  | Relative Humidity (noncondensing) |  |  |
|  | Operating | 10 to 90% |  |
|  | Nonoperating | 5 to 95% |  |
|  | Shock |  |  |
|  | Operating | 10 G, 11 ms, half sine |  |
|  | Non operating | 60 G, 11 ms, half sine |  |
|  | Vibration |  |  |
|  | Operating | .25 G, 5-500 Oct/Min sweep rate |  |
|  | Nonoperating | 1.0 G, 5-500 Oct/Min sweep rate |  |
|  | Maximum Altitude (unpressurized) |  |  |
|  | Operating | 10,000 ft/3.15 km |  |
|  | Nonoperating | 30,000 ft/9.14 km |  |
| 12.1-Inch Color TFT SVGA Display | Dimensions (HxW) Diagonal Size | 7.26 x 9.68 in/18.45 cm x 24.6 cm 12.1 in/30.7 cm |  |
|  | Mounting | Internal Panel Supports |  |
|  | Number of Colors | 64K Colors |  |
|  | Contrast Ratio | Greater than 100:1 |  |
|  | Brightness | 100 cd/m2 |  |
|  | Pixel Resolution |  |  |
|  | Pitch | .3075 x .3075 mm |  |
|  | Format | 800 x 600 |  |
|  | Configuration | RGB Stripe |  |
|  | Backlight Character Display |  | Edge Lit — |
|  | Total Power Consumption |  | 3.9 W |
|  | Display Inverter Board |  |  |
|  | Input Voltage |  | 10-20 Vdc |
|  | Output Voltage |  | 600 Vrms |
|  | Maximum Input Power |  | 3.2 W |
|  | Maximum Output Power |  | 2.5 W |

| 12.1-Inch Color STN Display | Dimensions (HxW) Diagonal Size |  |  |  |  | 7.26 x 9.68 in/18.45 x 24.6 cm 12.1 in/30.7 cm |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Mounting |  |  |  |  | Internal Panel Supports |  |  |  |  |  |  |  |
|  | Number of Colors |  |  |  |  | 256 |  |  |  |  |  |  |  |
|  | Contrast Ratio |  |  |  |  | Greater than 30:1 |  |  |  |  |  |  |  |
|  | Brightness |  |  |  |  | 75 cd/m2 @ 4.0 mA |  |  |  |  |  |  |  |
|  | Pixel Resolution |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pitch |  |  |  |  | .3075 x .3075 mm |  |  |  |  |  |  |  |
|  | Format |  |  |  |  | 800 x 600 |  |  |  |  |  |  |  |
|  | Configuration |  |  |  |  | RGB Stripe |  |  |  |  |  |  |  |
|  | Backlight |  |  |  |  | Edge Lit |  |  |  |  |  |  |  |
|  | Character Display |  |  |  |  | — |  |  |  |  |  |  |  |
|  | Total Power Consumption Display Inverter Board |  |  |  |  | 3.9 W |  |  |  |  |  |  |  |
|  | Input Voltage |  |  |  |  | 10-20 Vdc |  |  |  |  |  |  |  |
|  | Output Voltage |  |  |  |  | 600 Vrms |  |  |  |  |  |  |  |
|  | Maximum Input Power |  |  |  |  | 3.2 W |  |  |  |  |  |  |  |
|  | Maximum Output Power |  |  |  |  | 2.5 W |  |  |  |  |  |  |  |
| SMART Hard Drives1 |  |  |  | 1.6-GB |  |  | 2.0-GB |  | 3.2-GB |  |  | 4.0-GB |  |
|  | Formatted Capacity Per Drive |  |  | 1.6 GB |  |  | 2167.6 GB |  | 3.3 GB |  |  | 4.1 GB |  |
|  | Drive Type |  |  | 65 |  |  | 65 |  | 65 |  |  | 65 |  |
|  | Transfer Rate |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Media |  |  | 36.3 to 67.4 Mb/s |  |  | 61.6 to 39.1 Mb/s |  | 51.7 to 83.4 Mb/s |  |  | 51.7 to 83.4 Mb/s |  |
|  | Interface |  |  | 16.67 MB/s |  |  | 16.6 MB/s |  | 16.6 MB/s |  |  | 16.6 MB/s |  |
|  | Sector Interleave |  |  | 1:1 |  |  | 1:1 |  | 1:1 |  |  | 1:1 |  |
|  | Seek Times (typical, including settling) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Track-to-track |  |  | 3 ms |  |  | 4 ms |  | 4 ms |  |  | 4 ms |  |
|  | Average |  |  | 12 ms |  |  | 13 ms |  | 13 ms |  |  | 13 ms |  |
|  | Maximum |  |  | 24 ms |  |  | 23 ms |  | 23 ms |  |  | 23 ms |  |
|  | Disk Rotational Speed |  |  | 4000 rpm |  |  | 4006 rpm |  | 4009 rpm |  |  | 4009 rpm |  |
|  | Physical Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Cylinders |  |  | 5158 |  |  | 4975 |  | 6975 |  |  | 6975 |  |
|  | Heads |  |  | 4 |  |  | 6 |  | 5 |  |  | 6 |  |
|  | Sectors per track |  |  | 108 to 198 |  |  | 110 to 180 |  | 144 to 240 |  |  | 144 to 240 |  |
|  | Bytes per sector |  |  | 512 |  |  | 512 |  | 512 |  |  | 512 |  |
|  | Logical Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Cylinders |  |  | 3162 |  |  | 4200 |  | 6304 |  |  | 7944 |  |
|  | Heads |  |  | 16 |  |  | 16 |  | 16 |  |  | 16 |  |
|  | Sectors per track |  |  | 63 |  |  | 63 |  | 63 |  |  | 63 |  |
|  | Bytes per sector |  |  | 512 |  |  | 512 |  | 512 |  |  | 512 |  |
|  | Buffer Size |  |  | 128 KB |  |  | 128 KB |  | 468 KB |  |  | 468 KB |  |

| Diskette Drive . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Diskette Size | 3.5-inch |
| --- | --- | --- |
|  | Light | None |
|  | Height | .43 in/1.1 cm |
|  | Bytes per Sector | 512 |
|  | Sectors per track |  |
|  | High density | 18 (1.44-MB)/15 (1.2-MB) |
|  | Low density | 9 |
|  | Tracks per side |  |
|  | High density | 80 (1.44-MB)/80 (1.2-MB) |
|  | Low density | 80 |
|  | Read/Write heads | 2 |
|  | Access Times |  |
|  | Track-to track (high/low) | 3 ms/6 ms |
|  | Average (high/low) | 94 ms/174 ms |
|  | Settling time | 15 ms |
|  | Latency average | 100 ms |
| AC Adapter | Dimensions (HxWxL) | 1.42 x 2.83 x 4.69 in/3.6 x 11.9 x 7.2 cm |
|  | Weight | .66 lb/.3 kg |
|  | Power Supply (Input) Nominal Voltage Current Voltage Range Input Frequency | 120 VAC <1.11 A 90-265 VAC 47 to 63 Hz |
|  | Power Supply (Output) Nominal Voltage Current | +18.5 +/- 0.25 VDC 40 to 42 W |

|  | Temperature Normal Operating Operating Nonoperating | 140°F/60°C 41 to 104°F/5 to 40°C -4 to 185°F/-20 to 85°C |
| --- | --- | --- |
|  | Relative Humidity Operating Nonoperating | 10 to 95% 10 to 95% |
| Lithium Ion Modular Battery Pack . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Dimensions (HxWxL) Weight | .77 x 4.35 x 5.34 in/1.9 x 11.0 x 13.6 cm .88 lb/.39 kg |
|  | Energy |  |
|  | Voltage | 14.4V |
|  | Amp-hour capacity | 2.8 Ah |
|  | Watt-hour capacity | 40 Wh |
|  | Temperature Operating Nonoperating | 50° to 104° F/10° to 40° C -12° F to 140°F/-30° to 60°C |

| Lithium Ion Handle Battery Pack | Dimensions (HxWxL) Weight | .89 x 1.53 x 11 in/1.52 x 3.9 x 27.9 cm .88 lb/.39 kg |
| --- | --- | --- |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Energy Voltage Amp-hour capacity Watt-hour capacity | 14.4 V 2.8 Ah 40 Wh |
|  | Temperature Operating Nonoperating | 50° to 104° F/10° to 40° C -12° F to 140°F/-30° to 60°C |
| 20X Max2 Mobile CD Unit | Dimensions (HxWxD) | .86 x 11.69 x 8.95 in/2.18 x 29.7 x 22.7 cm |
|  | Weight | 2.0 lb/.91 kg |
|  | Power Supply (Input) Operating Voltage Operating Current Rated Voltage Rated Current Line Frequency | 100 to 240 VAC 1.10 Amp Maximum 100 to 240 VAC 1.0 Amp Maximum 47 to 63 Hz |
|  | Temperature Operating Nonoperating | 50 to 95°F/10 to 35°C -4 to 140°F/-20 to 60°C |
|  | Relative Humidity Operating Nonoperating | 20 to 80% 5 to 90% |
|  | Altitude Operating Nonoperating | 10,000 ft/3.15 km 30,000 ft/9.14 km |
|  | Shock Operating Nonoperating | 5 G, 11 ms, half sine 140 G, 2 ms, half sine |
|  | Vibration Operating Nonoperating | 0.25 G, 5 to 500 Hz, 0.5 octave/min sweep rate 0.75 G, 5 to 500 Hz, 0.5 octave/min sweep rate |
|  | Compatible Disc Formats | CD-ROM (Mode 1, 2 and 3) CD-DA CD-XA ready (Mode 2 Form 1, Form 2) CD-I ready (Mode 2, Form 1 and 2) CD-R (read only) CD Plus Photo CD (single and multi-session) CD Extra |
|  | Center Hole Diameter | 15 mm |
|  | Disc Diameter | 12 cm, 8 cm |
|  | Disc Thickness | 1.2 mm |
|  | Track Pitch | 1.6 µm |

| 20X Max2 CD-ROM Drive (included with Mobile CD Unit) (cont’d) | Laser Beam divergence Output power Type Wave length |  | 53.5 +/- 1.5° 0.13 +/- .1 mw Semiconductor Laser GaA1As 780 +/- 25 nm |
| --- | --- | --- | --- |
|  | Access Times (typical, including settling) Random |  | < 150 ms |
|  | Full stroke |  | < 600 ms |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. | Audio Output Level Line out Headphone |  | 0.7 Vrms none |
|  | Cache Buffer |  | 128 KB |
|  | Data Transfer Rate (typical, including settling) Sustained |  | 3.0 MB/s, 20X Outer Diameter |
|  |  |  | 150 KB/s, single |
|  | Asynchronous Burst |  | 8.3 MB/s |
|  | Startup Time |  | < 10 seconds |
|  | 1 Hard drive specifications may vary slightly depending on vendor. 2 Data transfer rates will vary from 1276 KB/s to 3000 KB/s. |  |  |
|  | Compaq, registered United States Patent and Trademark Office. Armada, ArmadaStation, MiniStation, MultiLock, QuickBlank, QuickLock, are trademarks and/or service marks of Compaq Computer Corporation. Other product names mentioned herein may be trademarks and/or registered trademarks of their respective companies. ©1998 Compaq Computer Corporation. All rights reserved. Printed in the U.S.A. |  |  |