Compaq Storage Works TL891/TL892 Series DLT Tape Storage Library Libcenter Application Readme.

Thank you for downloading Compaq’s TL891/TL892 series firmware. This firmware is used to update the TL890, TL891, TL892, TL891DLX, and TL892DLX DLT Tape Storage Libraries.

- Installing the Libcenter Application

For Windows based systems using Windows 95, 98, NT or 2000.

Copy the self-extracting zip file to your hard drive.

Double click on the file to expand the file to a directory

( you may want to create a c:\Libcenter directory).

Click “next” to specify the target directory.

Click “next” to extract the files to the directory. Once the

files are extracted go to the directory and double click on the

setup .exe file. This will guide you through the installation

process.

- Starting the Libcenter Application

Connect the db9 male serial cable to an available COM port on the host computer. Connect the female db9 connector to the serial port on the library.

Start the utility by opening the Libcenter directory and double click on the Q

Icon.

The main Libcenter screen will appear, choose the “connect” box.

Select the appropriate Com port and baud rate (38400).

- Downloading Firmware to the Library

Choose the “Download”.

The “Downloading data to the library “dialog box will appear.

A “Download Menu” will appear.

Choose “Flash Image” to download firmware.

Choose the name and location of the firmware image file to

Download.

Choose “ok” to download firmware.

Follow the instructions in the dialog box to enable the firmware

in the library.

Choose “Yes” when the library is attempting to sync with the host.

The firmware Download should proceed and the library will reboot

upon successful completion.