CONTROLLER SPECIFICATIONS

RISC Processor........................Intel i960RD

Speed.................................66MHz

PCI...................................2.1

I2O...................................Compliant

ECC EDO RAM...........................4MB to 64MB

JEDEC SIMM............................72 pin

Flash.................................1MB

Hardware XOR..........................Yes

ECC Memory Support....................Yes

PHYSICAL SPECIFICATIONS

PCI card..............................Half length

Power Requirements....................+5.0v (+-5%) 3.5 A (max); 17.5 watts

Temperature...........................0 to 55є C

MTBF..................................200,000 hrs.

Warranty..............................Three years

Certification.........................FCC Class B, CE, UL, PC 98 (planned),Windows NT, Novell

SCSI SPECIFICATIONS

- Supports SCSI1, SCSI2, Ultra SCSI, and Ultra2 SCSI devices

- Full support for Low Voltage Differential (LVD) devices

- Up to 80MB/sec burst data rate

- Automatic SCSI bus termination

- Disconnect/Reconnect

- 64 concurrent host commands

- 256 tagged commands per controller

- Automatic sector remapping

- Ultra2 SCSI (LVD) cable length up to 25 meters for two devices or 12 meters for 15 devices

- One Wide Ultra2 SCSI LVD channel

- Up to 15 devices per channel

- Up to 2.70TB per system (depending on configuration; multiple controllers and SCSI channels on system board)

RAID FAULT TOLERANT AND FLEXIBILITY FEATURES

- RAID levels 0, 1, 0+1, 3, 5,10, 30, 50, JBOD

- Transparent disk drive rebuild

- Up to eight controllers per system (NT)

- Logical drive size of 2TB, 32 logical drives per controller

- Immediate RAID Availability; background initialization

- Variable stripe size per controller

- Variable cache line size per controller

- Up to eight non-disk devices per controller

- Drive traveling during power off

- SAF-TE enclosure management

- Automatic error recovery

- Write through or write back cache support/logical device

- Automatic detection of failed drives

- Automatic rebuild of hot spare drive

- Hot drive swapping support

- ECC detection/correction on cache memory

- Support for SMART capable drives

- Configuration data stored on disk and NVRAM

RAID MANAGEMENT

- Global Array Manager

- Client server GUI interface

- Create, manage, expand and monitor programs

- Error statistics and performance information

- BCU BIOS config utilities - RAID Manager

SYSTEM DRIVER SUPPORT

- Windows NT server 4.0 and enterprise edition

- NetWare 4.11 & above, 5.x and SMP

- SCO UnixWare 2.1, 7.0, UNIX OSR 5.x

- DOS 5.x, 6.x and above

AcceleRAID 250

DACPTL-1-xx

DACPTLM-1-8E (Distrib. Mod.)

RAID adapter with Ultra2 SCSI; also uses SCSI channels on system board