Intel AL440LX Software and Manual CD

This CD has four directories:

- Acrobat - Contains the Adobe Acrobat software that is needed to view the product warranty, the Product Guide (manual) and other documentation on this CD

- Manuals - Contains the Product Guide in multiple languages

- Software - contains a number of subdirectories.  The subdirectories contain audio drivers, LANDeskÔ Client Manager (LDCM) software and a software utility that adapts Windows* 95 .INF files so that the operating system recognizes the PIIX4 component on the AL440LX.

- Warranty - contains the product warranty

The contents of these directories and their subdirectories are described below.

ACROBAT, MANUALS and WARRANTY

These directories contain the product documentation, including the Product Guide and the product warranty.  The AL440LX Motherboard Product Guide and product Warranty are both provided in Adobe Acrobat .PDF format.  The Adobe Acrobat Reader is included on this CD and must be installed before you can view the .PDF documents.

INSTALLATION INSTRUCTIONS FOR ADOBE ACROBAT READER IN WINDOWS 3.1

- In File Manager, click on File, then Run.

- At the command line within the dialog box enter the drive letter of the CD ROM followed by \Acrobat\16bit\ar16e30.exe (e.g.: D:\Acrobat\16bit\ar16e30.exe).  Press enter.

- Follow the directions in the ar16e30 setup program.

INSTALLATION INSTRUCTIONS FOR ADOBE ACROBAT READER IN WINDOWS 95 or WINDOWS NT

- In File Manager, click on File, then Run.

- At the command line within the dialog box enter the drive letter of the CD ROM followed by \Acrobat\32bit\ar32e30.exe (example: D:\Acrobat\32bit\ar32e30.exe). Press enter.

- Follow the directions in the ar32e30 setup program.

After the Adobe Acrobat installation is complete, view the product documentation and warranty information by opening the file present in the appropriate language directory under the Manuals or Warranty directory on the CD.

SOFTWARE

The Software directory contains subdirectories that include the audio drivers for the Yamaha OPL3-SA3 sound system (Audio), LANDesk Client Manager (LDCM) and the ISA Configuration Utility (Dosicu, Winicu).

AUDIO DRIVERS

This subdirectory contains the Yamaha Audio drivers in Windows NT, Windows 95, Windows 3.11 and OS/2.  Directions for installing these drivers is shown below as well as in README files along with the audio drivers.

INSTALLATION INSTRUCTIONS FOR WINDOWS NT 4.0

Note:  this installation assumes that you have correctly installed Windows NT on your computer and that your system is able to boot and run without errors.  If not, refer to the Windows NT User Manual for help before continuing.

- Make a DOS diskette; (e.g. format a:).

- On the CD, change into the \Software\Audio\WinNT directory.

- Before expanding the drivers you must first read and agree to the end-user license agreement enclosed in the file LICENSE.TXT included with the SW.EXE file.

- Expand drivers from CD to the floppy by using the following command from the DOS prompt: SW -D a:

- From the Windows NT desktop, click on the START button, choose Settings, then Control Panel.

- Double click on the Multimedia icon, then select the Devices tab.

- Click on the Add button, select "Unlisted or Updated Driver", and click on OK.

- When the Install Driver window appears, insert your the Yamaha Driver diskette you just created and type the drive letter of the diskette drive, then click OK.

- When the Add Unlisted or Updated Driver window appears, select the OPL3-SA Sound System then click on OK.

- If you are reinstalling the drivers, you will see a Driver Exists screen that asks you if you would like to use the current driver or install a new driver. Click on the New button to continue.

- At this point you will see a Yamaha OPL3-SA Series Driver window.  If you want to change the settings for the sound card, select the drop down menu next to I/O Address or IRQ and select your new resources.  If you wish to change the resources for this device, select the new setting from the drop down menus labeled I/O Address, and IRQ.  If you wish to change the DMA channel, select the radial button next to the resource you want to use, (choices are DMA 0, 1, and 3).

- When you are finished, click on OK. You will now see a System Setting Change window.

- Select Restart Now to finish the installation and reboot the system.

Note:  During the reboot you should hear the NT logon sound after you log in.  If you do not, check for resource conflicts using the Windows NT Diagnostics utility, located in the Administrative Tools folder.

INSTALLATION INSTRUCTIONS FOR WINDOWS NT 3.51

This installation assumes that you have correctly installed Windows NT on your computer and that your system is able to boot and run without errors.  If not, refer to the Windows NT User Manual for help before continuing.

- Make a DOS diskette; (e.g.  format a:)

- On the CD, change into the \Software\Audio\WinNT directory.

- Before expanding the drivers you must first read and agree to the end-user license agreement enclosed in the file LICENSE.TXT included with the SW.EXE file.

- Expand drivers from CD to the diskette by using the following command from the DOS prompt:  SW -D a:

- From the Windows NT Program Manager, select the Main icon, then double click on Control Panel.

- Select the Drivers icon, click on the Add button, select "Unlisted or Updated Driver", then click on OK.

- When the Install Driver window appears, insert the Yamaha Driver diskette you just created, type the drive letter of the diskette drive, then click OK.

- When the Add Unlisted or Updated Driver window appears, select the OPL3-SA Sound System, and click on OK.

- If you are reinstalling the drivers, you will see a Driver Exists screen that asks you if you would like to use the current driver or install a new driver. Click on the New button to continue.

- At this point you will see a Yamaha OPL3-SA Series Driver window.  If you want to change the settings for the sound card, select the drop down menu next to I/O Address or IRQ and select your new resources. If you wish to change the resources for this device, select the new setting from the drop down menus labeled I/O Address, and IRQ.  If you wish to change the DMA channels, select the radial button next to the resource you want to use, (choices are DMA 0, 1, and 3).

- When you are finished, click on OK.

- You will now see a System Setting Change window, select Restart Now to finish the installation and reboot the system.

Note:  During the reboot you should hear the Windows sound after you log in.  If you do not, check resource conflicts using the Windows NT Diagnostics utility located in the Administrative Tools folder.

INSTALLATION INSTRUCTIONS FOR OPL3-SA3 ON RETAIL WINDOWS 95

- Make a DOS diskette and label it Disk1; (e.g.  format a:)

- On the CD, change into the \Software\Audio\Win95 directory.

- Before expanding the drivers you must first read and agree to the end-user license agreement enclosed in the file LICENSE.TXT included with the SW.EXE file.

- Expand drivers from CD to the diskette by following these steps:

- With the diskette labeled Disk1 in drive a:, at the DOS prompt change directories to the disk1directory on the CD.

- Type the following command at the DOS prompt:  SW a:

- From the Windows 95 desktop, click on the START button, choose SETTING, then CONTROL PANEL.

- Double click on the SYSTEM icon and select the DEVICE MANAGER tab.

- If you see an entry labeled OTHER DEVICES, double click on it so that this category drops down.  If there is an entry called OPL3-SA2 or OPL3-SA3 SOUND CARD, highlight it and click on REMOVE.

- Scroll down the device list and double click on SOUND, VIDEO AND GAME CONTROLLERS so that it drops down.  You will then see a list of devices that have been previously installed.  Select any previously installed Sound Cards and Gameports, then click on REMOVE for each one.

- Select OK to close DEVICE MANGER and close CONTROL PANEL.

- Insert the diskette labeled DISK1 into floppy drive a:.

- Select START | RUN and type  A:\SAUNINST.EXE and press OK to continue. (Note: B:\SAUNINST.EXE may be substituted if you are using the b: drive for your installation).

- At the first OPLSAx screen select NEXT.  Click on NEXT again to continue.

- When you see the screen "Uninstall Completed" click on FINISH.

- Remove your diskette and click on OK to restart the computer.

- Your computer will now restart.  The first screen you will see should display "NEW HARDWARE FOUND" which will refer to the OPL3-SA3.

- Insert the diskette you created labeled Disk1 into your a: drive.

- Select "Driver from disk provided by hardware manufacturer" and click on OK to continue.

- At the next screen (Install from diskette) enter either a: or b: depending on which diskette drive you are using. Then click on OK to continue.

- After the files have been copied to your hard drive, you may receive the prompt "Please insert the disk labeled Windows 95 CD-ROM and then click OK”.  If you receive this message, place your Windows 95 CD-ROM in the CD-ROM carrier.

- Click on OK at this point, enter D:\WIN95 (or using whatever CD-ROM driver letter is appropriate for your system) and click on OK to continue.

- If the sound produced is not clear or your sound does not work properly, choose START | SHUTDOWN | RESTART YOUR COMPUTER and click on Yes.

- Your computer will reboot and you should hear the Windows 95 sound as the computer reboots.

INSTALLATION INSTRUCTIONS FOR OPL3-SA3 ON OEM WINDOWS 95

- Make a DOS diskette labeled Disk1; (e.g. format a:).

- On the CD, change into the \Software\Audio\Win95 directory.

- Before expanding the drivers you must first read and agree to the end-user license agreement enclosed in the file LICENSE.TXT included with the SW.EXE file.

- Expand drivers from CD to a diskette by following these steps:

- With the diskette labeled Disk1 in drive a:, at the DOS prompt change directories to the \Software\Audio\Win95 directory on the CD.

- Type the following command at the DOS prompt:  SW a:

- During the installation, Windows 95 detects the audio as OPL3-SA3 Sound System and prompts the user that the Update Device Driver Wizard will complete the installation the drivers.

- Insert the diskette labeled Disk1containing the audio device driver.  Click on next.

- Windows 95 then prompts that the Yamaha OPL3-SAx Sound System has been found.  Click on Finish.  Repoint to the diskette drive if necessary.

- If prompted by Windows 95 please insert the Windows 95 CD to finish installation of the joystick driver.

OVERINSTALL INSTRUCTIONS FOR WINDOWS 95

If your system has an earlier version of the Yamaha Sound System device drivers:

- Under Windows 95 run "SAuninst.exe" from the driver diskette labeled Disk1.

- When prompted, restart your system and remove the diskette.

- After reboot, if prompted, insert the driver diskette.

- If not prompted to insert the driver diskette, check the device manager, remove any entries pertaining to Yamaha and reboot.  Follow the instructions on how to install the device drivers above.

PLAYING SOFTWARE WAVETABLE

- Click Start, point to Settings, and click Control Panel.

- Double-click the Multimedia icon.

- Choose the MIDI tab.

- Select Yamaha OPL3SA SoftSynth and click OK.

- Open a media player and play a MIDI file.  Use the Wave device section of the mixer to control the volume and the balance of the software wavetable music.

- Click Start, point to Settings, and click Control Panel.

- Double-click the OPLSoftSynth Icon and change the Quality setting to improve the sound quality.

INSTALLATION INSTRUCTIONS FOR WINDOWS 3.11

- Make a DOS diskette; (e.g. format a:)

- On the CD, change into the \Software\Audio\Win311 directory.

- Before expanding the drivers you must first read and agree to the end-user license agreement enclosed in the file LICENSE.TXT included with the SW.EXE file.

- Expand drivers from CD to the floppy by typing the following command at the DOS prompt:  SW a:.

- Exit Windows and insert the diskette you just created into a diskette drive.

- At the DOS Prompt, type x:\install, where x is the letter of the diskette drive.

- The installation program will then prompt for creation of a temporary directory.  Respond <Y>es.

- Next, the installation program will prompt for an install directory.  Use the default directory or enter a new one and press return.  Follow the installation prompts and answer the questions as appropriate.

- When the configuration window appears, set the various resources to available values.  A common one for the OPL3-SA3 is listed on the left and one for the OPL3-SA is listed on the right, below :

OPL3-SA3

Sound Blaster

Base I/O Adr  220h

IRQ           5

DMA Channel   1

WSS

WSS Base Adr  530h

IRQ           5

DMA Channel   0

Other Device

MODEM I/O     OFF

MODEM IRQ     OFF

IDE1 I/O      OFF

IDE2 I/O      OFF

IDE IRQ       OFF

MPU, Ctrl, Adlib, Game

MPU base I/O  330h

Ctrl I/O      370h

Adlib         388h

Game Port     201h

Enter OK when the resource configuration has been completed and respond <n>o when prompted to run the resource configuration editor during boot.  When prompted, remove the temporary directory by responding <Y>es.

INSTALLING THE CD AUDIO DRIVER

- In Windows 3.11, open the Main program group.

- Double-click on the Control Panel icon and then the Drivers icon.

- Click on Add and select the [MCI] CD Audio, then click OK.

PLAYING SOFTWARE WAVETABLE

- In Windows 3.11, open the Main program group.

- Double Click on the Control Panel applet.

- Double Click on Midi Mapper.

- Select OPL3-SA SYNTH and click on it.

- Click on the close button.

- Double Click on OPLSoftSynth.

- Set the Quality, Reverb, and Filter values.  Click on the OK button.

- Open Media Player or the Yamaha Y-Station application and load a Midi file.

- If using Media Player, click on the Device Menu and select the Midi Sequencer.

- Open a Midi file.  You are now ready to play Midi files using the software wavetable.  Click on the play button in Media Player.

CONTROLLING YMERSION (3D SPATIALIZER)

- Double Click on the YAMAHA program group.

- Double Click on the YAMAHA STATION applet.

- Click on the 3D button at the top right hand corner of the Audio Rack.

- Set the 3D effects to either one of the Presets, Manual or off.

- If set to manual, a mixer panel will appear.  Adjust the Bass, Treble, and Ymersion sliders for effect.

- Click on the menu bar and select close to close.

- Play a sound in the Yamaha Station application and the sound will be enhanced.

INSTALLATION INSTRUCTIONS FOR OS/2 WARP VERSION 3:

- Make a DOS diskette; (i.e. format a:)

- On the CD, change into the \Software\Audio\OS2 directory.

- Before expanding the drivers you must first read and agree to the end-user license agreement enclosed in the file LICENSE.TXT included with the SW.EXE file.

- Expand drivers from CD to the diskette by typing: SW a:

- When installing OS/2 be sure to install the Multimedia Applications.  Be sure that your BIOS settings are set to NOT use a PnP OS if using Warp 3.0.  OS/2 will autodetect the Yamaha device as a Sound Blaster* Pro.  Do NOT install this device;  remove it from the setup.  The Yamaha driver will be installed as part of the Multimedia Application Install.

- Open the Multimedia Folder located on the Desktop.

- Within the Multimedia Folder open the Multimedia Application Install Icon.

- Insert the driver diskette you just created and change the source drive to read from the diskette.

- Highlight and select the Yamaha OPL3-SA Series Driver, then click Install.

- OS/2 will then prompt you that the CONFIG.SYS will be changed, select YES.

- Enter 1 for the number of Yamaha OPL3-SA Series Audio devices, then click OK.

- Set your resource configuration to match your system.

- TYPICAL VALUES FOR THE OPL3-SA3 ARE:

- Interrupt Level:			IRQ 7

- WSS DMA Channel:			0, 1

- WSS base I/O Port:			530h

- FM Synthesizer Base I/O Port:		388h

- MPU401 Base I/O Port:		Disable

- MPU401 Interrupt Level:		SA2/SA3

- Control Register Base I/O Port:		390h(SA2 or SA3)

- SoundBlaster Register Base I/O Port:	Enable(I/O:220h)

- Game Port Supporting:			Enable(I/O:201h)

- Game Port Supporting(Joystick Driver):	Enable(I/O:201h)

- Click OK when finished.

- To enable support for OPL4-ML (Hardware wavetable) or external MIDI devices, set MPU401 Support to enabled.  If no Hardware Wavetable devices or External MIDI devices are present set the MPU401 Base I/O to disabled default settings.  OS/2 will then prompt that the changes have taken place.

- Reboot if necessary.

INSTALLATION INSTRUCTIONS FOR OS/2 WARP VERSION 4:

- Make a DOS diskette; (i.e. format a:)

- On the CD, change into the \Software\Audio\OS2 directory.

- Before expanding the drivers you must first read and agree to the end-user license agreement enclosed in the file LICENSE.TXT included with the SW.EXE file.

- Expand drivers from CD to the diskette by typing: SW a:.

- When installing OS/2 be sure to install the Multimedia Applications.  OS/2 will autodetect the Yamaha device as a Sound Blaster Pro.  Do NOT install this device; remove it from the setup.  The Yamaha driver will be installed as part of the Multimedia Application Install.

- Once OS/2 is loaded double click on the OS/2 System icon on the Desktop.

- Double click the System Setup Icon.

- Double click the Install/Remove Icon.

- Double click the Multimedia Application Install Icon.

- Insert the driver diskette you just created and change the source drive to read from the diskette.

- Highlight and select the Yamaha OPL3-SA Series Driver, then click Install.

- OS/2 will then prompt you that the CONFIG.SYS will be changed; select YES.

- Enter the number of Yamaha OPL3-SA Audio devices to be 1, then click OK.

- Set your resource configuration to match your system.

- TYPICAL VALUES FOR THE OPL3-SA3 ARE:

- Interrupt Level:				IRQ 7

- WSS DMA Channel:			0, 1

- WSS base I/O Port:			530h

- FM Synthesizer Base I/O Port:		388h

- MPU401 Base I/O Port:			Disable

- MPU401 Interrupt Level:			SA2/SA3

- Control Register Base I/O Port:		390h(SA2 or SA3)

- SoundBlaster Register Base I/O Port:	Enable(I/O:220h)

- Game Port Supporting:			Enable(I/O:201h)

- Game Port Supporting(Joystick Driver):	Enable(I/O:201h)

- Click OK when finished.

- To enable support for OPL4-ML (Hardware wavetable) or External MIDI devices MPU401 Support must be set to enabled.  If no Hardware Wavetable devices or External MIDI devices are present please select the MPU401 Base I/O disabled default settings.  OS/2 will then prompt that the changes have taken place.

- Reboot if necessary.

Installing the Wide Control Application

- Double click on the OS/2 system icon located on the Desk Top.

- Double click on the Drives Icon.

- Insert the drivers diskette into your diskette drive.

- Highlight the OPL3DEMO.EXE applet by clicking on it.

- Right mouse click on OPL3DEMO.EXE, then select copy.

- Select Desktop, then click the Copy button.

- Run the application by double-clicking the OPL3DEMO.EXE icon.

- Control the volume of Wide/Treble/Bass by moving the slider-bar.

OPTIONS

When installation successfully completes, the following lines are added to CONFIG.SYS:

DEVICE=[MMOS2 dir]\OPL3SA.SYS (*)

DEVICE=[MMOS2 dir]\GAMEDD.SYS (*)

DEVICE=[MMOS2 DIR]\OPL3VDD.SYS (*)

DEVICE=[MMOS2 DIR]\GAMEVDD.SYS

where (*) is:

/I:xx:5,7,9,10,11			'Interrupt for Windows Sound System

/D:xx,yy:0,1,3			'DMA channel for Windows Sound System

/W:xxx:530,604,E80,F40		'Base I/O address for Windows Sound System

/F:xxx:388,398,3A8,3B8		'Base I/O for OPL3

/M:xxx:330,320,310,300		'Base I/O address for MPU401

/C:xxx:F86,390,620,910,A10	'Base I/O address for control registers

/N:xxx				'Name of the device driver

/A:xx:5,7,9,10			'Interrupt for MPU401 (0 for SA2/SA3)

/S:xxx:0 disable,220,240,260	'Base I/O address for Sound Blaster

/J:x,yyy  x:0(Disable), 1(Enable)	'Control for Joystick

yyy: 201,20A,20E

LDCM  (LanDesk Client Manager)

The LDCM directory contains LANDesk Client Manager 3.0, a software program designed by Intel to provide system managability using the Desktop Management Interface.  LDCM supports Windows 95, Windows NT 3.51, and Windows NT 4.0.

LDCM 3.0 provides four key areas of support for management of systems built with the AL440LX motherboard:

- PC Health Monitor: alerts for PC conditions along with display of recommendations for addressing problematic conditions

- Categorical View:  14 icons, each of which can be used to display system attributes and useful system information

- DMI Browser:  provides a tabbed view of component/group/attribute from the DMI service layer database

- General System Controls:  provides features such as access security (user defined), system file snapshots (read/write), remote file transfer and remote reboot

Due to the architecture of the AL440LX motherboard, LDCM 3.0 will not provide the hardware monitoring functions available on previous motherboards.  Thus, the following capabilities will not be available:  motherboard temperature monitoring, chassis and processor fan monitoring, power supply voltage monitoring.  These hardware monitoring features will once again be available with LDCM 3.1, expected in November 1997.   If you need these features ask your system integrator to contact Intel Customer Support for copies of LDCM 3.1 when it is available.

INSTALLATION INSTRUCTIONS FOR WINDOWS 95:

Before installing the LDCM software you must first read and agree to the end-user license agreement enclosed in the file LICENSE.TXT or LICENSE.DOC included in root directory of this CD.

In Windows, click on Start, then Run.  In the command line within the dialog box enter the CD ROM drive letter followed by \software\LDCM\win95\local\setup.exe (e.g.: D:\software\win95\local\setup.exe). Press enter.

Follow the directions in Setup.

If you are installing this on an Administrator's computer, you may additionally upgrade to the Adminstrator's version of LDCM.  Note: The Local version of LDCM must be installed before the Administrator’s version.  To upgrade to the Administrator's version of LDCM, in Windows click on Start, then Run. In the command line within the dialog box enter the CD ROM drive letter followed by \software\LDCM\win95\admin\setup.exe

(Example: D:\software\win95\admin\setup.exe). Press enter.  Follow the directions in Setup.

Documentation for LDCM

In addition to the online help which is available in the Local version of LDCM, the Administrator version contains an Administrator's manual in Adobe Acrobat format.  Once installation of LDCM is complete, to install the Acrobat reader, execute the installation program which is found by default in c:\dmi\ldcm\acroread.exe (loaded by LDCM).  More information about Adobe Acrobat can be found in the file c:\dmi\ldcm\readme.txt after the install is complete.

INSTALLATION INSTRUCTIONS FOR WINDOWS NT:

Before installing the LDCM software you must first read and agree to the end-user license agreement enclosed in the file LICENSE.TXT or LICENSE.DOC included in root directory of this CD.

In Program Manager click on: File, then Run.  In the command line within the dialog box enter the CD ROM drive letter followed by \software\LDCM\winNT\local\setup.exe (example: D:\software\winNT\local\setup.exe).  Press enter.  Follow the directions in Setup.

If you are installing this on an Administrator's computer, you may additionally upgrade to the Adminstrator's version of LDCM. Note: The Local version of LDCM must be installed before the Administrator’s version.

To upgrade to the Administrator's version of LDCM, in File Manager click on File, then Run.  In the command line within the dialog box enter the CD ROM drive letter followed by \software\LDCM\winNT\admin\setup.exe (Example: D:\software\winNT\admin\setup.exe).  Press enter.  Follow the directions in Setup.

Documentation for LDCM

In addition to the online help which is available in the Local version of LDCM, the Administrator version contains an Administrator's manual in Adobe Acrobat format.  Once installation of LDCM is complete, to install the Acrobat reader, execute the installation program which is found by default in c:\dmi\ldcm\acroread.exe (loaded by LDCM).  More information about Adobe Acrobat can be found in the file c:\dmi\ldcm\readme.txt after the install is complete.

DOSICU/WINICU (DOS/WIN ISA Configuration Utility)

The ISA Configuration Utility (ICU) for both DOS and Windows 3.1 is included on the CD.  The ISA configuration utility is software to help you configure system resources when using older ISA add-in cards with a non-Plug and Play operating system in a system with a Plug and Play BIOS.  Windows 95 is a Plug and Play operating system, so no separate ICU is required.

Installing ICU under Windows 3.1/3.11

In Windows click on File, then Run.  In the command line within the dialog box enter the CD ROM drive letter followed by \software\winicu\install.exe (example: D:\software\winicu\install.exe).  Press enter.

To create DOS ICU diskettes:

- Have two DOS diskettes available labeled Disk1 and Disk2.

- From the SOFTWARE\DOSICU\Disk1directory on the CD, copy all the files to the diskette labeled Disk1.

- From the SOFTWARE\DOSICU\Disk2 directory on the CD, copy all the files to the diskette labeled Disk2.

- Place Disk1 in drive A:,  type install and hit Enter.

In order to use the ICU.exe program on a DOS based system, you must enter the BIOS setup program and change the option "Boot with Plug and Play OS" to "None".  In Setup choose "Advanced/PNP Configuration/Configuration Mode", then hit Enter.  Change from "Use Setup Utility" to "Use ICU".  Finally, change "Boot with Plug and Play OS" to "None".

PIIX4

This subdirectory contains files that make up a software utility which will automatically update Windows 95 .INF files so that the operating system recognizes the PIIX4 component.  Due to a limitation of the utility it is not possible to change all the necessary lines of the .INF files.  Instead,  the utility directly writes the Windows 95 Registry.  As a result, after the utility is run, if the PIIX4 keys are removed from the Registry, the .INF files will not hold all the information needed to completely rebuild the Registry.  Thus, the utility will need to be run again.

In general, there is only a small likelihood that the PIIX4 keys will be removed.  However,  if Regedit is used to view the Registry and the PIIX4 keys are mistakenly removed or driver hardware inadvertently deletes a key or a user incorrectly removes the keys from the Registry, then Windows 95 will no longer be able to recognize the PIIX4.  The update utility will need to be run again.

The utility works only with versions of Windows 95.  In order to run the utility you will need to read and agree to a software license agreement.  The text of the software license agreement will be displayed when you run the program setup.exe.  Please see the readme.txt file within the PIIX4 subdirectory for more details.

The directory structure of this CD ROM is as follows:

CD Root directory |

Readme.txt         |

|

| --- Acrobat |

|                    | --- 16bit

| 	        |

| 	        | --- 32bit

|

| --- Manuals |

|                     | --- English

|

|

| --- Software |

|                     | --- Audio      |

|                     |                      | --- Win311

|                     |                      | --- OS2

|                     |                      | --- Win95

|                     |                      | --- WinNT

|                     |

|                     | --- Dosicu     |

|                     |                      | --- Disk1

|                     |                      | --- Disk2

|                     |

|                     | --- Ldcm       |

|                     |                      | --- Win95 |

|                     |                      |                  |--- Admin

|                     |                      |                  | --- Local

|                     |                      |

|                     |                      | --- WinNT|

|                     |                      |                  | --- Admin

|                     |                      |                  | --- Local

|                     |

|                     | --- Piix4

|                     |

|                     | --- Winicu

|

|

| --- Warranty

*Other brands and names are the property of their respective owners.