OPSD BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97124-6497

DATE:	February 3, 1998

FROM:	Intel Customer Support - Oregon

SUBJECT:	4L4ML0X0.86C BIOS Release 2

About This Release

o	4L4ML0X0.86C.0004.P02 (Release 02)

o	Cirrus 5465 (500MHz RAMBUS memory) AGP video BIOS (AD stepping) identified as:  "CL-GD546x Laguna PCI VGA BIOS Version 1.62d

o	PXE (or Preboot eXecution Environment) BIOS Version 0.99b

Features/Errata Fixed In This Release

P02-0004

- Added L2 Cache ECC Enable/Disable Setup option to Advanced menu.

- Fixed incorrect slot information in type-9 DMI structures.

- Moved Advanced:Peripheral Configuration:Floppy disk controller option to Main:Floppy Options menu.

- Moved Hard Disk Pre-delay setup option to the Boot Menu.

- Added "AGP Aperture Size" Setup Option to the Advanced:Video Configuration menu to allow user to change the aperture size for the AGP Video Controller.  The current options available are: 64 MB and 256 MB.  The default is 64 MB

- Corrected APM resume on Magic Packet from Suspend state.

- Corrected APM Inactivity Timer so that IDE activity will	prevent entering Suspend state.  Previously, IDE activity did not prevent Suspend state after first Suspend state

- Fixed problem of cache remaining enabled when it has been disabled in Setup.

- Fixed problem which caused intermittent failures with INT 13h function 41h (Check Extensions Present).

P01-0003

- Initial release