OPSD BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97214

DATE:	July 29, 2026

TO:	OMCO Business Unit

CC:	iPG / OPSD BIOS Engineering

FROM:	OPSD BIOS Engineering

SUBJECT:	PD440FX Standard (DT0) BIOS Release 5.0

About This Release

- BIOS 1.00.06.DT0  Release 6.0

- Features/Errata Fixed In This Release

- Release 6.0

- Banner changed to Release 6.0

- Added  the BIOS Setup option of “L2 Cache ECC Support” when a Pentium(R) II processor with L2 Cache ECC support is detected.  The default is enabled.

- Added  support for C1 stepping of Pentium II processors.

- Fixed the issue where accessing the DMI Event Log  in 32 bit protected mode would corrupt memory.

- Fixed the issue where the PCI IRQ routing table was not being passed to operating systems.

- Fixed the issue where the Intel EtherExpress (TM) PRO/100 Server adapter would not operate in the PD440FX.

- Fixed the issue where the DMI interface was not reporting the Pentium II processor L1 cache.

Release 5.0

- Banner changed to Release 5.0

- Fixed a problem where Int 15h functions were returning with interrupts disabled.

- Updated memory detection to handle 64MB EDO SIMMs.  There is no checking for 64MB DIMMs.

- Correctly set #SERR in the North Bridge if a PCI-to-PCI bridge is installed in the  system. Previously, the #SERR was not being set which caused ECC multi-bit errors not to generate an NMI.

- Added POST support to recover a missing, corrupted, or invalid Event Log Header in the User Flash Area.

- Added boot virus support.

- Fixed a problem where an interrupt was assigned to the PS/2 mouse port even if a mouse was not connected to the port.

- Release 4.0

- Banner changed to Release 4.0.

- Added support for LS-120 drives.

- Added a check for century rolling over when the system is in a power saving state.

- Fixed the problem the APM BIOS incorrectly sets the resume timer if there is an user alarm active.

- Added support to activate both DMA and PIO mode of operation for ATA and ATAPI devices.  If the device supports DMA (Ultra or MultiWord), Bus Mastering capability will be enabled as well.  This will allow non PIIX knowledgeable bus mastering drivers to operate properly.

- Modified maximum length of largest DMI structure from 256 bytes to 512 bytes since structures were being truncated during DMI function 51h.

- Changed maximum amount of Event Log entries from 254 to 255.

- Added ability for the BIOS to prevent assignment of drive letter to specific ATA devices based on the identify drive information.

- Updated processor identification in Setup utility:

- Processor Type:  Pentium® Pro with MMX™ technology  changed to

- Processor Type:  Pentium® II

- Fixed problem of temporarily grayed entries on the BIOS setup screen the first time Setup is accessed.

- Modified BIOS flash routine to address issues (e.g. DMI strings being lost) related to partial erasure of the onboard flash memory as a result of power cycling during a flash update to the VPD area.

- Updated the Pentium II processor’s L2 Cache Initialization.  Fixed a problem where machine check exceptions will  occur with (BBL ECC errors) with ECC BSRAMs.

- Added support for DMI type 12, 13 structures.  Fixed a problem where DMI incorrectly reports DIMMs are ECC.

- Fixed a problem where the system would not enter a sleep moded after 1 minute, even if the user sets the Inactivity Timer in BIOS to 1 minute.

- Fixed a problem where, if the user presses Sleep button continuously the system will not recognize any further sleep inputs.

- Fixed a problem where setting VESA Video Power Down to "disable" will still allow the video to be powered down..

- Fixed a problem where flashing in a User Logo makes the system unbootable.

- Release 3.0

- Banner change to Release 3.0

- BIOS now refers to Pentium® II processor

- Release 2.0

- There was no external Release 2.0

- Release 1.0

First released production BIOS