MONTEST V1.2

by

Paul Ratcliffe

(c) June 1997

This program is designed for use on PC VGA cards (and compatibles) with

either a monochrome or colour monitor attached. Obviously the usable

facilities will be rather limited on a monochrome monitor.

It is designed to allow the user to accurately set up their monitor for

optimum performance. The signals generated match as closely as possible to

those used by myself in BBC TV studios.

This program should not cause your hardware any problems. Only documented

BIOS calls are made and nothing else.

If, however, you have a problem, the problem rests entirely with the user

and no responsibility is accepted by the author.

USE OF THIS PROGRAM IS ENTIRELY AT YOUR OWN RISK.

OPERATION

The program is menu driven and is, hopefully, fairly self explanatory. The

two most important keys are the ESCape key and the Spacebar. Spacebar

allows you to cycle through the various options when you have selected an

item from the main menu. ESCape is used to return from the selected item to

the main menu and can also be used to exit the program.

MENU ITEMS

There are 11 of these at present. The purpose and facilities available on

each of these will now be described.

PLUGE

PLUGE - Picture Line Up Generating Equipment.

This is THE reference signal to which your monitor should be adjusted. The

controls which are adjusted here are brightness and contrast.

Brightness should be adjusted so that the thin vertical bar on the left

hand side of the screen is just visible when you are viewing under normal

lighting conditions. N.B. This is slightly different to BBC PLUGE due to

the inability to generate signals below 0v on a VGA card.

If you set it up in daylight it will not be the same as with artificial

lights on or with no lights on (if you have a habit of working in the

dark!), so unless you want to be endlessly tweaking, set it up for the

lighting conditions you normally use and it won't be too far out for the

others.

Contrast should be set so that the white bar at the top on the right hand

side of the screen is as bright as you want high intensity white displayed.

There is a bit of personal preference involved here.

There are defined settings, but again it is dependent on the incident light

on the screen and unless you have a spotmeter, it is very academic.

Brightness and Contrast are interactive so you should adjust each at least

twice.

No user keys are available as this is the reference signal.

Grille (Low Resolution)

This is used to check the convergence of your monitor. That is, it makes

sure the red, green and blue signals from the three guns form three images

on the face of the tube which overlay each other exactly.

Colours available are:	Cyan - Blue/Green convergence

Magenta - Blue/Red convergence

Yellow - Red/Green convergence

White - Red/Green/Blue convergence

Twittered versions of Cyan, Magenta and Yellow are also available which

alternately switches each of the pair of primary colours on/off at a

determined rate. This sometimes make alignment easier by adjusting for

minimum movement between the two colours.

Convergence is usually adjustable internally.

Picture geometry can also be adjusted using Grille. The controls for this

may be either internal or external.

Grille (High Resolution)

This is the same as Grille (Low Resolution) except that the lines drawn on

the screen are much 'thinner'. It is more difficult to converge accurately

and indeed some monitors may not be capable especially in the corners of

the tube.

Stepwedge

This generates a 16 level grey scale from black to full amplitude white.

This is used to adjust colour balance such that the colours at the black

end are black/dark grey and the colours at the white end are

light grey/white. It is very common to see poorly grey scaled monitors

which are say magenta in the blacks and blue in the whites. There should be

no colour whatsoever and it shouldn't change with varying brightness or

contrast either. Not many monitors perform perfectly in this area.

Colour balance controls are usually internal, however some may be external.

Do not adjust unless the monitor is thoroughly warmed up. This also applies

to all other adjustments.

Colour Bars 100%

This generates full amplitude, fully saturated colour bars. Not terribly

useful for monitor line up but included for completeness.

Colour Bars 75% (EBU bars)

This generates 75% amplitude, fully saturated colour bars, except for the

white bar which is 100% amplitude. Not terribly useful for monitor line up

but included for completeness.

Purity

This checks that the electron beam from the gun falls in the correct place

on the screen and generates the correct colour. Red is the most sensitive

to check by eye but green and blue should be checked as well. The screen

should be a uniform colour and not have patches of any other colour.

Purity is adjusted internally.

EHT regulation

This checks how well the monitor handles large changes in signal level. The

size of the raster should stay constant for perfect regulation. On most

monitors it will change, often by a considerable amount. Unfortunately, you

can't do anything about it except redesign the power supplies! Check before

buying the monitor!

Monochrome

This reduces the saturation (colour) control to zero effectively turning

everything to monochrome. This is a software setting which only exists

until the next mode change and does not affect the hardware.

Colour

This increases the saturation control to maximum (normal) restoring the

screen to normal colour. It is not possible to over-saturate on a VGA card

unlike a TV. This is a software setting which only exists until the next

mode change and does not affect the hardware.

Quit

Exits the program.

KEYS

Up/Down

These adjust the contrast of the displayed picture. This is a software

setting which only exists until the next mode change and does not affect

the hardware.

Right/Left

These adjust the saturation of the displayed picture. This is a software

setting which only exists until the next mode change and does not affect

the hardware.

Plus/Minus

These affect the twitter frequency used on the Grill option.

Spacebar

This allows you to cycle through the various displays of an item selected

from the main menu.

ESCape

This is used to return from a selected item to the main menu and can also

be used to exit the program.

HISTORY

Version 1.2  June 1997

Added 75% (EBU) bars.

Version 1.1  May 1997

Correct the levels of the PLUGE bars.

Fix minor problems with the keyboard handling (function keys etc.)

Version 1.0  Apr. 1993

Initial release.

OTHER

It is recommended that you do NOT remove the case from your monitor unless

you are sure YOU KNOW WHAT YOU ARE DOING, why you are doing it and that

somebody else is around in case you run into trouble. There are very high

voltages inside monitors - please beware!

If you use this program could you please let me know. Also if you have any

comments, criticisms, bug reports or anything that could be improved.

I will reply to all messages received.

Paul Ratcliffe:	Internet: paul.ratcliffe@bbc.co.uk

Internet: paul.ratcliffe@virgin.net

Netmail to 050/074 GT Power Network