*************************************

********* UN57064 CORRECTIVE SERVICE DISKETTE *********

*************************************

TCP/IP 2.0 for OS/2 NFS Corrective Service Diskette Information

This README file for the corrective service diskette (CSD) reflects

new and changed functions for the TCP/IP 2.0 for OS/2 Network File

System (TM) kit.

Note:  You must install the Base Kit CSD (UN56401) before you

install this CSD.

---------------------------------------------------------------------

Topics incuded in this CSD are:

1. Installing this CSD

2. Previous CSD Information

3. Obtaining Future CSDs Electronically

4. Changes to NFSSTART

5. Changes to EXPORTS File

6. The MOUNT Command

7. The UMOUNT Command

8. NFS Security

9. The NFSCLEAN Command

10. APAR Fixes

---------------------------------------------------------------------

1. Installing this CSD

To install this CSD from a diskette at your workstation, insert the

diskette into your A: drive and enter:

A:>TCPINST

---------------------------------------------------------------------

2. Previous CSD Information

This CSD incorporates the corrective service and new functions

that were previously released in CSD UN52836.  Please see the file

README.BAS in the <tcpip>\DOC directory for a description of the

functions that were originally released in UN52836.

---------------------------------------------------------------------

3. Obtaining Future CSDs Electronically

You can obtain Corrective Service Diskettes (CSDs) electronically

by:

o   Using FTP to access SOFTWARE.WATSON.IBM.COM as Anonymous.

NOTE:  Fixes for TCP/IP for OS/2 are located in the

pub\tcpip\os2 directory.

o   Establishing a modem connection to the NCSBBS bulletin board

at 1-919-517-0001.

NOTE:  Set your modem settings to: none,8,1.

The latest CSDs for TCP/IP Version 2.0 for OS/2 are shown in the

following table:

+=============================+===========================+

| KIT NAME                    | PACKAGE NAME              |

+=============================+===========================+

| Applications Kit            | APP20CSD.PKG              |

+-----------------------------+---------------------------+

| Base Kit                    | TCP20CSD.PKG              |

+-----------------------------+---------------------------+

| Domain Name Server Kit      | DNS20CSD.PKG              |

+-----------------------------+---------------------------+

| DOS/Windows Access Kit      | DOS20CSD.PKG              |

+-----------------------------+---------------------------+

| Extended Networking Kit     | XNT20CSD.PKG              |

+-----------------------------+---------------------------+

| NetBIOS Kit                 | NET20CSD.PKG              |

+-----------------------------+---------------------------+

| Network File System Kit     | NFS20CSD.PKG              |

+-----------------------------+---------------------------+

| OSF/Motif Kit               | MOT20CSD.PKG              |

+-----------------------------+---------------------------+

| Programmer's Tool Kit       | PGM20CSD.PKG              |

+-----------------------------+---------------------------+

| X Window System Client Kit  | XCL20CSD.PKG              |

+-----------------------------+---------------------------+

| X Window System Server Kit  | PMX20CSD.PKG              |

+-----------------------------+---------------------------+

*************************************************

********* UN57064 CORRECTIVE SERVICE DISKETTE STARTS HERE *********

*************************************************

---------------------------------------------------------------------

4. Changes to NFSSTART

NFSSTART is now an executable file instead of a command file.  Now,

when entering this command in the TCPSTART.CMD file, it should be

entered as:

nfsstart

instead of the previous format:

call nfsstart.cmd

Note:  To avoid conflicting commands and to preserve changes you

may have made to NFSSTART.CMD, the installation program

automatically renames NFSSTART.CMD to NFSSTART.BAK when

you install this CSD. You must make changes to your

TCPSTART.CMD file to ensure that NFS starts properly.

There are two ways to make the changes to TCPSTART.CMD:

- Open and save the Configuration Notebook (run and save

TCPIPCFG.EXE)

- Manually edit TCPSTART.CMD to call NFSSTART.EXE instead

of the old NFSSTART.CMD

---------------------------------------------------------------------

5. Changes to EXPORTS File

The hostname field of the NFSD EXPORTS file now accepts

dotted-decimal IP addresses as well as hostnames.  Dotted-decimal

addresses have the advantage of not requiring the resolver during

initialization.

In addition, to be more efficient, NFSD now converts the hostnames

in the EXPORTS file to IP addresses while the EXPORTS file is being

read rather than every time a mount request arrives.

---------------------------------------------------------------------

6. The MOUNT Command

The MOUNT command's -u and -g parameters have been modified to

accept a wider range of values.

>>--mount--.-----.--.-----------------.--.-----.--.--------.-->

'- -c-'  '- -v--.--------.-'  '- -a-'  '- -uuid-'

'-passwd-'

>----.--------.--.------------.--.-----------.--.-----.------->

'- -ggid-'  '- -lloginid-'  '- -ppasswd-'  '- -s-'

>-- drive_letter-- host_name:exported_file_system------------><

-uuid     Specifies the user ID (uid).  The uid must be a number

between -2 147 483 647 and +2 147 483 647.

Note:  The range of values for the uid includes spaces

for readability.  When entering your uid, you

should not include any spaces.

-ggid     Specifies the group ID (gid).  The gid must be a number

between -2 147 483 647 and +2 147 483 647.  The GID value

is passed to the NFS server for authentication.

Note:  The range of values for the gid includes spaces

for readability.  When entering your gid, you

should not include any spaces.

Notes:

1.  If the client is mounting an OS/2 server, a UID and GID are

still needed for the connection, but are not used for

security.

2.  The OS/2 NFS client supports versions 1 and 2 of the PCNFSD

protocol.  NFS determines which version is running each time

you issue a MOUNT command.

3.  If you use PCNFSD when using the MOUNT command and

PCNFSD returns -2, -2 for the UID and GID, (indicating the

'nobody' user), you can still mount a directory.

---------------------------------------------------------------------

7. The UMOUNT Command

The following warning has been added to the UMOUNT command

description:

Warning:  To avoid unpredictable results, wait until all processing

stops before unmounting a drive.

---------------------------------------------------------------------

8. NFS Security

The NFS server protects files by limiting access to directories

through:

- The EXPORTS file

- PCNFSD

----------------------

8.1 The EXPORTS file

The \TCPIP\ETC\EXPORTS file (on the OS/2 NFS server) must list each

directory to be made available to NFS clients.  If a directory is

listed, all of its files and subdirectories are available to the NFS

client.  However, the EXPORTS file allows the NFS server's

administrator to limit client access to directories (based upon the

client's IP address or hostname).  The following is an example of

the contents of an EXPORTS file:

d:\share\joe         joe.somewhere.com

d:\share\mary        pink.triangle.edu

In this example, a user would have to use a machine with an

address of joe.somewhere.com to access any of the files and

subdirectories under the d:\share\joe directory.  Similarly, to

access any files under d:\share\mary, requests would need to

originate from the machine using an address of pink.triangle.com.

Note: The hostname field of the NFSD EXPORTS file now accepts

dotted-decimal IP addresses as well as hostnames.

Dotted-decimal addresses have the advantage of not requiring

the resolver during initialization.

----------------------

8.2 PCNFSD

NFS also provides security by enforcing authentication through

PCNFSD.  PCNFSD is a server which maps a valid login name and

password to a uid (user ID).

When the OS/2 NFS client issues a MOUNT command to access an NFS

server's directories, the MOUNT command checks to see if PCNFSD is

available.  If it is, the OS/2 MOUNT command prompts you for your

login name and password which it then passes to PCNFSD.  If PCNFSD

authenticates the login name and password, it passes back the

associated uid.  The OS/2 NFS client then uses that uid for each NFS

request, ignoring any previously specified uid.  If PCNFSD reports

that the login name or password is invalid, the MOUNT command is

terminated and access to the server's files is denied.

PCNFSD uses the PASSWD file, located in the directory specified by

the ETC environment variable.  Each line in the PASSWD file contains

a user's:

- Login name

- Password

- User id (uid)

- Group id (gid)

- Full name

- Home directory

- Login shell

As additional security, the password is encrypted in the file using

a randomly chosen encryption key.  Because each machine uses a

different key, a user can use the same password on different

machines with no loss of security.  The user's full name, home

directory, and login shell fields are provided only for Unix

compatibility, and are not used by OS/2.

Notes:

1. You can update the PASSWD file using the PASSWD command.

You should not edit the PASSWD file manually because of its

specific syntax.

2. If the PASSWD file contains path information in one or

more of its fields, the colon normally used as part of the

OS/2 path will appear in the PASSWD file as a semicolon.

This is normal, and is due to the fact that PASSWD uses the

colon character to separate the fields.

----------------------

8.3 Starting and Running PCNFSD

You can start PCNFSD from an OS/2 command line, or you can set up

PCNFSD to autostart.  The PCNFSD syntax is:

>>--pcnfsd------------------------------------------------------><

Note:  Because TCPIPCFG does not currently recognize PCNFSD, every

time you run TCPIPCFG, it will remove PCNFSD from TCPSTART.

Therefore, you should start PCNFSD from the TCPEXIT.CMD file

located in \TCPIP\BIN.  If this file exists, TCPSTART.CMD

executes it as its final action.

After you start PCNFSD, all error messages and warnings are logged

to SYSLOGD (if running).  Whenever a client's authentication fails,

a message is logged to SYSLOGD.  If a client repeatedly fails to

authenticate, you should consider investigating the problem, because

it could be an unauthorized attempt to access your data.

----------------------

8.4 The PASSWD Command

Use the PASSWD command to create the PASSWD file, and to add or

modify the information in that file.

Note:  The OS/2 PASSWD file is compatible with any non-System 5 Unix

PASSWD file.  System 5 based Unix shadows passwords in a

different file.

>>--passwd--.-----.-- login_name--------------------------------><

|- -a-|

|- -f-|

'- -s-'

-a          Specifies that PASSWD is to add information for the

user (login_name) to the PASSWD file.  After you press

Enter, PASSWD prompts you for the user's:

-  Password (twice, for verification)

-  Uid

-  Gid

-  Full name

-  Home directory

-  Login shell

The login name and its associated uid must be unique.

If either the login name or the uid is already listed

in the PASSWD file, PASSWD does not change the file.

-f          Specifies that you are changing the FINGER information

for the specified user.  This parameter is provided for

Unix compatibility and is not used by OS/2.

Note:  The FINGER command displays information about

users on a remote host.  On Unix, finger

information for a login name reveals information

about the person to whom that login name belongs.

It can include the user's full name, office,

telephone number, and other information deemed

appropriate by the user.

-s          Specifies that you are changing the default shell for

the specified user.  This parameter is provided for

Unix compatibility and is not used by OS/2.

login_name  Specifies the user's login name.

If you do not specify other parameters with login_name,

PASSWD allows you to change the specified user's

password.  PASSWD first requests the current password to

verify that the user has the authority to change the

password.  PASSWD then requests the new password twice,

the second time as verification.

----------------------

8.5 SYSLOGD

If you are running SYSLOGD, applications can record error messages

and warnings for future reference.  Messages can be:

- Recorded in a file on your local host

- Forwarded to a central location

If everyone forwards the messages to a central location, the system

administrator can browse and discover problem areas easily instead

of logging onto each host to check the SYSLOG message depository.

You can start SYSLOGD from an OS/2 command line, or you can set up

SYSLOGD to autostart.  The SYSLOGD syntax is:

>>--syslogd--.------------.-------------------------------------><

'- -t target-'

-t target   Specifies the target host, where target can be:

o   A fully qualified path name

o   An IP address

o   An IP host name

If target specifies a fully-qualified filename, SYSLOGD will append

all messages to that file.  If target is an IP address or IP

hostname, SYSLOGD forwards all messages to that host's SYSLOGD.  If

the -t option is not specified, SYSLOGD defaults to a file called

SYSLOG.MSG in the \TCPIP\ETC subdirectory.

You should inspect the target file occasionally, because the file

will grow depending on which applications are running and how many

messages they record.  In addition, you should monitor all recorded

program errors (for example, authentication failures).

Note:  Because TCPIPCFG does not currently recognize SYSLOGD, every

time you run TCPIPCFG, it will remove SYSLOGD from TCPSTART.

Therefore, you should start SYSLOGD from the TCPEXIT.CMD file

located in the \TCPIP\BIN subdirectory.  If this file exists,

TCPSTART.CMD executes it as its final action.

---------------------------------------------------------------------

9. NFSCLEAN Command

The description of the NFSCLEAN command function as stated in the

Command Reference is incorrect.  The following replaces the original

description:

If NFS has been stopped without unmounting all attached drives,

use the NFSCLEAN command to unmount drives that are still

attached.

---------------------------------------------------------------------

10. APAR Fixes

The following is a list of APARs fixed since this release became

available.  APARs are grouped numerically by CSD.

----------------------

10.1 APARs Fixed in This CSD (UN57064)

+============+=============+=======================================+

|            | MODULES     |                                       |

| APAR       | AFFECTED    | DESCRIPTION                           |

+============+=============+=======================================+

| PN46796    | NFSCTL.EXE  | Fixed:  Symbolically linked           |

|            |             | directories could not be linked by    |

|            |             | applications.                         |

+------------+-------------+---------------------------------------+

| PN46880    | NFSD.EXE    | Fixed:  User starts the NFS client,   |

|            |             | connects to the SERVER, and receives  |

|            |             | error messages, resulting in an abend.|

+------------+-------------+---------------------------------------+

| PN48329    | NFSCTL.EXE  | Fixed:  NFSCTL receives divide by     |

|            |             | zero error msg and terminates.        |

+------------+-------------+---------------------------------------+

| PN48338    | NFSD.EXE    | Fixed:  NFSD will hang when client    |

|            |             | tried to access file dated 1/1/80.    |

+------------+-------------+---------------------------------------+

| PN49844    | NFSD.EXE    | Fixed:  Wrong error code returned     |

|            |             | if MKDIR invoked when directory       |

|            |             | already exists.                       |

+------------+-------------+---------------------------------------+

| PN50108    | NFSCTL.EXE  | Fixed:  DOS LAN requestor deleted     |

|            |             | wrong files through an NFS mounted    |

|            |             | drive.                                |

+------------+-------------+---------------------------------------+

| PN50393    | NFSD.EXE    | Fixed:  Unable to access              |

|            |             | two-directory deep filesystem if a    |

|            |             | client name is specified in EXPORTS   |

|            |             | file.                                 |

+------------+-------------+---------------------------------------+

| PN52057    | NFS200.IFS  | Fixed:  DOSQUERYPATHINFO level 7      |

|            |             | request not fixed in UN52836.         |

+------------+-------------+---------------------------------------+

| PN52251    | NFSCTL.EXE  | Fixed:  Unable to set access rights   |

|            |             | on NFS mounted drive.                 |

+------------+-------------+---------------------------------------+

| PN53322    | NFSCTL.EXE  | Fixed:  Some applications cannot save |

|            |             | files to an NFS mounted drive.        |

+------------+-------------+---------------------------------------+

| PN54375    | NFSCTL.EXE  | Fixed: DIR of an NFS drive on some    |

|            |             | servers would return the '.'          |

|            |             | directory and then give back an       |

|            |             | error code.                           |

+------------+-------------+---------------------------------------+

| PN55249    | NFSCTL.EXE  | Fixed:  MS Word can't save a document |

|            |             | when file sharing is enabled.         |

+------------+-------------+---------------------------------------+

| PN55250    | NFSCTL.EXE  | Fixed:  NFS can only mount/unmount    |

|            |             | about 7000 times.                     |

+------------+-------------+---------------------------------------+

----------------------

10.2 The following APARs were fixed in CSD UN52836.  These

changes are included in this CSD as well.

+============+=============+=======================================+

|            | MODULES     |                                       |

| APAR       | AFFECTED    | DESCRIPTION                           |

+============+=============+=======================================+

| PN45546    | NFSCTL.EXE  | Fixed: Copying over a file with a     |

|            |             | different UID but the same GID will   |

|            |             | not work.                             |

+------------+-------------+---------------------------------------+

| PN45780    | NFSCTL.EXE  | Fixed: A date of 1/1/80 and a time    |

|            |             | between midnight and 1 AM caused a    |

|            |             | trap.                                 |

+------------+-------------+---------------------------------------+

| PN46618    | NFSBIOD.EXE | Fixed: NFS limited to 40 mounts.      |

+------------+-------------+---------------------------------------+

| PN46869    | NFSCTL.EXE  | Fixed: Directory gets SYS0005 if      |

|            | NFSDIR.EXE  | there are files you do not have       |

|            |             | access to.                            |

+------------+-------------+---------------------------------------+

| PN46970    | NFSCTL.EXE  | Fixed: In NFS, XCOPY with wildcard    |

|            |             | characters only (*.*) fails.          |

+------------+-------------+---------------------------------------+

| PN47285    | NFSCTL.EXE  | Fixed: NFS client does not handle     |

|            |             | daylight savings time correctly.      |

+------------+-------------+---------------------------------------+

| PN47491    | UMOUNT.EXE  | Fixed: When a user unmounts an        |

|            |             | RS/6000, the ETC/RMTAB file is not    |

|            |             | being updated on the RISC/6000.       |

+------------+-------------+---------------------------------------+

| PN48574    | NFSCTL.EXE  | Fixed: NFSCTL does not support        |

|            |             | DosQueryPathInfo level 7.             |

+------------+-------------+---------------------------------------+

| PN48770    | NFSCTL.EXE  | Fixed: Mounting a server when client  |

|            | MOUNT.EXE   | uses 4OS2 results in files not        |

|            |             | displaying with DIR command.          |

+------------+-------------+---------------------------------------+