TAM Sample Application

Caveat: Although the TAM sample included with this beta is a functional

answering machine, it does not contain the error recovery or feature set

required of a robust application.

Overview

The TAM application is designed to operate both as a standard telephone and

a telephone answering system.  When using either the telephone to take a

call, or the answering system to review messages, the application allows the

user to select either a desk telephone handset or an external

microphone/speaker as an input/output device.  This allows for private

reviewing of messages through the handset, and adds speakerphone capability

to a standard telephone through the microphone and speaker devices.

To implement this dual functionality, the application is defined to operate

in two distinct modes, a 'Telephone' mode and a 'Message Review' mode.  The

operating mode is set by the user, through the 'Mode' pulldown menu.

In 'Telephone' mode, the application acts as a standard telephone.  In this

mode, the system answers incoming calls, plays an announcement, and records

messages.  When system output is set to 'Handset', the desk telephone is

connected directly to the teleph one line.  When the output is set to

'Speaker', the telephone is taken off hook and the system microphone and

speaker are enabled.

In 'Message Review' mode, the application can play recorded messages to

either the speaker or telephone handset.  All message play controls are

located under the 'Play Control' menu.  Note that when message reviewing is

taking place, the system does not ans wer any incoming calls.

The handset volume is not adjustable under the sample application, but the

speaker volume (used for both speakerphone and message review operations)

can be set using the 'Volume' menu.

System Setup

1.  Copy tam.exe and the .tam files to a \tam subdirectory (or other

convenient subdirectory) on your system.

2.  If you wish to add a tam icon to the desktop:

a.  Open the Templates folder and drag a copy of the Program icon to the

desktop.

b.  Enter the following information in Program Settings: path and filename

of the executable (tam.exe); working directory; title for the icon

(e.g. TAM).

3.  To start TAM, simply double-click on the TAM  icon (or enter

tam from an OS/2 command line).

The TAM application is mostly self sufficient, but there are some

initialization steps to perform when setting up a new answering system.

Below are specific instructions for the various initialization procedures.

Recording an Announcement

The first step in using the TAM sample application is to record a new

announcement.  This is done by selecting the 'Message Review' mode option

under the 'Mode' menu, and then selecting 'Record Announcement...' under the

'Configure' menu.  A three part dial og box is displayed.

The top portion of the dialog box is used for recording.  The 'Record

from...' box in the upper left hand corner of the dialog box determines the

input recording device.  The default device is set to 'Microphone' so if you

wish to record your announcement f rom the telephone handset, first select

the 'Telephone Handset' button in the 'Record from...' box.

After the input device has been chosen, click on the 'Begin Recording'

button and start speaking (once you hear a beep) into the input device you

have chosen.  When done, click on the 'End Recording' button.  To listen to

the announcement you just recorded, select an output device in the 'Play

to...' box, and click on the 'Play Announcement' button.  The announcement

plays to the output device selected.  If you wish to use the newly recorded

announcement, click on the 'Ok' button.  Otherwise, to keep the old a

nnouncement, click on the 'Cancel' button.

After closing the dialog box, make sure you place the application back into

telephone mode by selecting the 'Telephone' mode option in the 'Mode'

pulldown menu.

Setting the Ring Count

The ring count determines the number of rings before which the system

answers the telephone to record an incoming message.  The TAM application

has two ring counts, one for when messages are available, and one for when

no messages are available.  A common ' toll saver' feature is to set the

device to answer on the first ring when new messages are available, but not

to answer before the fourth ring when there are no new messages.  This

allows the user to hang-up when calling remotely before the system answers

when there are no messages available.

To set the ring count, select the 'Set Ring Count...' item in the

'Configure' menu.  A dialog box is displayed, prompting for the two types of

ring counts.  After entering a new ring count for when messages are

available and one for when no messages available, press the 'OK' button to

use the new counts, or press the 'Cancel' button to abort any changes.

Setting the Command Code

The TAM application allows the user to retrieve messages from a remote

telephone, by calling the device, and when prompted to record a message,

entering instead a 3-digit command code on the touch-tone keypad.  The

command code is configurable, and can be changed any time.  To set a new

command code, select the 'Set Command Code...' item on the 'Configure' menu.

A dialog box prompting for a new command code is displayed.  After entering

a new command code, press the 'OK' button to accept the change, or press the

'Cancel' button to abort any changes and keep the original code.

Using the Speakerphone

In addition to providing the answering machine function, the TAM application

turns any speaker/microphone connected to the Mwave board into a

speakerphone.

Initiating a Speakerphone Call

Before initiating a call for use woth the speakerphone, verify that the

application is in telephone mode by selecting the 'Telephone' option under

the 'Mode' pulldown menu, and that the system is connected to the handset by

selecting the 'Handset' item under the 'Mode' pulldown menu.

Note: These two options should always be set when you are not reviewing

messages.

The speakerphone call is initiated by dialing the desired number using your

standard desk phone.  After the number has been dialed, the speakerphone is

initiated by selecting the 'Speaker' option under the 'Mode' pulldown menu.

After this has been done, yo u can hang-up the telephone handset.

Speakerphone Volume Control

While the speakerphone call is in progress, the output volume of the speaker

can be adjusted by selecting a volume level from the 'Volume' pulldown menu.

Terminating a Speakerphone Call

To terminate a speakerphone call, leave the desk phone handset on hook, and

place the system back into handset connect mode by selecting the 'Handset'

item under the 'Mode' pulldown menu.

Reviewing Messages Locally

The number of active messages on the system is shown in the application

window whenever the application is open on the desktop.  If new messages

arrive while the application is in icon form, new messages can be detected

by the flashing of the icon text.  Th e title bar on the application window

also flashes if new messages have arrived since the last message review.

Playing Recorded Messages

To review messages, select the 'Message Review' mode from the 'Mode'

pulldown menu.  Incoming calls are not be answered while you are in this

mode.

Next, you should select the output device to which to play your messages.

You can play messages to either the telephone handset or the external

speaker.  To use the handset, select the 'Handset' option on the 'Mode'

pulldown menu.  To use an external speake r, select the 'Speaker' option on

the 'Mode' pulldown menu.  When using the speaker, you can adjust the

speaker volume be selecting a new volume level from the 'Speaker Volume'

pulldown menu.

When in message review mode, some of the entries on the 'Play Control'

pulldown menu become visible.  To start reviewing messages, select the

'First' item on the 'Play Control' pulldown menu.  This prompts the system

to play the first active message.  If the 'First' item is grayed on the

menu, then you have no active messages.

After the first message has played, you have a choice of either replaying

the message, keeping the message and playing the next message, or erasing

the message and playing the next message.  To replay the message, select the

'Repeat' option on the 'Play Co ntrol' pulldown menu.  To keep this message

and play the next message in the queue, select the 'Next' option.  To erase

this message and play the next message (if any), select the 'Erase' option.

After playing the second message, you can go back and play th e previous

message by selecting the 'Previous' option.

Note: When you have finished reviewing messages, you should always return

the system to telephone mode by selecting the 'Telephone' mode option under

the 'Mode' pulldown menu.

Message Positioning Controls

While a message is playing, you can step back 5 seconds in the message, step

forward 5 seconds in the message, or pause the message playback.  When

paused, the playback will remain stopped until unpaused (pause selected a

second time).  These options are pe rformed by selecting the 'Back 5

seconds', 'Ahead 5 seconds', or 'Pause' menu entries under the 'Play

Control' pulldown menu.

Message Speed Controls

As well as being able to skip around in a message using 'Step Back' and

'Skip Forward', the TAM application also allows you to set the play speed of

the message, so that you can play back messages at an accelerated or

decelerated rate.  To change the messa ge play speed, select a new speed

(either 'Play Slow', 'Play Normal', or 'Play Fast') from the 'Play Control'

pulldown menu.  The speed setting you choose remains in effect until you

exit the message review mode.

Reviewing Messages Remotely

Message review is also possible using a touch-tone telephone, when calling

from a remote location.  To gain access to the message review mode of the

application, first call the system and let the answering machine answer the

call.  When the announcement beg ins to play, enter the 3 digit command code

you selected during the application setup process.  The system now tells you

how many active messages you have.  At this point, you can begin entering

touch-tone keypad commands to review messages as described in the previous

section.  Below is a list of the available keypad command options:

Message Play Commands:

Press '1' to play first/next active message

Press '2' to erase current message and play next active message

Press '3' to replay or restart the current message

Press '4' to archive current message and play next active message*

Message Play Control Commands:

Press '51' to back up 5 seconds in current active message

Press '52' to pause and resume playing active message

Press '53' to skip forward 5 seconds in current active message

Note: Message saving is not available under the sample application.  Remote

telephone command operation is one (of many) areas that can be greatly

enhanced in an actual TAM / Voice-Mail application.