.*      SCCSID = @(#)vflpy.doc	6.2 91/04/11

.* symbol definitions

.im ..\..\equate.sci

.im ..\..\macros.sci

:h3.Virtual Floppy Device Driver Architecture

:Bissues.vflpyisu.isu

:h4.Problem Description/Objectives

The current &vdsk. component only virtualizes VDM hard disk Int 13h accesses.

This is insufficient for supporting VDM applications such as "FastBack"

which accesses the floppy ports directly.

:ul.The issues of supporting such software are:

:li.Resolve the port access contention between the system (OS/2 protected

apps.) and the VDMs.

OS/2 protected mode applications do not access the floppy hardware

directly.  The access to the floppy is done through the file system

and eventually serialized by the physical floppy disk device driver.

Therefore, floppy accesses can be classified into two classes: system

accesses and VDM accesses.  System accesses are done via the physical

floppy device driver (PFLPY), whereas the VDM accesses are done via

the virtual floppy device driver (VFLPY).  Therefore, VFLPY must be

able to communicate with PFLPY to negotiate physical floppy controller

ownership.

:li.Resolve the access contention between different VDMs.

Similar to the physical floppy device driver which serializes all

system floppy accesses, VFLPY is responsible for serializing all

VDM floppy accesses.

:li.Maintain the integrity of the floppy hardware states.

The physical floppy controller itself is a state machine.  A floppy

operation involves multiple bytes I/O.  Therefore, it is very

important to guarantee the atomicity of the floppy operation.  In

the previous releases of OS/2, this is guaranteed by the physical

device driver.  In Cruiser, since we have two potential owners of

the physical floppy hardware (PFLPY and VFLPY), both PFLPY and VFLPY

must make sure that the current floppy operation is completed before

turning the control over to the other owner.  In PFLPY, it is simple

because the operation is initiated by the device driver itself

and the device driver always keeps track of the floppy controller

states.  But for the VFLPY, it is more complicated because the operation

is initiated by the DOS application.  Therefore, VFLPY must be able

determine the end of the operation.

:li.Floppy interrupt routing.

The physical floppy controller is capable of generating interrupts

when an operation is done.  This interrupt must be simulated to the

VDM if it has the floppy ownership.

:li.DMA support.

The floppy controller requests DMA services from the DMA controller

doing the data transfers during some of the operations.  Therefore,

it is important for the VFLPY to communicate and cooperate with

VDMA to synchronize the operation on both ends.

:eul.

:h4.Solutions/Justification

In supporting the MVDM (Multiple Virtual Dos Machines) environment and

be a better DOS than DOS, it is important to support direct floppy

hardware access of some of the DOS applications such as FastBack.

Other copy protected software may also access the floppy hardware or

use the Int 13h interface.

The organization is shown in the following diagram:

:fig id=vflpyorg place=inline frame=box

+----------+                    +---------+

|          |                    |         |------------------+

|OS/2 Apps.|                    |VDM Apps.|-----------------+|

|          |                    |         |<-----------+    ||

+----------+                    +---------+            |    ||

|    |             Int 21 calls | |Direct port  sim.|    ||

DevIOCTL|    |File System +-------------+ |access      intr.|    ||

calls|    |calls       |               |                 |    ||

v    v            v               v                 |    ||

+-----------------------------+  +--------------+    +----+ ||

|OS/2 Kernel                  |  |Virtual Floppy|--->|VPIC| ||

|  +-----------+-------------+|  |Device Driver |Intr+----+ ||

|  |File System|Dos Emulation||  |   (VFLPY)    |Sim.       ||

|  +-----------+-------------+|  |              |-----+     ||

+-----------------------------+  +--------------+     |     ||

|           req. own | ^ ^ |  ^mask/   |intr.||

|DevIOCTL  +---------+ | | |  |unmask  |     ||

|          |  +--------+ | |  +------+ |     ||

v          v  |req. own  | |sim      | v     ||

+-------------------------------+       | |h/w    +----+    ||

| Physical Floppy Device Driver |-------+ |access |VDMA|<---+|

+-------------------------------+Floppy   |       +----+     |

|                 Intr.    |  +---------------+

|                          |  |

v                          v  v

+-----------------------------------------------+

| Physical Floppy Controller Hardware           |

+-----------------------------------------------+

:figcap.VFLPY component organization

:efig.

Obtaining Physical Floppy Hardware Ownership

When the system first starts up, the physical floppy device driver

owns the physical floppy hardware.  All floppy accesses from the

OS/2 applications will go through the file system and/or the physical

floppy device driver and eventually serialized into requests.

Initially, VFLPY does not own the floppy controller and it hooks all

the floppy ports.  It will request ownership from the PFLPY whenever

the VDM accesses any floppy port or whenever the VDMA informs VFLPY that

the floppy DMA channel has been unmasked, whichever comes first.

If the immediate ownership request is not successful, the VDM

will be blocked until PFLPY calls back and grants the ownership.  Note that

there is no need for a timeout in the block because PFLPY is assumed to be

well-behaved and will grant the ownership as soon as it has finished its

operation.  When VFLPY gets the ownership, it unhooks all the floppy ports

so that subsequent accesses will be passed to the hardware directly.

Releasing Physical Floppy Hardware Ownership

Once VFLPY has obtained the floppy ownership, it will not release it

until PFLPY is requesting for it.  VFLPY, upon receiving ownership request

from the PFLPY, must decide if it is safe (the physical floppy is in a

consistent state) to release the ownership.  It makes the decision based

on if VDMA has informed VFLPY about the floppy DMA channel being masked.

Since the VFLPY is not monitoring the floppy port accesses, it has no idea

when the floppy operation is done.  Therefore, it makes an assumption that

when the floppy DMA channel is masked or the floppy motor has been turned

off, the floppy operation is done.  To make sure this is the case, VFLPY

will read the floppy status register to see if all the status bytes have

been read when VDMA informs VFLPY about the channel mask event.  Therefore,

an atomic floppy operation is defined by the first floppy port access or DMA

unmask event as the starting point and the DMA mask event or motor turn off

as the ending point.  Floppy ownership can only be granted in between atomic

floppy operations.

Physical Floppy Interrupt Routing

Physical floppy interrupts are always routed to the PFLPY by the interrupt

manager.  If VFLPY has the exclusive hardware ownership when the interrupt

happens, PFLPY will route the event to the VFLPY and VFLPY will simulate

the interrupt to the VDM.  When VFLPY receives interrupt events from the

PFLPY, it will also inform VDMA about it.

BIOS Int 13h Support

Since VFLPY is supporting port accesses, there is no attempt in VFLPY

to virtualize Int 13h accesses.  The ROM BIOS Int 13h code is allowed to run.

But since Int 13h interface is for the hard disk as well as the floppy,

VDSK still hooks it in order to screen out the hard disk accesses

and to allow the floppy accesses to go to the ROM.

BIOS Data Area Maintenance

Since VFLPY allows ROM Int 13h code to run, it will take care of the

BIOS data area update automatically.

Drive Motor Timeout Simulation

ROM BIOS uses the timer tick interrupt handler to do the drive motor

timeout.  In the VTimer, we simulate the drive motor

timeout in the timer tick interrupt simulation.  It decrements the

BIOS floppy motor timeout count and if it reaches zero, VFLPY is

called to turn off the floppy motor.

:h4.Architectural Review

To be performed.

:h3.Virtual Floppy Device Driver Design

:h4.Design Overview

:h5.I/O port trapping handlers

Initially, the VDM does not have the ownership of the floppy hardware.

Therefore, all the floppy I/O port accesses are trapped to a common

handler.  This handler will check if the VDM already owns the floppy.

If not, it will try to request the ownership either from another VDM or

from the PFLPY.  If the ownership request fails, it will block until

the ownership is granted.  When the VDM obtains the floppy ownership,

it disable all the floppy I/O port traps for the VDM to let all subsequent

floppy port accesses to go to the hardware directly.

The floppy controller has the following registers:

:ul

:li.Status Register A (port 3f0h - PS/2 only)

:li.Status Register B (port 3f1h - PS/2 only)

:li.Digital Output Register (port 3f2h)

:li.Digital Input Register (port 3f7h)

:li.Configuration Register (port 3f7h)

:li.Diskette Drive Controller Status Register (port 3f4h)

:li.Diskette Drive Controller Data Register (port 3f5h)

:eul

The VDM will keep the floppy ownership indefinitely until somebody else

is requesting for it.  The requester may be another VDM or the PFLPY.

As long as the VFLPY thinks that the VDM is still busy with the floppy

hardware, it will not release the ownership to the new requester.  Since

VFLPY is not monitoring the VDM's accesses to the floppy ports, it makes

the assumption that the VDM is done with the floppy if the floppy motor

is off or if VDMA informs VFLPY that the VDM has masked the floppy DMA

channel plus the fact that all the status bytes in the floppy controller

have been read.

While the VDM is owning the floppy hardware, all floppy interrupts from

the PFLPY are simulated to the owning VDM.

Even if the VFLPY determines that VDM is done with the floppy hardware,

it will not release the floppy ownership unless there is a requester

requesting for it.  In other words, the ownership will only be changed on

demand.

:h4.Exported Interfaces

.*

:h5.Exported data declarations

.im1 vflpyext.sin

.*

:h5.Exported procedure interface flag definitions

.im1 vflpyexk.sin

.*

:h5.Exported procedure headers

.im1 vflpyexp.sin

.*

.*

:h4.Imported Interfaces

.im1 vflpyimp.sin

.*

.*

:h4.Design Constraints

:ul.

:li.Since the floppy interrupt is still simulated and the DMA transfer

is supervised by the VDMA, the performance of the floppy access may

be slightly lower than it would be on DOS.  The majority of the DOS

software should have no problem with this, but there may be a few

copy protected softwares which are highly sensitive to the access timing

that may fail.

:eul

:h4.Design Review

To be performed.

:h3.Virtual Floppy Device Driver Implementation

:h4.Internal Interfaces

.*

:h5.Local data declarations

.im1 vflpylot.sin

.*

:h5.Local procedure interface flag definitions

.im1 vflpylok.sin

.*

:h5.Local procedure headers

.im1 vflpylop.sin

.*

.*

:h4.Implementation

To be added.

:h4.Implementation Review

To be performed.

:h3.Virtual Floppy Device Driver Appendix (included for each review)

:h4.Glossary

To be added.

:h4.Size and Performance Considerations

To be added.

:h4.Implementation Estimates

To be added.

:h4.Design History

None.