Engineering Release Notice

OEM Name:					LSI

Component:				Hypercfg(LINUX)

Engineering Release Date:		11/17/2003

Engineer:					Bibhu

Products Supported:			MegaIDE ATA 133-2, 649 and CSB6

Released To:  	Release Coordinator

Anuj Jain

Version Numbers:

Component			Current Version		Previous Version

- Hypercfg	  		3.8 November 17,2003	3.7 October 02,2003

### Reason for Release

# /T, Ti option are dropped from menu item.

# Bug Fixes

#### Bug Fix #1

Tracking #:	N/A

Customer:	N/A

Symptom: 	N/A

Fix:  N/A

How to validate: N/A

# Enhancements

# Enhancement #1

# Tracking #: LSID100006190

Customer: 	LSI

# Purpose:  /T, /Ti options are dropped from menu item.

Enhancement details: User input parsed and /T, /Ti option not taken into account.

How to validate  Take CSB6 system. Create LD. Test for the options.

# Component Features

/S – Silent Operation:

When this option is set, the utility does not prompt the user.

/F[FILENAME]:

This Option redirects the output to the specified FILENAME

If FILENAME is not specified then the default file is taken to be HYPERCFG.DAT.

/Yx[filename]

This option is used for error logging

If filename is not specified the default is hypercfg.log

‘x’ = f – default – log only errors.

‘x’ = c – continuous logging

CONFIGURATION OPTIONS:

The options specified with ‘[]’ are optional and those within ‘<>’ are mandatory.

These options are used to configure the RAID.  The valid switches are:

/A[m]:D1[,D2,D3,D4] /G[N]:[SIZE1] /G[N1]:[SIZE2]– Automatic RAID configuration

m = S[STRIPE SIZE] – configures RAID in STRIPE MODE(RAID-0) with specified STRIPE SIZE. STRIPE SIZE must lie within the range [32, 64, 128, 256, 512, 1024, 2048, 4096]. If invalid STRIPE SIZE is specified the utility assumes 64K STRIPE SIZE.  Default STRIPE SIZE is 64k

m = M – configures RAID in MIRROR MODE (RAID-1)

m = R[STRIPE SIZE] – configures RAID in RAID 10 mode with specified STRIPE SIZE.  STRIPE SIZE must lie within the range [64…4096], in multiples of 4K.

D1, D2…D4 specifies the drive that participates in the array.

Valid nos. are 00,, 10,  for Port#0, Port#1 respectively.

/G[n]:[size] creates a logical array in the array ‘n’ with the size specified

Size specified is in MB

Default value for n is 0 and if size is not specified, the whole array is allotted to the logical drive.

DRIVE-ID:

Drive id is specified by /C[n] /D[cd]

/C[n] – Specifies the card no. If n is not specified, then it is taken to be 0

/D[cd] – Specifies the drive no.

c = Channel No. (0 – for Primary, 1- for Secondary) – default is 0

d – Always 0 for this controller, only Master is supported.

/L /C[n] /D[cd] [/b[filename]]– Displays the configuration sector of the specified drive

- If ONLY /L is used, the configuration sector of the first available drive is displayed.

The order of searching for the first available drive is:

Card 0 Channel 0 Drive 0 – Primary Master

Card 0 Channel 1 Drive 0 – Secondary Master

.

.

.

- If either /D or /C is specified then the configuration sector of the

specified drive is displayed

- /b[fn] – saves the IRCD in a bin file – fn. If fn is not specified, default is 		       HYPERCFG.BIN

/E<o> /C[n] /D[cd]  - Erases the configuration/error log sector of specified drive

o = C – Erases the configuration sector

o = E – Erases the error log sector

o = A – Erases both configuration and error log sector

/@ /b[filename] To restore configuration from binary file use /@ /b[binfile] if binfile is not specified, then default is HYPERCFG.BIN

DISPLAY OPTIONS:

This option determines how the data is displayed. This option is valid only for /I option.

/W[o] – Specifies display mode

o = V – Display in verbose mode

Default Option: If [o]  is not specified, then display will be as HEX Dump.

MISCELLANEOUS OPTIONS:

/M[r/<max-address>] /C[n] /D[cd] (NOTE: This feature is available only in DOS version)

This option allows the user to set the maximum accessible sectors on the

Harddrive that supports this feature. The address is specified in HEX

/Mr – Resets the max. address to NATIVE MAX ADDRESS of HDD.

/P[S] /C[n] /D[cd] – Switches the drive to specified state.

S = S – Drive is powered on in Suspend state.

Default Option: If [S] is not specified, then the drive is switched on in Active state.

/V – Detects the Presence of RAID BIOS and displays the version of BIOS (NOTE: Feature availability depends on driver used for non-DOS platforms)

/R[n] [/$[FILENAME]] [/B[FILENAME]] /C[n] /D[cd] - Reads and displays the ‘n’th sector

If ‘n’ is not specified then sector no. is taken to be 0

/B option dumps specified sector to the specified file in BIN format

/$ option dumps specified BIN file to the ‘n’th sector

The default filename is HYPERCFG.BIN

/X – This option pauses and waits for keystroke if any error occurs on executing HyperCFG.

/I[a] /C[n] /D[cd] [/B[filename]] - Displays the information about the specified drive

/Ia displays ID Device Packet for all drives present.

/Bfilename – Saves the 512 byte ID info. Pkt. To filename. If filename

is not specified, then the default filename is HYPERCFG.BIN

This option should NOT be used with /Ia option.

/Z[E/D] – Enables / Disables SMART feature. Default option is ENABLE

Note:

DMA and Write cache are made ON by default while creating R0/R1/R10

###### Known Restrictions

None

Known Bugs

# None.

# Suggested Additional Areas to Test

None

###### Developer Level Testing

Testing done on 649 controller work wise follows IRCD 2.x and same as CSB6 environment.